3GPP TSG-RAN WG2 Meeting #132	R2-2509066

Dallas, USA,  Nov. 17th - 21st

Source:                Qualcomm Incorporated, vivo, CATT, Xiaomi

Title:	Discussion on low mobility criteria for measurement relaxation and offloading

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

In last RAN2 meeting, whether to introduce low mobility as condition for measurement relaxation/offloading was discussed but no conclusion. This contribution discusses this issue and proposes not to introduce low mobility criteria.

# Discussion

In Rel-16, UE low mobility criterion was introduced as one condition for measurement relaxation as follows:

---------------------------------------------------------------38.304--------------------------------------------------------------

When the UE is required to perform measurements of intra-frequency cells or NR inter-frequency cells or inter-RAT frequency cells according to the measurement rules in clause 5.2.4.2:

-	if lowMobilityEvaluation is configured and cellEdgeEvaluation is not configured; and

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP:

-	the UE may choose to perform relaxed measurements for intra-frequency cells, NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8];

-	if cellEdgeEvaluation is configured and lowMobilityEvaluation is not configured; and

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	the UE may choose to perform relaxed measurements for intra-frequency cells according to relaxation methods in clauses 4.2.2.9 and 4.2C.2.7 in TS 38.133 [8];

-	if the serving cell fulfils Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ:

-	the UE may choose to perform relaxed measurements for NR inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.10, 4.2.2.11 and 4.2C.2.8 in TS 38.133 [8];

-	if both lowMobilityEvaluation and cellEdgeEvaluation are configured:

-	if the UE has performed normal intra-frequency, NR inter-frequency, or inter-RAT frequency measurements for at least TSearchDeltaP after (re-)selecting a new cell; and

-	if the relaxed measurement criterion in clause 5.2.4.9.1 is fulfilled for a period of TSearchDeltaP; and

-	if the relaxed measurement criterion in clause 5.2.4.9.2 is fulfilled:

-	the UE may choose to perform relaxed measurements for NR intra-frequency cells, inter-frequency cells or inter-RAT frequency cells according to relaxation methods in clauses 4.2.2.9, 4.2.2.10, 4.2.2.11, 4.2C.2.7 and 4.2C.2.8 in TS 38.133 [8];

In the Rel-16 RRM relaxation, the criteria of lowMobilityEvaluation and cellEdgeEvaluation are independent and can be configured optionally. Even without low mobility criteria, UE can also perform measurement relaxation using cell edge criteria.

Observation 1: In the Rel-16 RRM relaxation, the criteria of lowMobilityEvaluation and cellEdgeEvaluation are independent and can be configured optionally. Even without low mobility criteria, UE can also perform measurement relaxation using cell edge criteria.

If low mobility criteria is also introduced for Rel-19 RRM relaxation/offloading, some issues need to be further discussed, e.g. which case the mobility criteria should be applied, RRC relaxation mode or offloading mode or both. For RRM relaxation mode, it should use MR measurement or LR measurement to evaluate low mobility. Whatever using MR or LR measurement, RAN4 has to be involved to further investigate requirements, which will introduce a large amount of RAN4 work, e.g. the UE behavior when only one condition (cell edge or low mobility) is configured, or when two conditions are configured but only one condition or both conditions are fulfilled, etc. In Rel-16, RAN4 have defined separate requirements for all these cases. Similar situation will happen in RAN4 discussions, which may lead a long discussion.

Besides, since current criteria for RRM relaxation/offloading could be based on LR measurements, UE could evaluate whether it is in cell edge quickly, even UE moves fast there is no issue as it could perform RRM relaxation/offloading when it is not in cell edge.

Considering the WID is closed, and the existing entry/exit condition can work without low mobility criteria, it is proposed not to introduce low mobility criteria (including MR or LR measurement based) to evaluate whether to enter or exit from Rel-19 RRM relaxation/offloading mode.

- Not to introduce low mobility criteria (including MR or LR measurement based) to evaluate whether to enter or exit from Rel-19 RRM relaxation/offloading mode.

# Conclusion

- Not to introduce low mobility criteria (including MR or LR measurement based) to evaluate whether to enter or exit from Rel-19 RRM relaxation/offloading mode.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)