3GPP TSG-RAN WG2 Meeting #132	R2-2509130

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.5.2

Source: 	Fujitsu

Title:	Corrections to SIB1 request [F001] [F002]

Document for:	Discussion and Decision

- Introduction

This contribution will discuss the remaining issues related to SIB1 request, i.e. RIL [F001] [F002] [1] and provide our proposals.

- Discussions

In current RRC specification [2], request for on-demand SIB1 is captured in clause 5.2.2.3.3b as below:

##### 5.2.2.3.3b Request for on-demand SIB1 The UE shall, while SDT procedure is not ongoing: 1> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig included in od-sib1-Config for this cell in stored valid version of SIB26; 3> if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig is reached is received from lower layers as defined in TS 38.321 [3]: 4> perform the actions as specified in clause 5.2.2.5. 3> if acknowledgement for SIB1 request is received from lower layers: 4> acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately; 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2; 1> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL and criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfigSUL included in od-sib1-Config for this cell in stored valid version of SIB26; 3> if indication that maximum number of PRACH attempts as configured in sib1-RequestConfigSUL is reached is received from lower layers as defined in TS 38.321 [3]: 4> perform the actions as specified in clause 5.2.2.5. 3> if acknowledgement for SIB1 request is received from lower layers: 4> acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately; 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2; 1> if cell reselection occurs while waiting for the acknowledgment for SIB1 request from lower layers: 2> reset MAC; 1> if the UE is unable to acquire the SIB1: 2> perform the actions as specified in clause 5.2.2.5.

According to highlighted part, RRC will trigger the lower layer to initiate the Random Access procedure on normal/ supplementary uplink if criteria to select normal/supplementary uplink is met.

In current MAC specification [3], criteria to select normal/supplementary uplink is defined in clause 5.1.1:

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall: 1> flush the Msg3 buffer; 1> flush the MSGA buffer; 1> set the PREAMBLE_TRANSMISSION_COUNTER to 1; 1> if the Random Access procedure is initiated on a Serving Cell; or 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell and the PDCCH order indicates preamble initial transmission; or 1> if the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell, which is different from the cell to which the MAC entity performed the last Random Access Preamble transmission, and the PDCCH order indicates preamble re-transmission: 2> set the PREAMBLE_POWER_RAMPING_COUNTER to 1; 1> set the PREAMBLE_BACKOFF to 0 ms; 1> set POWER_OFFSET_2STEP_RA to 0 dB; 1> set POWER_OFFSET_RO_TYPE to 0 dB; 1> if the carrier to use for the Random Access procedure is explicitly signalled: 2> select the signalled carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the signalled carrier. 1> else if the carrier to use for the Random Access procedure is not explicitly signalled; and 1> if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and 1> if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL: 2> select the SUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the SUL carrier. 1> else: 2> select the NUL carrier for performing Random Access procedure; 2> set the PCMAX to PCMAX,f,c of the NUL carrier. NOTE 4: Void. 1> perform the BWP operation as specified in clause 5.15, except when the Random Access procedure is initiated by the PDCCH order for an LTM candidate cell; …

According to highlighted part, there are three criteria to select supplementary uplink and one of the criteria is the serving cell is configured with supplementary uplink. This is captured in clause 5.2.2.4.2 of RRC specification [2], i.e. the action upon reception of SIB1, as below:

4> if supplementaryUplink is present in servingCellConfigCommon; and 4> if the UE supports one or more of the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and 4> if the UE supports at least one additionalSpectrumEmission in the nr-NS-PmaxList for a supported supplementary uplink band; and 4> if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on the frequency bands indicated in the frequencyBandList for the supplementaryUplink; and 4> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15] and TS 38.101-2 [39]) which - is smaller than or equal to the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which - is wider than or equal to the bandwidth of the initial uplink BWP of the SUL: 5> consider supplementary uplink as configured in the serving cell; 5> select the first frequency band in the frequencyBandList for the supplementaryUplink which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation; 5> apply a supported supplementary uplink channel bandwidth with a maximum transmission bandwidth which - is contained within the carrierBandwidth (indicated in supplementaryUplink for the SCS of the initial uplink BWP), and which - is wider than or equal to the bandwidth of the initial BWP of the SUL; 5> apply the first listed additionalSpectrumEmission which it supports among the values included in nr-NS-PmaxList within frequencyBandList for the supplementaryUplink; 5> if the additionalPmax is present in the same entry of the selected additionalSpectrumEmission within nr-NS-PmaxList for the supplementaryUplink: 6> apply the additionalPmax in supplementaryUplink for SUL; 5> else: 6> apply the p-Max in supplementaryUplink for SUL; NOTE 2: For an out of coverage L2 U2N Remote UE or L2 Intermediate U2N Relay UE in RRC_IDLE or RRC_INACTIVE receiving SIB1 from its connected parent L2 U2N Relay UE, it is up to Remote UE implementation whether to consider and apply the following parameters: frequencyBandList, carrierBandwidth, frequencyShift7p5khz, frequency band, channel bandwidth, the configuration included in the servingCellConfigCommon, the specified PCCH configuration, additionalSpectrumEmission, additionalPmax, and p-Max. 2> else: 3> consider the cell as barred in accordance with TS 38.304 [20]; and 3> perform cell re-selection to other cells on the same frequency as the barred cell as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, or intraFreqReselection2RxXR for 2Rx XR UEs is set to notAllowed as specified in TS 38.304 [20];

It means that the UE determines whether the serving cell is configured with supplementary uplink after the SIB1 is received. So, the UE cannot evaluate whether the criteria to select supplementary uplink are met upon SIB1 request.

Observation 1: there is UE implementation problem since the UE cannot determine whether the serving cell is configured with supplementary uplink, i.e. one of the criteria to select supplementary uplink upon SIB1 request.

To solve the problem, there are two options:

- Option 1: modify MAC specification so that the UE is not required to determine whether the serving cell is configured with supplementary uplink

- Option 2: modify RRC specification so that the UE can determine whether the serving cell is configured with supplementary uplink upon SIB1 request

According to RAN1 parameter list [4], the UE needs to check the supported frequency band(s) and the supported frequency band is one of conditions to determine whether the serving cell is configured with supplementary uplink. It implies that the UE is required to check the UE capability. So, Option 1 is not aligned with RAN1 agreement and Option 2 is preferred.

| frequencyBandList | New or existing | Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field.This refers to the IE within FrequencyInfoUL-SIB. | SEQUENCE (SIZE (1..maxNrofMultiBands)) OF NR-MultiBandInfo | per [frequencyInfoUL] |
| --- | --- | --- | --- | --- |

Proposal 1 [F001]: Modify RRC specification so that the UE can determine whether the serving cell is configured with supplementary uplink upon SIB1 request, at least according to the supported band(s).

The next issue is how to implement the procedure required by RAN1 according to the RAN1 parameter list, i.e. the UE shall apply the first listed band which it supports in the frequencyBandList field. Although it is captured in the field description, it is unclear since there could be two uplink configurations, i.e. one for normal uplink and the other for supplementary uplink. For example, the UE can apply both configurations when SIB26 is received no matter if the uplink is configured with the cell or not. Alternatively, the UE can only apply corresponding configurations for the selected UL carrier which is configured for the cell.

Observation 2: it is unclear on how to implement the procedure required by RAN1 according to the RAN1 parameter list, i.e. the UE shall apply the first listed band which it supports in the frequencyBandList field.

Proposal 2 [F002]: Modify RRC specification so that the UE applies the first listed band which it supports for the selected uplink upon SIB1 request.

In summary, changes to clause 5.2.2.3.3b of RRC specification are required to implement both proposals.

Proposal 3 [F001/F002]: Adopt the TP as below:

##### 5.2.2.3.3b Request for on-demand SIB1 The UE shall, while SDT procedure is not ongoing: 1> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfig and criteria to select normal uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> select the first frequency band in the ul-frequencyBandList for the normal uplink which the UE supports in sib1-RequestConfig included in od-sib1-Config for this cell in stored valid version of SIB26; 2> trigger the lower layer to initiate the Random Access procedure on normal uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfig included in od-sib1-Config for this cell in stored valid version of SIB26; 3> if indication that maximum number of PRACH attempts as configured in sib1-RequestConfig is reached is received from lower layers as defined in TS 38.321 [3]: 4> perform the actions as specified in clause 5.2.2.5. 3> if acknowledgement for SIB1 request is received from lower layers: 4> acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately; 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2; 1> if od-SIB1-Config for this cell in stored valid version of SIB26 includes sib1-RequestConfigSUL; and 1> if the UE supports one or more of the frequency bands indicated by the ul-frequencyBandList in the sib1-RequestConfigSUL for this cell: 2> consider supplementary uplink as configured in the serving cell; 1> if criteria to select supplementary uplink as defined in TS 38.321[3], clause 5.1.1 is met: 2> select the first frequency band in the ul-frequencyBandList for the supplementary uplink which the UE supports in sib1-RequestConfigSUL included in od-sib1-Config for this cell in stored valid version of SIB26; 2> trigger the lower layer to initiate the Random Access procedure on supplementary uplink in accordance with TS 38.321 [3] using the PRACH preamble(s) and PRACH resource(s) in sib1-RequestConfigSUL included in od-sib1-Config for this cell in stored valid version of SIB26; 3> if indication that maximum number of PRACH attempts as configured in sib1-RequestConfigSUL is reached is received from lower layers as defined in TS 38.321 [3]: 4> perform the actions as specified in clause 5.2.2.5. 3> if acknowledgement for SIB1 request is received from lower layers: 4> acquire the requested SIB1 message as defined in as specified in TS 38.213 [13], clause 23, immediately; 4> upon acquiring SIB1, perform the actions specified in clause 5.2.2.4.2; 1> if cell reselection occurs while waiting for the acknowledgment for SIB1 request from lower layers: 2> reset MAC; 1> if the UE is unable to acquire the SIB1: 2> perform the actions as specified in clause 5.2.2.5.

- Conclusion

This document discussed the remaining open issues for SIB1 request, i.e. RIL F001/F002 and provided our text proposal.

Observation 1: there is UE implementation problem since the UE cannot determine whether the serving cell is configured with supplementary uplink, i.e. one of the criteria to select supplementary uplink upon SIB1 request.

Proposal 1 [F001]: Modify RRC specification so that the UE can determine whether the serving cell is configured with supplementary uplink upon SIB1 request, at least according to the supported band(s).

Observation 2: it is unclear on how to implement the procedure required by RAN1 according to the RAN1 parameter list, i.e. the UE shall apply the first listed band which it supports in the frequencyBandList field.

Proposal 2 [F002]: Modify RRC specification so that the UE applies the first listed band which it supports for the selected uplink upon SIB1 request.

Proposal 3 [F001/F002]: Adopt the TP as above.

# Reference

- R2-2509085, “NES Comments File”, Ericsson report, RAN2#132

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification", v19.0.0

- 3GPP TS 38.321: "NR; Medium Access Control (MAC); Protocol specification", v19.0.0

- R2-2508022, “LS on Rel-19 higher layers parameters list Post RAN1#122bis (R1-2508205; contact: Ericsson)”, RAN1 LS in, To: RAN2 Cc: RAN4, RAN3, RAN2#132