3GPP TSG-RAN WG2 Meeting #132	R2-2509172

Dallas, US, November 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1377 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on Type 2 and Type 4 UE Capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NonCol_intraB_ENDC_NR_CA_Ph2-Core, NonCol_intraB_ENDC_NR_CA |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the RAN2 discussion, - For the type 2 capability, the UE is allowed to report it with the parent BC for the CA case, with the restriction that this capability is applicable only to the fallback BC with two CCs for the CA case (as defined in the RAN4 spec). - For the type 4 capability, the UE is allowed to report it with the parent BC for the CA and EN-DC case, with the restriction that this capability is applicable only to the fallback BC with single CC on the NR side for the EN-DC case and with two CCs for the CA case (as defined in the RAN4 spec). However, this has not been included in the spec yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the above agreement to the field descriptions of the intraBandNR-CA-non-collocated-r18/ intraBandNR-CA-non-collocated-r19/ interBandMRDC-WithOverlapDL-Bands-r19 as following: 1: Add “The UE is allowed to indicate this capability for the band combination with more than 2 intra-band NR CCs, but this capability is only applicable for the corresponding fallback band combination with only 2 intra-band NR CCs.” to the field description of the intraBandNR-CA-non-collocated-r18/ intraBandNR-CA-non-collocated-r19; 2: Add “The UE is allowed to indicate this capability for the band combination with more than 1 NR CC on the corresponding NR band (i.e. the NR band that overlaps with EUTRA band), but this capability is only applicable to the fallback band combination with only single CC on that NR band.” to the field description of the interBandMRDC-WithOverlapDL-Bands-r19. Impact analysis Impacted 5G architecture options: NR CA, EN-DC Impacted functionality: Type 2 and Type 4 requirements for the non-collocated CA and EN-DC Inter-operability: - If the UE is implemented according to the CR and the network is not, the network may misunderstand these capabilities for the parent BC. - If the network is implemented according to the CR and the UE is not, the UE is unable to indicate these capabilities if they are reported with parent BC, which may lead to signalling overhead. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network may misunderstand these capabilities for the parent BC case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.4, 4.2.7.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of the R2-2508508, delete the modification to the chapter 4.2.2 |  |  |  |  |  |  |  |  |

4.2.7.4	CA-ParametersNR

/*************************************Omit the unrelated part************************************/

| intraBandFreqSeparationUL-AggBW-GapBW-r16 Indicates the UL frequency separation class between lower edge of lowest CC and upper edge of highest CC of Intra-band UL non-contiguous CA, i.e. including both the aggregated bandwidth and the gap bandwidth. 3 frequency separation classes are introduced and the values are defined in Table 5.3A.5-2 of TS 38.101-1 [2]. | BC | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| intraBandNR-CA-non-collocated-r18 Indicates whether the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [2], and TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2] (i.e. Type 2 UE). If the capability is not reported, the UE only supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in TS 38.133 [5] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [2] (i.e. Type 1 UE). A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-r18 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9]. The UE is allowed to indicate this capability for the band combination with more than 2 intra-band NR CCs, but this capability is only applicable for the corresponding fallback band combination with only 2 intra-band NR CCs. | BC | No | N/A | FR1 only |
| intraBandNR-CA-non-collocated-r19 Indicates whether the UE supports Type 1 UE requirements and Type 4 UE requirements in TS 38.101-1 [2]. If the capability is not reported, the UE supports Type 1 UE requirements in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of intraBandNR-CA-non-collocated-r18. A UE supporting this feature shall also support network controlled indication of the MTTD/MRTD and RF requirements by nonCollocatedTypeNR-CA-v1900 for intra-band non-collocated NR-CA, as defined in TS 38.331 [9]. The UE is allowed to indicate this capability for the band combination with more than 2 intra-band NR CCs, but this capability is only applicable for the corresponding fallback band combination with only 2 intra-band NR CCs. | BC | No | N/A | FR1 only |

4.2.7.9	MRDC-Parameters

/*************************************Omit the unrelated part************************************/

| interBandContiguousMRDC Indicates for an inter-band (NG)EN-DC/NE-DC combination, where the frequency range of the E-UTRA band is a subset of the frequency range of the NR band (as specified in Table 5.5B.4.1-1 of TS 38.101-3 [4]), that the UE supports intra-band contiguous (NG)EN-DC/NE-DC requirements (see TS 38.101-3 [4]). If the field is absent for such an inter-band (NG)EN-DC/NE-DC combination, the UE supports intra-band non-contiguous (NG)EN-DC/NE-DC requirements. | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| interBandMRDC-WithOverlapDL-Bands-r16 Indicates whether the UE supports FDD-FDD or TDD-TDD inter-band (NG)EN-DC/NE-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and NE-DC MTTD/MRTD according to clause 7.5.5/7.6.5 in TS 38.133 [5] and inter-band RF requirements (i.e. Type 2 UE). If the capability is not reported, the UE supports FDD-FDD or TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC/NE-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and intra-band RF requirements (i.e. Type 1 UE). | BC | No | N/A | FR1 only |
| interBandMRDC-WithOverlapDL-Bands-r19 Indicates whether the UE supports TDD-TDD inter-band (NG)EN-DC operation with overlapping or partially overlapping DL bands with an (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [5] and inter-band UE RF requirements for inter-band EN-DC including 7.10B in TS38.101-3 [4] (i.e., Type 4 UE). Type 4 supported UE shall also meet Type 1 UE requirements which are TDD-TDD inter-band operation with overlapping or partially overlapping DL bands with (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [5] and inter-band UE RF requirements excluding 7.10B in TS38.101-3 [4] (i.e., Type 1 UE). If the capability is not reported, the UE supports the Type 1 UE requirements. A UE supporting this feature shall also indicate support of requirementTypeIndication-r18. A UE supporting this feature shall also support network controlled indication of MTTD/MRTD and RF requirements by nonCollocatedTypeMRDC-v1900 for inter-band non-collocated MRDC, as defined in TS 38.331 [9]. The UE is allowed to indicate this capability for the band combination with more than 1 NR CC on the corresponding NR band (i.e. the NR band that overlaps with EUTRA band), but this capability is only applicable to the fallback band combination with only single CC on that NR band. | BC | No | N/A | FR1 only |