3GPP TSG-RAN WG2 #132	R2-2509233

# Dallas, USA, 17th – 21st November 2025

Agenda Item:	 8.16

Source:	Ericsson

Title:	RIL E101 E102 BDS Correction

Document for:	Discussion

# 1	Introduction

In this paper, we discuss the RIL issues and provide analysis on RILs.

# 2 	Discussion

## 2.1 E101 Field midiAlmB2bIHealth-r19

As per BDS B2b ICD document, the satellite health is provided in table 7-12 as below. The 5th bit of the table is for B2b_I signal whereas 7th bit is for B1C, 6th bit is for B2a signal.

We see that this information is provided by updating the legacy fields reference and also adding a new BOOLEAN. It appears that the new BOOLEAN is redundant as the information about satellite health status can be provided by the existing field similar to that for B1C and B2a.

#### –	AlmanacReducedKeplerianSet

-- ASN1START

AlmanacReducedKeplerianSet ::= SEQUENCE {

svID					SV-ID,

redAlmDeltaA			INTEGER (-128..127),

redAlmOmega0			INTEGER (-64..63),

redAlmPhi0				INTEGER (-64..63),

redAlmL1Health			BOOLEAN,

redAlmL2Health			BOOLEAN,

redAlmL5Health			BOOLEAN,

...,

[[

redAlmB2bIHealth-r19			BOOLEAN			OPTIONAL	-- Cond MultiBDS-Signal

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| MultiBDS-Signal | The field is mandatory present when B2bI signal health is indicated along with other B1C and/or B2a Signals. Otherwise, it is not present. |

AlmanacReducedKeplerianSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

redAlmDeltaA Parameter A, metres [4], [5], [6], [7], [39], [49], [56]. Scale factor 2+9 metres.

redAlmOmega0 Parameter 0, semi-circles [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-6 semi-circles.

redAlmPhi0 Parameter 0, semi-circles [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-6 semi-circles.

redAlmL1Health Parameter L1 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the Satellite clock health state (the 8th bit) defined in table 7-14 [39] for BDS B1C, table 7-14 [49] for BDS B2a and table 7-12 [56] for BDS B2b.

redAlmL2Health Parameter L2 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B1C signal health state (the 7th bit) defined in table 7-14 [39] for BDS B1C, table 7-14 [49] for BDS B2a and table 7-12 [56] for BDS B2b.

redAlmL5Health Parameter L5 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B2a signal health state (the 6th bit) defined in table 7-14 [39] for BDS B1C, table 7-14 [49] for BDS B2a and table 7-12 [56] for BDS B2b.

redAlmB2bIHealth This field indicates the B2b_I signal health state (the 5th bit) defined in table 7-12 [56].

The same is applicable also for below IE.

#### –	AlmanacMidiAlmanacSet

-- ASN1START

AlmanacMidiAlmanacSet ::= SEQUENCE {

svID					SV-ID,

midiAlmE				INTEGER (0..2047),

midiAlmDeltaI			INTEGER (-1024..1023),

midiAlmOmegaDot			INTEGER (-1024..1023),

midiAlmSqrtA			INTEGER (0..131071),

midiAlmOmega0			INTEGER (-32768..32767),

midiAlmOmega			INTEGER (-32768..32767),

midiAlmMo				INTEGER (-32768..32767),

midiAlmaf0				INTEGER (-1024..1023),

midiAlmaf1				INTEGER (-512..511),

midiAlmL1Health			BOOLEAN,

midiAlmL2Health			BOOLEAN,

midiAlmL5Health			BOOLEAN,

...,

[[

midiAlmB2bIHealth-r19			BOOLEAN			OPTIONAL	-- Cond MultiBDS-Signal

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| MultiBDS-Signal | The field is mandatory present when B2bI signal health is indicated along with other B1C and/or B2a Signals. Otherwise, it is not present. |

AlmanacMidiAlmanacSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

midiAlmE Parameter e, dimensionless [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-16.

midiAlmDeltaI Parameter i, semi-circles [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-14 semi-circles.

midiAlmOmegaDot Parameter , semi-circles/sec [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-33 semi-circles/second.

midiAlmSqrtA Parameter , metres1/2 [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-4 metres1/2.

midiAlmOmega0 Parameter 0, semi-circles [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-15 semi-circles.

midiAlmOmega Parameter , semi-circles [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-15 semi-circles.

midiAlmMo Parameter M0, semi-circles [4], [5], [6], [7], [39, [49], [56]. Scale factor 2-15 semi-circles.

midiAlmaf0 Parameter afo, seconds [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-20 seconds.

midiAlmaf1 Parameter af1, sec/sec [4], [5], [6], [7], [39], [49], [56]. Scale factor 2-37 seconds/second.

midiAlmL1Health Parameter L1 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the satellite clock health state (the 8th bit) defined in table 7-14 [39] for BDS B1C, table 7-14 [49] for BDS B2a and table 7-12 [56] for BDS B2b.

midiAlmL2Health Parameter L2 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B1C signal health state (the 7th bit) defined in table 7-14 [39] for BDS B1C, table 7-14 [49] for BDS B2a and table 7-12 [56] for BDS B2b.

midiAlmL5Health Parameter L5 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B2a signal health state (the 6th bit) defined in table 7-14 [39] for BDS B1C, table 7-14 [49] for BDS B2a and table 7-12 [56] for BDS B2b.

midiAlmB2bIHealth This field indicates the B2b_I signal health state (the 5th bit) defined in table 7-12 [56].

For BDS B1C and BDS B2a, no new health flag has been added but we see it only exist for B2I. Hence, it would be good to clarify and remove the redundant bit, if so, as part of ASN.1 review. The field redAlmB2bIHealth-r19 in IE AlmanacReducedKeplerianSet and the field midiAlmB2bIHealth in IE AlmanacMidiAlmanacSet are redundant.

A conditional tag is added to the field description redAlmB2bIHealth-r19 and midiAlmB2bIHealth-r19 to describe the presence/absence of this field.

## 2.2 E102 Editorial Correction

We see the below field description needs to be updated. This description as such exists from Rel-17 but we can correct the field description at least from Rel-19. The sentence is grammatically incorrect as it ends with for and is difficult to read. There are 3 occurrences of such, and it can be updated in Rel-19 as part of ASN.1 review.

#### –	GNSS-SSR-OrbitCorrectionsSet2Req

The IE GNSS-SSR-OrbitCorrectionsSet2Req is used by the target device to request the GNSS-SSR-OrbitCorrectionsSet2 assistance from the location server.

-- ASN1START

GNSS-SSR-OrbitCorrectionsSet2Req-r17 ::= SEQUENCE {

refEphReq-r17					ENUMERATED { b1c,..., navicL1-v1900},

gnss-SSR-OrbitCorrectionsSet2Req-r17

GNSS-SSR-OrbitCorrectionsReq-r15	OPTIONAL,	-- Cond OC2-Req

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| OC2-Req | The field is mandatory present if the target device requests GNSS-SSR-OrbitCorrectionsSet2; otherwise it is not present. |

GNSS-SSR-OrbitCorrectionsSet2Req field descriptions

refEphReq This field, if present, indicates the reference ephemeris that the target device requests for the GNSS-SSR-ClockCorrectionsSet2.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For BDS B1C and BDS B2a, no new health flag has been added but we see it only exist for B2I. Hence, it would be good to clarify and remove the redundant bit, if so, as part of ASN.1 review. The field redAlmB2bIHealth-r19 in IE AlmanacReducedKeplerianSet and the field midiAlmB2bIHealth in IE AlmanacMidiAlmanacSet are redundant.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A conditional tag is added to the field description redAlmB2bIHealth-r19 and midiAlmB2bIHealth-r19 to describe the presence/absence of this field.

# References

TS 37.355, LPP