3GPP TSG RAN WG2#132	R2-2509416

Dallas, USA, 17th - 21st, November 2025	was R3-258263

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0428 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SN initiated inter-SN SCG LTM procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Ofinno, Ericsson, CATT, Qualcomm, Huawei, Google, LG Electronics, Nokia, Samsung, Lenovo, China Telecom) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. In Rel-18, - For SN-initiated SCG LTM without MN involvement, in Figure 10.3.2-6, UE sends the RRCReconfigurationComplete message to MN (step 8) and then the UE performs RACH procedure (step 10). - For SN initiated subsequent CPAC, in Figure 10.20-2, UE sends the RRCReconfigurationComplete message to MN (step 11) and then the UE performs RACH procedure (step 13). In Rel-19, however, for SN initiated inter-SN SCG LTM (i.e., Figure 10.5.2-5), it’s defined in the reverse order, i.e., UE performs RACH procedure first (step 20) and the UE sends the RRCReconfigurationComplete message to MN (step 21). It’s confusing when reading the specifications for DC procedures. 2. In SN initiated inter-SN SCG LTM procedure (i.e., Figure 10.5.2-5), currently step 23 and 24 (SN modification request procedure) are mandatory, which is not correct. The source SN may not be a candidate for a subsequent LTM. Thus, the source SN should be released. Instead of sending a SN modification request message, MN should send SN release request message to the source SN. 3. The signaling flow for inter-SN SCG LTM contains a few errors and inaccurate or missing information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Following Rel-15/16/17 DC procedures, a note is added to clarify the order for step 20 and step 21: - NOTE: The order the UE sends the MN RRCReconfigurationComplete* message (step 21) and performs the random access procedure towards the PSCell (step 20) is not defined. The successful random access procedure towards the target PSCell is not required for a successful completion of the RRC Reconfiguration procedure. 2. In Figure 10.5.2-5, step 23 and 24 (SN modification request procedure) is changed optional as we did in Rel-18 for SN initiated subsequent CPAC. Text descriptions were added to clarify: - If the last serving SN is not configured as a candidate SN, the MN triggers the MN initiated SN Release procedure to inform the last serving SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the last serving SN the address of the SN of the selected candidate PSCell and if applicable, starts normal data forwarding. 3. In Figure 10.5.2-5 on the signaling flows, several IE names are corrected, descriptions for TCI state configuration are added, and related clarifications are included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. It’s confusing when reading the specifications since Rel-18 and Rel-19 are aligned. 2. It’s not correct to mandate MN to trigger SN modification procedure to the source SN. 3. Ambiguities remain in the signaling flows for inter-SN SCG LTM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-258263. |  |  |  |  |  |  |  |  |

CHANGES START

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 10.5.2	MR-DC with 5GC

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

1.	The source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required with a LTM Request Indication to the MN providing a list of suggested LTM candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells and the upper limit for the number of PSCells that can be prepared by each candidate SN to the MN. Source SN may send L1 RS configuration request and/or early sync information request for the suggested candidate PSCell(s). The source SN may send the configuration ID mapping list for the suggested candidate PSCell(s) to the MN for subsequent inter-SN SCG LTM.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN provides the list of Proposed LTM No Security Change ID which can be used in each candidate SN, and forward the received measurement results to each candidate SN(s). The MN may transfer L1 RS configuration request and/or early sync information for suggested candidate PSCell(s) to SN. The MN also provides upper limit for the number of PSCells to each candidate SN. The MN may select one of the candidate SN(s) and request providing the SCG reference configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

NOTE 7:	The subsequent SCG LTM configurations can be included within an MN or an SN RRC message. For intra-SN SCG LTM in MN RRC message case, the source SN and the target SN shown in Figure 10.5.2-5 are the same node, and step 2 can be skipped.

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by the candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement or CSI acquisition, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN. The candidate SN provides the assigned LTM No Security Change ID for each candidate PSCell. The candidate SN may also provide the TCI States Configuration List and an indication of the complete or delta RRC configuration against the SCG reference configuration to the MN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement or CSI acquisition on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for the accepted candidate PSCell(s) to the MN for subsequent inter-SN SCG LTM.

7.	The MN transfers, during the LTM preparation phase, configuration ID mapping list, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via SN Modification Request message. The MN provides list(s) of KSN and associated sk-Counter values.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN Modification Request Acknowledge message. The candidate SN(s) may also provide the CSI report configuration.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN and the source SN and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding addresses as received from the candidate SN(s). In case multiple candidate SNs are prepared, the MN includes a list of data forwarding addresses to the source SN. The MN may also provide list(s) of KSN and associated sk-Counter values to the source SN.

12. In case of SN terminated bearers, early data forwarding may take place. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN(s). For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s).

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer message.

15.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9. The UE can also perform L3 measurement reporting to the source SN, including beam level measurement results on cell(s) which are configured as LTM candidate cell(s) according to the received network configuration.

16.	The source SN decides to execute cell switch to a target PSCell.

17.	The source SN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID.

18/19.	The source SN sends a Cell Switch Notification message, to indicate the triggering of LTM to the UE, including the target PScell ID and the TCI state ID(s).

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

NOTE 8: The order the UE sends the MN RRCReconfigurationComplete* message (step 21) and performs the random access procedure towards the PSCell (step 20) is not defined. The successful random access procedure towards the target PSCell is not required for a successful completion of the RRC Reconfiguration procedure.

23-27. The MN triggers the MN initiated SN Modification procedure to inform the last serving SN that UE has accessed to the target SN, and the last serving SN to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses based on the data forwarding proposals of the MN and the selected candidate SN. If applicable, the last serving SN to start normal data forwarding, timing is due to implementation. If the last serving SN is not configured as a candidate SN, the MN may trigger the MN initiated SN Release procedure to inform the last serving SN to stop providing user data to the UE, and triggers the Xn-U Address Indication procedure to inform the last serving SN the address of the SN of the selected candidate PSCell and if applicable, starts normal data forwarding.

28.	If applicable, a PDU Session path update procedure is triggered by the MN.

29. The MN may send the UE Context Release message to inform the source SN to release radio and C-plane related resources associated to the UE context if no LTM candidate PSCell(s) exist in the source SN. Any ongoing data forwarding may continue.

NOTE 9:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>