3GPP TSG RAN WG2#132	R2-2509463

Dallas, USA, 17th - 21st, November 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 1082 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On introducing LP-WUS disabling indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (CATT, Nokia, Huawei, Ericsson, ZTE, Qualcomm) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RAN2 has agreed to support enabling/disabling LP-WUS for a UE with NAS signalling and assume that UE is allowed to use LPWUS in IDLE and INACTIVE without such NAS singalling. Accordingly, CN needs to inform gNB that LP-WUS is enabled/disabled per UE in IDLE or INACTIVE mode. Thus, an indication to disable LP-WUS in the Core Network Assistance Information provided by AMF for RRC INACTIVE UE should be introduced in NGAP. Also, in Xn RAN paging message, the indication of disabling LP-WUS should be introduced to facilitate the neighbour gNBs determining whether LP-WUS is disabled or not. - For LP-WUS subgroup paging by the neighbour node for UEs in RRC_INACTIVE, the NG-RAN node may send the Further Extended UE Identity Index Value to the neighbour nodes in XnAP RAN Paging message. However, it has not been captured in the description. - The Further Extended UE Identity Index value and the condition for AMF to send it to gNB have not been captured in the description. This creates issues if the gNB does not receive it and decides to use LP-WUS together with PEI or eDRX. Considering LP-WUS can be disabled by NAS, AMF should send the Further Extended UE Identity Index value if only LP-WUS is not disabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the LP-WUS Disable Indication in the Core Network Assistance Information IE provided by AMF. - Add the LP-WUS Disable Indication in the XnAP RAN paging message. - List bullets for each IE in the CN assistance information for readability. - Simplify sentences on XnAP RAN paging. - Add the Further Extended UE Identity Index Value in the XnAP RAN Paging message. - Add the Further Extended UE Identity Index Value in the Core Network Assistance Information and the condition for AMF to send it to gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - If LP-WUS is disabled over NAS but gNB still using LP-WUS subgroup paging to UE, LP-WUS indication sent by gNB is useless. Moreover, in that case, other UEs which share the same subgroup with that UE would be falsely alarmed due to the associated LP-WUS aimed towards that UE, which is not desirable. - LP-WUS subgroup paging cannot be carried out by the neighbour nodes for UEs in RRC_INACTIVE. - The gNB may need the 20 bits UE identity index value while AMF sends only the 16 bits UE identity index value for LP-WUS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1573 TS 38.413 CR 1354 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev#0 was endorsed by RAN3 in R3-258872. Rev#1 did the following changes compared with Rev#0: - Merge the CR endorsed by RAN3 in R3-258060, - Correct the style issues, - Update the cover page accordingly. |  |  |  |  |  |  |  |  |

>>>>>>>>>>>> Start of changes <<<<<<<<<<<<

### 9.2.2	Mobility in RRC_INACTIVE

#### 9.2.2.1	Overview

RRC_INACTIVE is a state where a UE remains in CM-CONNECTED and can move within an area configured by NG-RAN (the RNA) without notifying NG-RAN. In RRC_INACTIVE, the last serving gNB node keeps the UE context and the UE-associated NG connection with the serving AMF and UPF.

For a UE in RRC_INACTIVE with eDRX cycle longer than 10.24 seconds, the NG-RAN node may, based on implementation, send a request to the AMF to perform MT Communication Handling as described in TS 23.501 [3]. If the UE accesses a gNB other than the last serving gNB and upon successful UE context retrieval, the AMF considers that the UE is reachable (i.e., the MT Communicating Handling is deactivated) and triggers the MT DL data/signalling delivery upon receiving the NGAP Path Switch Request message.

If the last serving gNB receives DL data from the UPF or DL UE-associated signalling from the AMF (except the UE Context Release Command message) while the UE is in RRC_INACTIVE, it pages in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s).

Upon receiving the RAN Paging Request message from the AMF while the UE is in RRC_INACTIVE with eDRX beyond 10.24 seconds, the last serving gNB may page in its cells comprised in the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s), in order to trigger the UE to resume connection in RRC_CONNECTED state, or in RRC_INACTIVE state for DL SDT, based on the DL data size if included in the NGAP RAN Paging Request message.

Upon receiving the UE Context Release Command message while the UE is in RRC_INACTIVE, the last serving gNB may page in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s), in order to release UE explicitly.

Upon receiving the NG RESET message while the UE is in RRC_INACTIVE, the last serving gNB may page involved UEs in the cells corresponding to the RNA and may send XnAP RAN Paging to neighbour gNB(s) if the RNA includes cells of neighbour gNB(s) in order to explicitly release involved UEs.

Upon RAN paging failure, the gNB behaves according to TS 23.501 [3].

The AMF provides to the NG-RAN node the Core Network Assistance Information to assist the NG-RAN node's decision whether the UE can be sent to RRC_INACTIVE, and to assist UE configuration and paging in RRC_INACTIVE. The Core Network Assistance Information includes:

-	The UE registration area, which is taken into account by the NG-RAN node when configuring the RNA.

-	The UE specific DRX and UE Identity Index value, which are used by the NG-RAN node for RAN paging.

-	The Periodic Registration Update timer, which is taken into account by the NG-RAN node to configure Periodic RNA Update timer.

-	An indication if the UE is configured with Mobile Initiated Connection Only (MICO) mode by the AMF.

-	The Expected UE Behaviour, which is taken into account by the NG-RAN node to assist the UE RRC state transition decision.

-	The UE Radio Capability for Paging, which may be used by the NG-RAN node during RAN Paging.

-	The PEI with Paging Subgrouping assistance information, which may be taken into account by the NG-RAN node for subgroup paging in RRC_INACTIVE, except when the UE context contains an emergency PDU session in which case the PEI with Paging Subgrouping assistance information shall not be used according to TS 24.501 [28].

-	The LP-WUS with Paging Subgrouping assistance information, which is taken into account by the NG-RAN node for subgroup paging in RRC_INACTIVE.

-	The LP-WUS disable indication, which is taken into account by the NG-RAN node to disable LP-WUS to UE, including both CN based subgrouping and UE ID based subgrouping.

-	The Further Extended UE Identity Index value, if LP-WUS is not disabled, which is taken into account by the NG-RAN node for LP-WUS subgroup paging in RRC_INACTIVE.

-	The NR Paging eDRX Information, which is taken into account by the NG-RAN node to configure the RAN Paging when the NR UE is in RRC_INACTIVE.

-	The Paging Cause Indication for Voice Service, which is taken into account by the NG-RAN node to include the Paging Cause in RAN Paging for a UE in RRC_INACTIVE state.

-	The Hashed UE Identity Index Value, which is used by the NG-RAN node to determine the start point of PTW.

-	The CN support indication for MT Communication Handling, which is taken into account by the NG-RAN node when deciding to request the AMF for MT Communication Handling for a UE in RRC_INACTIVE state with long eDRX beyond 10.24 seconds as described in TS 23.501 [3].

When sending the XnAP RAN Paging to neighbour NG-RAN node(s), the PEI with Paging Subgrouping assistance information, the LP-WUS with Paging Subgrouping assistance information, the LP-WUS disable indication, the NR Paging eDRX Information for RRC_IDLE and for RRC_INACTIVE, the Paging Cause, the Hashed UE Identity Index Value and the Further Extended UE Identity Index Value may be included.

At transition to RRC_INACTIVE the NG-RAN node may configure the UE with a periodic RNA Update timer value. At periodic RNA Update timer expiry without notification from the UE, the gNB behaves as specified in TS 23.501 [3].

If the UE accesses a gNB other than the last serving gNB, the receiving gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB and may also trigger an Xn-U Address Indication procedure including tunnel information for potential recovery of data from the last serving gNB. Upon successful UE context retrieval, the receiving gNB shall perform the slice-aware admission control in case of receiving slice information and becomes the serving gNB and it further triggers the NGAP Path Switch Request and applicable RRC procedures. After the path switch procedure, the serving gNB triggers release of the UE context at the last serving gNB by means of the XnAP UE Context Release procedure.

In case the UE is not reachable at the last serving gNB, the gNB shall fail any AMF initiated UE-associated class 1 procedure which allows the signalling of unsuccessful operation in the respective response message. It may trigger the NAS Non Delivery Indication procedure to report the non-delivery of any non PDU Session related NAS PDU received from the AMF as specified in TS 38.413 [26].

If the UE accesses a gNB other than the last serving gNB and the receiving gNB does not find a valid UE Context, the receiving gNB can perform establishment of a new RRC connection instead of resumption of the previous RRC connection. UE context retrieval will also fail and hence a new RRC connection needs to be established if the serving AMF changes.

A UE in the RRC_INACTIVE state is required to initiate RNA update procedure when it moves out of the configured RNA. When receiving RNA update request from the UE, the receiving gNB triggers the XnAP Retrieve UE Context procedure to get the UE context from the last serving gNB and may decide to send the UE back to RRC_INACTIVE state, move the UE into RRC_CONNECTED state, or send the UE to RRC_IDLE. In case of periodic RNA update, if the last serving gNB decides not to relocate the UE context, it fails the Retrieve UE Context procedure and sends the UE back to RRC_INACTIVE, or to RRC_IDLE directly by an encapsulated RRCRelease message.

>>>>>>>>>>>> End of changes <<<<<<<<<<<<