3GPP TSG-RAN WG2 Meeting #132	 R2-2509465

# Dallas, TX, USA, November 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0567 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous LPP Corrections [GNSS LOS/NLOS] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core, LCS_NAVIC_L1_SPS_NR_LTE-Core, TEI18, NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Since the IEs NR-DL-TDOA-ProvideLocationInformation, NR-DL-TDOA-RequestLocationInformation, NR-DL-AoD-ProvideLocationInformation, NR-DL-AoD-RequestLocationInformation is used for both, UE-assisted and UE-based mode, “location measurements” in the description should be replaced by “location information” that is usually used to mean both, location estimate and location measurement. - The unit "meters" should be changed to SI unit "metres" (as used consistently in LPP) at various places. - "device" should be changed to "target device" at various places. - The field description for refEphReq in IEs GNSS-SSR-OrbitCorrectionsSet2Req, GNSS-SSR-ClockCorrectionsSet2Req, GNSS-SSR-URA-Set2Req is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - “location measurements” in the description is replaced by “location information”. - The unit "meters" is changed to "metres". - "device" is changed to "target device". - The field description for refEphReq is completed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors in the specifications will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.2, 6.5.2.4, 6.5.10.3, 6.5.10.5, 6.5.11.3, 6.5.11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.5.2.2	GNSS Assistance Data Elements

[…]

#### –	KlobucharModelParameter

-- ASN1START

KlobucharModelParameter ::= SEQUENCE {

dataID			BIT STRING (SIZE (2)),

alfa0			INTEGER (-128..127),

alfa1			INTEGER (-128..127),

alfa2			INTEGER (-128..127),

alfa3			INTEGER (-128..127),

beta0			INTEGER (-128..127),

beta1			INTEGER (-128..127),

beta2			INTEGER (-128..127),

beta3			INTEGER (-128..127),

...,

[[	alfa2Ext-r19		INTEGER (-512..511)			OPTIONAL,  -- Need ON

alfa3Ext-r19		INTEGER (-2048..2047)		OPTIONAL,  -- Need ON

beta2Ext-r19		INTEGER (-1024..1023)		OPTIONAL,  -- Need ON

beta3Ext-r19		INTEGER (-8192..8191)		OPTIONAL   -- Need ON

]]

}

-- ASN1STOP

KlobucharModelParamater field descriptions

dataID When dataID has the value ′11′ it indicates that the parameters have been generated by QZSS, and the parameters have been specialized and are applicable within the area defined in [7]. When dataID has the value ′01′ it indicates that the parameters have been generated by BDS, and UE shall use these parameters according to the description given in 5.2.4.7 in [23], [50]. When the dataID has the value ′10′, it indicates that the parameters have been generated by NavIC, and UE shall use these parameters according to the description given in [38], [55]. When dataID has the value ′00′ it indicates the parameters are applicable worldwide [4], [7].

alfa0 This field specifies the 0 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 2-30 seconds.

alfa1 This field specifies the 1 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 2-27 seconds/semi-circle.

alfa2, alfa2Ext This field specifies the 2 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 2-24 seconds/semi-circle2. If dataID has the value ‘10’ and if alfa2Ext field is present and supported by the target device, the target device shall ignore the field alfa2. Otherwise the target device shall ignore the field alfa2Ext.

alfa3, alfa3Ext This field specifies the 3 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 2-24 seconds/semi-circle3. If dataID has the value ‘10’ and if alfa3Ext field is present and supported by the target device, the target device shall ignore the field alfa3. Otherwise the target device shall ignore the field alfa3Ext.

beta0 This field specifies the 0 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 211 seconds.

beta1 This field specifies the 1 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 214 seconds/semi-circle.

beta2, beta2Ext This field specifies the 2 parameter of the Klobuchar model, as specified in [4], [23], [38, [50], [55]. Scale factor 216 seconds/semi-circle2. If dataID has the value ‘10’ and if beta2Ext field is present and supported by the target device, the target device shall ignore the field beta2. Otherwise the target device shall ignore the field beta2Ext.

beta3, beta3Ext This field specifies the 3 parameter of the Klobuchar model, as specified in [4], [23], [38], [50], [55]. Scale factor 216 seconds/semi-circle3. If dataID has the value ‘10’ and if beta3Ext field is present and supported by the target device, the target device shall target ignore the field beta3. Otherwise the device shall ignore the field beta3Ext.

[…]

#### –	GNSS-LOS-NLOS-GridPoints

The IE GNSS-LOS-NLOS-GridPoints is used by the location server to provide a list of grid point coordinates or an array of correction points ("grid") for which specific assistance data can be provided.

-- ASN1START

GNSS-LOS-NLOS-GridPoints-r18 ::= SEQUENCE {

gridPointsSetID-r18				INTEGER (0..16383),

horizontalGridPoints-r18		ArrayOfGridPoints-r18,

referenceAltitudeFine-r18		INTEGER (0..9)					OPTIONAL, -- Need OP

verticalGridPoints-r18			VerticalGridPoints-r18			OPTIONAL, -- Cond 3D

...

}

ArrayOfGridPoints-r18 ::= SEQUENCE {

referencePointLatitude-r18		INTEGER (-16777216.. 16777215),

referencePointLongitude-r18		INTEGER (-33554432.. 33554431),

numberOfStepsSouth-r18			INTEGER (0.. 255),

numberOfStepsEast-r18			INTEGER (0.. 255),

stepSouth-r18					SpatialDelta-r18,

stepEast-r18					SpatialDelta-r18,

bitmaskOfGrids-r18				CHOICE {

bog16-r18						BIT STRING (SIZE(16)),

bog64-r18						BIT STRING (SIZE(64)),

bog256-r18						BIT STRING (SIZE(256)),

...

}																OPTIONAL, -- Need OP

...

}

VerticalGridPoints-r18 ::=SEQUENCE {

referenceAltitudeCoarse-r18		INTEGER (-50..900),

numberOfStepsDown-r18			INTEGER (1..3),

stepDown-r18					SpatialDelta-r18,

upperValidityDeltaAltitude-r18	SpatialDelta-r18				OPTIONAL, -- Need OP

lowerValidityDeltaAltitude-r18	SpatialDelta-r18				OPTIONAL, -- Need OP

...

}

SpatialDelta-r18 ::= ENUMERATED {n1, n2, n3, n4, n5, n10, n20, n50, n100}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| 3D | This field is mandatory present if a 3D grid is provided; otherwise it is absent. |

GNSS-LOS-NLOS-GridPoints field descriptions

gridPointsSetID This field provides the ID of the spatial grid point set. It is a regionally unique arbitrary number that is used by the UE to ensure that provided assistance data associated to a spatial grid point set is being applied to the correct set of points. The grid point set ID identifies a grid defined by a reference point corresponding to the northwest corner (of the upper layer in case of a 3D grid), where the rest of the grid is defined by a number of steps and step lengths in the south, east (down in case of a 3D grid). The grid is valid within the horizontal perimeter of the grid and between an upper and lower validity altitude, where the upper validity altitude is (referenceAltitudeFine + 10* referenceAltitudeCoarse + upperValidityDeltaAltitude), and the lower validity altitude is (referenceAltitudeFine + 10* referenceAltitudeCoarse – numberOfStepsDown*stepDown – lowerValidityDeltaAltitude).

referencePointLatitude This field specifies the latitude for the reference point, expressed in the range of -90° , +90°, coded as a number between -224 and 224-1, coded in 2's complement binary on 25 bits. The relation between the latitude X in the range [-90°, 90°] and the coded number N is: N=X90∘224 where denotes the greatest integer less than or equal to x (floor operator). The reference point defines the northwest corner of the grid point array.

referencePointLongitude This field specifies the longitude for the reference point, expressed in the range -180°, +180°, coded as a number between -225 and 225-1, coded in 2's complement binary on 26 bits. The relation between the longitude X in the range [-180°, 180°) and the coded number N is: N=X180∘225 The reference point defines the northwest corner of the grid point array.

numberOfStepsSouth, numberOfStepsEast, numberOfStepsDown These fields specify the number of steps for south, east and down direction respectively.

stepSouth, stepEast, stepDown These fields specify the spacing of the grid points for south, east and down respectively. Enumerated values n1, n2, n3, n4, n5, n10, n20, n50, n100 correspond to 1, 2, 3, 4, 5, 10, 20, 50, 100 metres, respectively.

bitmaskOfGrids This field specifies the availability of grid data at the horizontal grid points in the array and applies to all altitude layers of the grid. If a specific bit is enabled (set to '1'), the grid is available. Only the first (numberOfStepsSouth+1)×(numberOfStepsEast+1) bits are used, the remainder are set to '0'. Starting with the northwest corner of the array (top left on a north oriented map) the grid points are enumerated with row precedence – first row west to east, second row west to east, until last row west to east – ending with the southeast corner of the array. If the field is omitted all grid points are used and none omitted.

referenceAltitudeFine If this field is present and - the field referenceAltitudeCoarse is also present, it provides the fine resolution of the 3D grid altitude of the upmost layer, or - the field referenceAltitudeCoarse is absent, it provides the altitude above ground level of the 2D grid, with a scale factor of 1m. If the field is absent, the default value is 0m.

referenceAltitudeCoarse This field is present if a 3D grid is provided and specifies the coarse altitude, scale factor 10m, of the upmost layer of the grid relative to the WGS84 ellipsoid. If this field is absent, a 2D grid is provided, valid for ground level.

upperValidityDeltaAltitude This field, if present, specifies the upper validity altitude relative to the grid upper layer altitude. Enumerated values n1, n2, n3, n4, n5, n10, n20, n50, n100 correspond to 1, 2, 3, 4, 5, 10, 20, 50, 100 metres, respectively. If the field is absent, the default value is 0m.

lowerValidityDeltaAltitude This field, if present, specifies the lower validity altitude relative to the lowest grid layer altitude. Enumerated values n1, n2, n3, n4, n5, n10, n20, n50, n100 correspond to 1, 2, 3, 4, 5, 10, 20, 50, 100 metres, respectively. If the field is absent, the default value is 0m.

[…]

#### –	NavModel-NavIC-KeplerianSet2

The IE NavModel-NavIC-KeplerianSet2 is used for NavIC L1 as defined in [55].

-- ASN1START

NavModel-NavIC-KeplerianSet2-r19	 ::= SEQUENCE {

navicL1-Toec-r19		INTEGER (0..2047),

navicL1-URAI-r19		INTEGER (-16..15),

navicL1-DeltaA-r19		INTEGER	(-33554432..33554431),

navicL1-Adot-r19		INTEGER	(-33554432..33554431),

navicL1-DeltaN0-r19		INTEGER (-262144..262143),

navicL1-DeltaNdot-r19	INTEGER (-4194304..4194303),

navicL1-M0-r19			INTEGER (-4294967296..4294967295),

navicL1-E-r19			INTEGER (0..8589934591),

navicL1-W-r19			INTEGER (-4294967296..4294967295),

navicL1-Omega0-r19		INTEGER (-4294967296..4294967295),

navicL1-OmegaDot-r19	INTEGER (-16777216..16777215),

navicL1-I0-r19			INTEGER (-4294967296..4294967295),

navicL1-IDot-r19		INTEGER (-16384..16383),

navicL1-Cis-r19			INTEGER (-32768..32767),

navicL1-Cic-r19			INTEGER (-32768..32767),

navicL1-Crs-r19			INTEGER (-8388608..8388607),

navicL1-Crc-r19			INTEGER (-8388608..8388607),

navicL1-Cus-r19			INTEGER (-1048576..1048575),

navicL1-Cuc-r19			INTEGER (-1048576..1048575),

...

}

-- ASN1STOP

NavModel-NavIC-KeplerianSet2 field descriptions

navicL1-Toec Parameter toec, time-of-ephemeris and clock in seconds as described under clause 6.1.2 in [55]. Scale factor 300 seconds.

navicL1-URAI Parameter User Range Accuracy Index (in metres). This is a one-sigma estimate of the user range errors in the navigation data for the transmitting satellite as described under clause 6.1.2 in [55].

navicL1-DeltaA Parameter ∆A, semi major axis difference at reference time (in metres) as described under clause 6.1.2 in [55]. Scale factor 2-9 metres.

navicL1-Adot Parameter A ̇, change rate in semi major axis at reference time (in metres/sec) as described under clause 6.1.2 in [55]. Scale factor 2-21 metres/sec.

navicL1-DeltaN0 Parameter n0, mean motion difference at reference time (semi-circles/sec) as described under clause 6.1.2 in [55]. Scale factor 2-44 semi-circles/second.

navicL1-DeltaNdot Parameter ∆n ̇, rate of mean motion difference at reference time (semi-circles/sec) as described under clause 6.1.2 in [55]. Scale factor 2-57 semi-circles/second.

navicL1-M0 Parameter M0, mean anomaly at reference time (semi-circles) as described under clause 6.1.2 in [55]. Scale factor 2-32 semi-circles.

navicL1-E Parameter e, eccentricity as described under clause 6.1.2 in [55]. Scale factor 2-34.

navicL1-W Parameter ω, argument of perigee as described under clause 6.1.2 in [55]. Scale factor 2-32 semi-circles.

navicL1-Omega0 Parameter Ω0, longitude of ascending node of orbit plane at weekly epoch (semi-circles) as described under clause 6.1.2 in [55]. Scale factor 2-32 semi-circles.

navicL1-OmegaDot Parameter Ω ̇, rate of change of right ascension (semi-circles/sec) as described under clause 6.1.2 in [55]. Scale factor 2-44 semi-circles/second.

navicL1-I0 Parameter i0, inclination angle at reference time (semi-circles) as described under clause 6.1.2 in [55]. Scale factor 2-32 semi-circles.

navicL1-IDot Parameter i ̇, rate of change of inclination angle (semi-circles/sec) as described under clause 6.1.2 in [55]. Scale factor 2-44 semi-circles/second.

navicL1-Cis Parameter Cis, amplitude of the sine harmonic correction term to the angle of inclination (radians) as described under clause 6.1.2 in [55]. Scale factor 2-30 radians.

navicL1-Cic Parameter Cic, amplitude of the cosine harmonic correction term to the angle of inclination (radians) as described under clause 6.1.2 in [55]. Scale factor 2-30 radians.

navicL1-Crs Parameter Crs, amplitude of the sine harmonic correction term to the orbit radius (metres) as described under clause 6.1.2 in [55]. Scale factor 2-8 metres.

navicL1-Crc Parameter Crc, amplitude of the cosine harmonic correction term to the orbit radius (metres) as described under clause 6.1.2 in [55]. Scale factor 2-8 metres.

navicL1-Cus Parameter Cus, amplitude of the sine harmonic correction term to the argument of latitude (radians) as described under clause 6.1.2 in [55]. Scale factor 2-30 radians.

navicL1-Cuc Parameter Cuc, amplitude of the cosine harmonic correction term to the argument of latitude (radians) as described under clause 6.1.2 in [55]. Scale factor 2-30 radians.

[…]

#### 6.5.2.4	GNSS Assistance Data Request Elements

[…]

#### –	GNSS-SSR-OrbitCorrectionsSet2Req

The IE GNSS-SSR-OrbitCorrectionsSet2Req is used by the target device to request the GNSS-SSR-OrbitCorrectionsSet2 assistance from the location server.

-- ASN1START

GNSS-SSR-OrbitCorrectionsSet2Req-r17 ::= SEQUENCE {

refEphReq-r17					ENUMERATED { b1c,..., navicL1-v1900},

gnss-SSR-OrbitCorrectionsSet2Req-r17

GNSS-SSR-OrbitCorrectionsReq-r15	OPTIONAL,	-- Cond OC2-Req

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| OC2-Req | The field is mandatory present if the target device requests GNSS-SSR-OrbitCorrectionsSet2; otherwise it is not present. |

GNSS-SSR-OrbitCorrectionsSet2Req field descriptions

refEphReq This field indicates the reference broadcast ephemeris for which the GNSS-SSR-OrbitCorrectionsSet2 are requested.

#### –	GNSS-SSR-ClockCorrectionsSet2Req

The IE GNSS-SSR-ClockCorrectionsSet2Req is used by the target device to request the GNSS-SSR-ClockCorrectionsSet2 assistance from the location server.

-- ASN1START

GNSS-SSR-ClockCorrectionsSet2Req-r17 ::= SEQUENCE {

refEphReq-r17					ENUMERATED { b1c,..., navicL1-v1900},

gnss-SSR-ClockCorrectionsSet2Req-r17

GNSS-SSR-ClockCorrectionsReq-r15	OPTIONAL,	-- Cond CC2-Req

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| CC2-Req | The field is mandatory present if the target device requests GNSS-SSR-ClockCorrectionsSet2; otherwise it is not present. |

GNSS-SSR-ClockCorrectionsSet2Req field descriptions

refEphReq This field indicates the reference broadcast ephemeris for which the GNSS-SSR-ClockCorrectionsSet2 are requested.

#### –	GNSS-SSR-URA-Set2Req

The IE GNSS-SSR-URA-Set2Req is used by the target device to request the GNSS-SSR-URA-Set2 assistance from the location server.

-- ASN1START

GNSS-SSR-URA-Set2Req-r17 ::= SEQUENCE {

refEphReq-r17					ENUMERATED { b1c,..., navicL1-v1900},

...

}

-- ASN1STOP

GNSS-SSR-URA-Set2Req field descriptions

refEphReq This field indicates the reference broadcast ephemeris for which the GNSS-SSR-URA-Set2 is requested.

[…]

#### 6.5.10.3	NR DL-TDOA Location Information

#### –	NR-DL-TDOA-ProvideLocationInformation

The IE NR-DL-TDOA-ProvideLocationInformation is used by the target device to provide NR DL-TDOA location information to the location server. It may also be used to provide NR DL-TDOA positioning specific error reason.

-- ASN1START

NR-DL-TDOA-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-DL-TDOA-SignalMeasurementInformation-r16

NR-DL-TDOA-SignalMeasurementInformation-r16

OPTIONAL,

nr-dl-tdoa-LocationInformation-r16		NR-DL-TDOA-LocationInformation-r16

OPTIONAL,

nr-DL-TDOA-Error-r16					NR-DL-TDOA-Error-r16			OPTIONAL,

...,

[[

nr-DL-TDOA-SignalMeasurementInstances-r17

SEQUENCE (SIZE (1..maxMeasInstances-r17)) OF

NR-DL-TDOA-SignalMeasurementInformation-r16

OPTIONAL,	-- Cond batchUEA

nr-DL-TDOA-LocationInformationInstances-r17

SEQUENCE (SIZE (1..maxMeasInstances-r17)) OF

NR-DL-TDOA-LocationInformation-r16

OPTIONAL	-- Cond batchUEB

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| batchUEA | The field is optionally present if the field nr-DL-TDOA-SignalMeasurementInformation is absent; otherwise it is not present. |
| batchUEB | The field is optionally present if the field nr-dl-tdoa-LocationInformation is absent; otherwise it is not present. |

[…]

#### 6.5.10.5	NR DL-TDOA Location Information Request

#### –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location information from a target device.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1),

dl-PRS-RSCPD-Request-r18 (2)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17			LOS-NLOS-IndicatorType1-r17,

granularity-r17	LOS-NLOS-IndicatorGranularity1-r17,

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL, -- Need ON

multiMeasInSameReport-r17					ENUMERATED { requested }		OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-JointMeasurementRequest-r18	SEQUENCE {

nr-DL-PRS-JointMeasurementRequestedPFL-List-r18		SEQUENCE (SIZE (2..3)) OF

INTEGER (0..nrMaxFreqLayers-1-r16)	OPTIONAL  -- Need ON

}																			OPTIONAL, -- Need ON

nr-DL-PRS-RxHoppingRequest-r18			SEQUENCE {

nr-DL-PRS-RxHoppingTotalBandwidth-r18		CHOICE {

fr1											ENUMERATED {mhz40, mhz50, mhz80, mhz100},

fr2											ENUMERATED {mhz100, mhz200, mhz400}

}																		OPTIONAL -- Need ON

}																			OPTIONAL -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRP-Pair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }	OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }		OPTIONAL  -- Need ON

]],

[[

timingReportingGranularityFactorExt-r18		INTEGER (-6..-1)				OPTIONAL, -- Need ON

nr-DL-PRS-MeasurementTimeWindowsConfig-r18

NR-DL-PRS-MeasurementTimeWindowsConfig-r18			OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The dl-PRS-RSCPD-Request means that the target device is requested to provide DL RSCPD measurement.

nr-AssistanceAvailability This field indicates whether the target device may request additional DL-PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTEG-Request This field, if present, indicates that the target device is requested to provide the nr-UE-Rx-TEG-ID in IE NR-DL-TDOA-SignalMeasurementInformation.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-TDOA-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in fields nr-AdditionalPathList or nr-AdditionalPathListExt.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-DL-TDOA-SignalMeasurementInstances (in the case of UE-assisted mode is requested) or nr-DL-TDOA-LocationInformationInstances (in the case of UE-based mode is requested) in IE NR-DL-TDOA-ProvideLocationInformation.

nr-DL-PRS-JointMeasurementRequest This field, if present, indicates that the target device is requested to perform joint measurement across aggregated PFLs.

nr-DL-PRS-JointMeasurementRequestedPFL-List This field, if present, indicates the target device is requested to perform joint measurements on the indicated two or three PFLs. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on.

nr-DL-PRS-RxHoppingRequest This field, if present, indicates that the target device is requested to use DL-PRS Rx hopping for performing RSTD, RSRP (if requested in nr-RequestedMeasurements), or RSRPP measurements (if requested in nr-RequestedMeasurements) and report the hopping information used for performing the measurements. This field is not included when dl-PRS-RSCPD-Request or nr-DL-PRS-JointMeasurementRequest is included.

nr-DL-PRS-RxHoppingTotalBandwidth This field, if present, indicates the total bandwidth in MHz across all hops for the DL-PRS measurement. If the configured value nr-DL-PRS-RxHoppingTotalBandwidth is larger than the configured DL-PRS bandwidth in the provided assistance data, the UE shall assume that the actual nr-DL-PRS-RxHoppingTotalBandwidth is equal to the width of the configured DL-PRS in the provided assistance data.

maxDL-PRS-RSTD-MeasurementsPerTRP-Pair This field specifies the maximum number of DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers. When requested for aggregated measurements by the location server, this field specifies the maximum number of aggregated DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers.

timingReportingGranularityFactor, timingReportingGranularityFactorExt This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) and value (-6..-1) corresponds to (kMinus6..kMinus1) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff. The UE shall ignore timingReportingGranularityFactor when the timingReportingGranularityFactorExt is included.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL-PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-RxTEG-Request should also be present. When the location server requests aggregated measurements, the target device is requested to measure the same aggregated DL-PRS Resources of a TRP with N different UE Rx TEGs.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46]. When requested for aggregated measurements by the location server, this field indicates processing of reduced number of samples for the aggregated measurements.

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability. When requested for aggregated measurements by the location server, this field indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability for the aggregated measurements.

nr-DL-PRS-MeasurementTimeWindowsConfig This field indicates DL-PRS Resource Set(s) occurring within time window(s) for performing measurements where the time window is indicated by a start time, periodicity, offset and duration.

[…]

#### 6.5.11.3	NR DL-AoD Location Information

#### –	NR-DL-AoD-ProvideLocationInformation

The IE NR-DL-AoD-ProvideLocationInformation is used by the target device to provide NR DL-AoD location information to the location server. It may also be used to provide NR DL-AoD positioning specific error reason.

-- ASN1START

NR-DL-AoD-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-DL-AoD-SignalMeasurementInformation-r16

NR-DL-AoD-SignalMeasurementInformation-r16

OPTIONAL,

nr-dl-AoD-LocationInformation-r16		NR-DL-AoD-LocationInformation-r16

OPTIONAL,

nr-DL-AoD-Error-r16						NR-DL-AoD-Error-r16				OPTIONAL,

...,

[[

nr-DL-AoD-SignalMeasurementInstances-r17	SEQUENCE (SIZE (1..maxMeasInstances-r17)) OF

NR-DL-AoD-SignalMeasurementInformation-r16	OPTIONAL,	-- Cond batchUEA

nr-DL-AoD-LocationInformationInstances-r17	SEQUENCE (SIZE (1..maxMeasInstances-r17)) OF

NR-DL-AoD-LocationInformation-r16			OPTIONAL	-- Cond batchUEB

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| batchUEA | The field is optionally present if the field nr-DL-AoD-SignalMeasurementInformation is absent; otherwise it is not present. |
| batchUEB | The field is optionally present if the field nr-dl-AoD-LocationInformation is absent; otherwise it is not present. |

[…]

#### 6.5.11.5	NR DL-AoD Location Information Request

#### –	NR-DL-AoD-RequestLocationInformation

The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location information from a target device.

-- ASN1START

NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-AoD-ReportConfig-r16					NR-DL-AoD-ReportConfig-r16,

...,

[[

multiMeasInSameReport-r17					ENUMERATED { requested }	OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-RxHoppingRequest-r18			SEQUENCE {

nr-DL-PRS-RxHoppingTotalBandwidth-r18		CHOICE {

fr1											ENUMERATED {mhz40, mhz50, mhz80, mhz100},

fr2											ENUMERATED {mhz100, mhz200, mhz400}

}																		OPTIONAL -- Need ON

}																			OPTIONAL -- Need ON

]]

}

NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16		INTEGER (1..8)				OPTIONAL, -- Need ON

...,

[[

maxDL-PRS-RSRP-MeasurementsPerTRP-r17		INTEGER (9..24)				OPTIONAL, -- Need ON

maxDL-PRS-RSRPP-MeasurementsPerTRP-r17		INTEGER (1..24)				OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17			SEQUENCE {

type-r17	LOS-NLOS-IndicatorType1-r17,

granularity-r17

LOS-NLOS-IndicatorGranularity1-r17,

...

}							OPTIONAL, -- Need ON

reducedDL-PRS-ProcessingSamples-r17			ENUMERATED { requested, ... }

OPTIONAL, -- Need ON

lowerRxBeamSweepingFactor-FR2-r17			ENUMERATED { requested }	OPTIONAL  -- Need ON

]],

[[

nr-DL-PRS-MeasurementTimeWindowsConfig-r18

NR-DL-PRS-MeasurementTimeWindowsConfig-r18	OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional DL-PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

multiMeasInSameReport This field, if present, indicates that the target device is requested to provide multiple measurement instances in a single measurement report; i.e., include the nr-DL-AoD-SignalMeasurementInstances (in the case of UE-assisted mode is requested) or nr-DL-AoD-LocationInformationInstances (in the case of UE-based mode is requested) in IE NR-DL-AoD-ProvideLocationInformation.

nr-DL-PRS-RxHoppingRequest This field, if present, indicates that the target device is requested to perform DL-PRS Rx hopping for performing RSRP or RSRPP measurements and report the hopping information used for performing the measurements.

nr-DL-PRS-RxHoppingTotalBandwidth This field, if present, indicates the total bandwidth of all hops in MHz. If the configured value nr-DL-PRS-RxHoppingTotalBandwidth is larger than the configured DL-PRS bandwidth in the provided assistance data, the UE shall assume that the actual nr-DL-PRS-RxHoppingTotalBandwidth is equal to the width of the configured DL-PRS in the provided assistance data.

maxDL-PRS-RSRP-MeasurementsPerTRP This field specifies the maximum number of DL-PRS RSRP measurements on different DL-PRS Resources from the same TRP. If this field with -r17 suffix is present, the field with -r16 suffix should not be present.

maxDL-PRS-RSRPP-MeasurementsPerTRP This field specifies the maximum number of DL-PRS RSRPP measurements on different DL-PRS Resources from the same TRP.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-AoD-SignalMeasurementInformation.

reducedDL-PRS-ProcessingSamples This field, if present and set to 'requested', indicates that the target device is requested to perform the requested measurements with reduced number of samples (M=1 or M=2) as specified in TS 38.133 [46].

lowerRxBeamSweepingFactor-FR2 This field, if present, indicates that the target device is requested to use a lower Rx beam sweeping factor than 8 for FR2 according to UE's capability.

nr-DL-PRS-MeasurementTimeWindowsConfig This field indicates DL-PRS Resource Set(s) occurring within time window(s) for performing measurements where the time window is indicated by a start time, periodicity, offset and duration.

[…]