3GPP TSG-RAN WG2 Meeting #75bis	R2- 114949

10 - 14 October 2011, Zhuhai, China

Agenda item:	    7.6.1.2

Source:	 ZTE

Title:	 Further discussion on DRX mechanism for IDC

Document for:		Discussion and Decision

- Introduction

At RAN2#75 meeting, DRX based TDM solution is believed to be a feasible solution[1] both for the long IDC interference patterns[2] and the shorter IDC interference patterns[3]. And in the WI approved at RAN#53[4], one key objective of the DRX based TDM solution is to identify and evaluate if enhancements for existing DRX mechanism are needed to better handle coexistence issues.

In this contribution, DRX solution for the long IDC interference patterns and DRX solution for the shorter IDC interference patterns are analyzed separately to see whether enhancement for existing DRX mechanism needed or not.

- Discussion

According to TR36.816, with the DRX based solution, the scheduling period for LTE corresponds to the active time of DRX operation defined in section 5.7 [5], while unscheduled period (period for ISM) corresponds to the inactive time [1]. The active time defined in current DRX includes [5]:

Table 1 Active time

- onDurationTimer① or drx-InactivityTimer ② or drx-RetransmissionTimer③ or mac-ContentionResolutionTimer ④(as described in subclause 5.1.5) is running; or - a Scheduling Request is sent on PUCCH and is pending (as described in subclause 5.4.4); or ⑤ - an uplink grant for a pending HARQ retransmission can occur and there is data in the corresponding HARQ buffer; or ⑥ - a PDCCH indicating a new transmission addressed to the C-RNTI of the UE has not been received after successful reception of a Random Access Response for the preamble not selected by the UE (as described in subclause 5.1.4). ⑦

With the above definition of active time, some detail analysis is given in the following to see whether the existing DRX mechanism is sufficient to ensure the scheduling period/unscheduled period for long IDC interference patterns and shorter IDC interference patterns.

  - DRX solution for long IDC interference patterns

Except with a fixed duration of onDurationTimer, the scheduling period may be extended by the following events:

Event1: New transmission assignment (②) or retransmission monitoring (③⑥)

Event2: SR pending (⑤)

Event3: RACH related (④⑦)

In other words, the unscheduled period (i.e. scheduling period for ISM) may not be guaranteed with the above three events. Besides, the HARQ feedback is independent of DRX operation. That is UE (LTE) can receive and transmit HARQ feedback during the unscheduled period, which will cause interference between LTE and ISM.

Take the final eNB configured DRX with 128ms cycle and 50ms onDurationTimer in TR36.816 for example, the above three events are analyzed one by one to see whether the existing DRX mechanism is sufficient to ensure the scheduling period/unscheduled period as well as maximize the LTE performance as much as possible.

Event1 and HARQ feedback:

As analyzed in [2], with IDC tuned DRX configuration (maxHARQ-Tx = 2, drx-RetransmissionTimer = 1) and some kind of scheduling restriction, e.g. the eNB stops scheduling UE RTT ms ahead of the onDurationTimer expires, the unscheduled period could be guaranteed and the LTE/ISM performance (e.g. overall performance such as data rate and packet error rate) will be maximized.

Event2:

If a Regular BSR has been triggered but with no UL resources allocated/configured, a SR will be triggered. And once a SR is sent on PUCCH, UE (LTE) will be in DRX active until a MAC PDU is assembled and this PDU includes a BSR which contains buffer status up to (and including) the last event that triggered a BSR [5]. If the SR is triggered and sent near the end of onDurationTimer (as illustrated in Figure 1), the active time (scheduling period) will be extended that the unscheduled period can’t be guaranteed. Take FDD for consideration, the unscheduled period may be reduced 16ms at most.

Figure 1

With the DRX configuration assumption in Figure 1, some simple timeline analysis between the DRX configuration and the possible SR configuration could be done. And we can find that the issue can hardly be resolved by well alignment of DRX On-duration and PUCCH-SR resource, e.g. for FDD, make the PUCCH-SR resource always falls into the on-duration time that is 16ms (8ms for SR pending and 8ms for the possible UL transmission for the UL packet which make the SR be canceled, assuming maxHARQ-Tx = 2) ahead before the expiration of the onDurationTimer. Besides, although for some SR periodicity (e.g. 40ms and 80ms), the rate of PUCCH-SR which falls into the last 16ms of the On-duration time is quite low, such as 1 occasion per 5 DRX cycle, for all the other SR configurations e.g. SR periodicity 1/2/5/10/20 with whatever the offset is, the rate of PUCCH-SR which falls into the last 16ms of the On-duration time is dramatically high, such as several occasions per DRX cycle. In this way, the unscheduled period can’t be guaranteed at all, e.g. with 21% reduction for the worst case.

It may be argued that the eNB could always configure 40ms /80 ms SR in this case and the SR will not always happen to be triggered near the end of onDurationTimer. Whereas, the configuration of the PUCCH-SR density depends on the packet delay budget as well as PUCCH channel burden. The trigger of Regular BSR depends on the traffic flow and there is no PUSCH for the Regular BSR RTT ms before the onDurationTimer expires to ensure the unscheduled period as analyzed for event1 and HARQ feedback. Thus it can’t be precluded that in some cases, the unscheduled period will be dramatically reduced.

The issue indicated above may be resolved by smart eNB implementation, e.g. the eNB sends a MAC CE to end the onDurationTimer 16ms ahead its expiration. Or with dsr-TransMax configured larger than 1, the definition of the scheduling period could be refined as ‘the scheduling period for LTE corresponds to the active time of DRX operation defined in section 5.7 [5] , with the exception of the active time caused by SR.

Event3:

There are 5 events to trigger RACH procedure in the RRC-CONNECTED state:

-	RRC Connection Re-establishment procedure;

-	Handover;

-	UL data arrival during RRC_CONNECTED requiring random access procedure, e.g. UL non-synchronised;

-	DL data arrival during RRC_CONNECTED requiring random access procedure, e.g. UL non-synchronised;

-	For positioning purpose during RRC_CONNECTED requiring random access procedure;

In case the RRC Connection Re-establishment is triggered, to re-establish the RRC connection as soon as possible, LTE should have priority over ISM and thus the DRX based TDM solution should be stopped or suspended.

For the above last two events, the eNB could delay initiating the RACH procedure if the eNB foresees that the RACH procedure could not be finished before the ending of scheduling period.

For the case of handover and UL data arrival, it is can’t be precluded that the handover will take place or UL data will arrive just before the last several millisecond of the On-duration time as illustrated in Figure 2. The handover can’t be delayed or else the UE may suffer RLF. Besides, the time UE waiting for the RA Response is out of the scope of the DRX mechanism, i.e. is not the active time. Thus, Msg2 will be interfered by coexistence interference likely and RLF occurs too. To ensure the LTE performance and consider that the occurrence of handover/UL data arrival issue in Figure 2 is quite rare, it is reasonable to make RACH procedure prioritized and no enhancement for DRX event3 is needed.

Figure 2

Proposal1: According to the above analysis, the current DRX mechanism could well handle the long pattern coexistence issue thus no further enhancement of DRX is needed. However, regarding the active time extended by SR, the definition of the scheduling period may be refined as ‘the scheduling period for LTE corresponds to the active time of DRX operation defined in section 5.7 [5] ,with the exception of the active time caused by SR . And besides, it is reasonable to make the RACH procedure prioritized over the coexistence transmission, or saying, the scheduling period should include the RACH time.

  - DRX solution for short IDC interference patterns

DRX was also analyzed and believed that it could be used also for short IDC interference patterns [3][6][8]. As indicated in [7], there may have two kinds of HARQ bitmap for the short IDC interference patterns, one with the length of 10ms and the other with the length of 30ms.

DRX patterns for some HARQ bitmaps with length of 10ms are given in [6] [8]. In the Table 2 below, some potential issues of these DRX patterns are illustrated and analyzed.

Table 2 Potential issues of DRX patterns for 10ms HARQ bitmaps

| Issues | configurations | impacts | potential resolution |
| --- | --- | --- | --- |
| Issues1: UL subframes reserved for BT change to be reserved for LTE | TDD configuration 1 BT slave | As illustrated in Figure 3 Annex, BT will be interfered(note1) | The eNB stops schedule UE on subframe#7 and #8 according to the UE informed HARQ bitmap. |
| Issues2: UL subframes reserved for LTE not in the DRX active time. | TDD configuration 0/3/5 BT slave | SRS/SR could not be sent at all | The eNB schedules UE e.g. on subframe#3 / #4 (TDD configuration3 and if SRS is configured). And define that the scheduling period includes active time and the scheduled UL subframes. Or Define that the scheduling period includes active and the UL subframes indicated in the UE reported HARQ bitmap. |
| Issues3: eNB must ensure all the DL subframes during On-duration be scheduled | TDD configuration 6 BT master[8] | Restriction on eNB scheduler |  |
| Issues4: active time extended by SR | All configurations except for TDD configuration 0/1, BT master | The time for BT transmission can’t be ensured at all and interference remains(note1) | The scheduling period should not include the active time caused by SR. |
| Issues5: active time extended by RACH | All configurations except for TDD configuration 0/1, BT master | interference or RLF in LTE (note1) | As analyzed in section 2.1, it is reasonable to make the RACH procedure prioritized over the coexistence transmission, or saying, the scheduling period should include the RACH time (note2). |
| Note1: To at least a pair of clean BT Tx/Rx instances in each BT interval, the simulation assumption in TR36.816 B.1.1 is adopted. Note2: Besides the analysis in section 2.1, for the short IDC interference pattern, e.g. with BT voice, the voice traffic transmitted by BT is actually from/to LTE. So it is even more reasonable to make the RACH procedure prioritized over the coexistence transmission. |  |  |  |

As analyzed above, the scheduling period/unscheduled period achieved from the DRX solution can’t well fit the HARQ bitmap at all. Thus, if to use DRX solution for short IDC interference patterns, the 5 issues shown above should be resolved. And to resolve the above problems, if the eNB must schedule UE according to the UE indicated HARQ bitmap and the scheduling period should include the active time plus the actually scheduled UL subframes, it seems that the DRX solution for short IDC interference patterns is actually a HARQ bitmap solution indeed.

Proposal2: If to use DRX solution for 10ms short IDC interference patterns, to well fit the HARQ bitmap, the issues shown in Table 2 should be considered.

DRX patterns for some HARQ bitmaps with length of 30ms is illustrated in Figure 6 and some potential issues of these DRX patterns are illustrated and analyzed in Table 3.

Table 3 Potential issues of DRX patterns for 30ms HARQ bitmaps

| configurations | bitmap [7] | DRX config | issues |
| --- | --- | --- | --- |
| TDD2 BT master | 111110111111111111110111111111 | OFFSET=6,CYCLE=15,ONDURATION=11 | New DRX cycle 15 should be introduced. |
| TDD3 BT master | 111111110111111110111111111011 | OFFSET=9,CYCLE=10,ONDURATION=5 | Compared to the original bitmap, will waste one DL subframe per DRX cycle. |
| TDD4 BT master | 111111011110111101111111111011 | OFFSET=7,CYCLE=10,ONDURATION=4 | 1. Compared to the original bitmap, may waste up to 50% DL subframe in a DRX cycle. 2、Interference not resolved, shown in Figure 4 Annex. |
| TDD5 BT master | 111110101111111011110111110111 | OFFSET=8,CYCLE=10,ONDURATION=6 | 1. Compared to the original bitmap, may waste up to 25% DL subframe in a DRX cycle. 2、Interference not resolved, shown in Figure 5 Annex. |

As analyzed in Table 3, if taken DRX solution for IDC interference patterns with length of 30ms, some new kind of DRX cycle maybe introduced and the interference may not be totally resolved.

Proposal3: If to use DRX solution for 30ms short IDC interference patterns, some new kind of DRX cycle maybe introduced and the interference may not be totally resolved.

- Conclusion

Proposal1: According to the above analysis, the current DRX mechanism could well handle the long pattern coexistence issue thus no further enhancement of DRX is needed. However, the scheduling period should not include the active time caused by SR pending. And besides, it is reasonable to make the RACH procedure prioritized over the coexistence transmission, or saying, the scheduling period should include the RACH time.

Proposal2: If to use DRX solution for 10ms short IDC interference patterns, to well fit the HARQ bitmap, the issues shown in Table 2 should be considered.

Proposal3: If to use DRX solution for 30ms short IDC interference patterns, some new kind of DRX cycle maybe introduced and the interference may not be totally resolved.

- Reference

- R2-114785 Agreements on IDC interference avoidance Rapporteur (CMCC)

- R2-114782 Analysis of DRX solution for IDC interference avoidance Ericsson ST-Ericsson

- R2-114784 Solutions for IDC interference in LTE + BT voice scenario Ericsson ST-Ericsson

- RP-111355 Signalling and procedure for interference avoidance for in-device coexistence CMCC

- 36.321 Medium Access Control (MAC) protocol specification

- R2-114331 Solutions for IDC interference in LTE + BT voice scenario Ericsson ST-Ericsson

- R2-114783 Corrections to timeline analysis Intel Corporation, Qualcomm Incorporated, ZTE, Pantech

- Email discussion [75#12] – LTE: Capture agreements on in-device coexistence interference avoidance [CMCC]

- Annex

Figure 3 (offset = 0)

Figure 4

Figure 5

Figure 6

Just one possible DRX configuration, there could be other DRX configurations for the suggested TDM pattern in TR36.816 Figure 5.2.1.2.1-1.

BT will also be interfered by LTE for a lot of timing offsets.