3GPP TSG-RAN WG2 #79	 R2-123293

13th to 17th August 2012, Qingdao, China

Agenda Item:	8

Source: 	CMCC

Title:  	Quasi fast return from GSM to UTRA TDD after voice service

Document for:	Discussion and Decision

Introduction

For operators deploying both UTRA TDD and GSM networks, it is expected that UTRA TDD cell has higher priority for UE to camp on. In real network, there is such requirement that when UE is handed over to GSM with voice service from UTRA TDD, and then UE needs to fast return to UTRA TDD once voice service terminates. During the procedure of returning to UTRA TDD, UE will experience certain period of unreachable time, i.e. the UE cannot be paged. In this contribution, we firstly analyze the unreachable time and then give some potential solutions to optimize the procedure.

Discussion

Problem in returning to UTRA TDD after voice service

For a UE that supports both UTRA TDD and GSM, it usually gives higher priority to camp on a UTRA TDD cell, and when there is a need for voice service, the network will hand over the UE to the GSM cell. After finishing the call, according to the current procedure, UE will firstly try to camp on the GSM cell, performing LAU/ RAU. After successfully camping on GSM, the UE will start to search for and reselect to UTRA TDD cell, and performs LAU/ RAU again. During these procedures, there are around 9 seconds unreachable time in GSM and 1-5 seconds unreachable time in UTRA TDD during which period the UE cannot be paged, which significantly degrade user experience. The illustration is shown in Fig. 1.

Fig. 1 illustration of unreachable time

Possible Solutions

Solution 1：FR solution

FR solution is the existing standardized solution for UE fast returning to UTRA TDD. In FR solution, when UE hangs up, the BSC sends the channel release message with neighbor cell information of UTRA TDD. When UE receives the release message, it will select to UTRA TDD cell based on those information. The unreachable time is around 1~5 seconds. A brief summary of FR is shown in Fig. 2.

Fig. 2 illustration of FR solution

Solution 2：Quasi fast return solution

Quasi fast return solution has not been standardized, but it has been widely applied in existing UTRA TDD network, and almost all the terminals with GSM and UTRA TDD support it. In this solution, UE can in advance acquire the cell information needed for returning to UTRA TDD through system information broadcasted by the UTRA cell where the voice service is initiated. Notice that this cell information is not based on the neighbor cell information broadcasted in SIB11 but rather an extension of cell selection and re-selection information of UTRA TDD SIB3 [1] to include all the potential frequency information of UTRA TDD of the operator, as well as the RSCP threshold of each primary frequency. Because the voice service can spend a very long time and the UE may be far away from that UTRA TDD cell when UE hangs up, the neighbor cell information in SIB11 may be invalid then.

The procedure is described as follows:

- When UE camps on UTRA TDD cell, it reads and stores the frequency information and RSCP thresholds broadcasted in SIB3.

- When UE hands over to GSM for voice service, simultaneously it will perform measurements on those stored UTRA TDD carrier frequencies, and prioritize the carrier frequencies whose RSCPs are higher than their RSCP thresholds based on their RSCP values from high to low.

- After UE finishes the voice service, it will not perform LAU and RAU, but rather search for a suitable cell to camp on from the UTRA TDD frequency with the highest priority. If no suitable cell can be found, UE will try to find a suitable cell on the UTRA TDD frequency with the second highest priority, and so on.

- After trying three frequencies, if still no suitable cell can be found, UE camps on GSM, and perform LAU and RAU in GSM. Fig. 3 illustrates this solution.

Fig. 3 quasi-fast return solution

Based on the above introduction of the two solutions, we can see that both of the solutions have nearly the same unreachable time: 1~5 seconds. The only difference between them is that FR needs the upgrade of BSCs whereas quasi-fast return solution needs the upgrade of RNCs. Furthermore, for FR solution, BSCs should know the neighbor cell information of UTRA TDD which means the related information should be transferred from UTRA to GSM. From operator’s point of view, we prefer quasi-fast return solution, as the investment in GSM is not in line with operator’s evolution strategy. Therefore, we kindly ask RAN2 to support quasi-fast return in UTRA TDD. The related modification of TS 25.331 is shown in the CR R2-123295.

Proposal: Quasi-fast return solution should be supported in UTRA TDD.

Conclusions and Proposal

In this document, we describe the scenario and requirement of UE returning to UTRA TDD from GSM and figure out the problem of unreachable time in this procedure. Then we list two potential solutions and make the following proposal:

Proposal: Quasi-fast return solution should be supported in UTRA TDD.

References

[1] 3GPP TS25.331 V11.1.0, Radio Resource Control (RRC); protocol specification (Release 11), March, 2012.