3GPP TSG RAN WG2 Meeting #79bis	R2-124438

Bratislava, Slovakia, 8 – 12 Oct 2012

Agenda Item:	7.8

Source: 	CMCC

Title:  	Consideration on returning from GERAN/UTRAN to E-UTRAN following CS fallback

Document for:	Discussion and Decision

Introduction

CS fallback, a solution that enables single radio multi-mode terminals to switch from E-UTRAN PS to UTRAN/GERAN CS for traffics like voice, helps operators avoid huge investment in 2G/3G CS to support voice call strategy for their initial E-UTRAN/EPC deployment. Nonetheless, extensive tests show that it suffers a severe problem when the UE is reselecting back to E-UTRAN after CSFB without any context of E-UTRAN cell information. In this contribution, we try to elaborate on this issue.

Discussion

Supporting voice service in E-UTRAN can be categorized into two phases. In the first phase, E-UTRAN is on the way of development, and its coverage is too limited to provide continuous voice experience. Operators need 2G/3G to carry guaranteed voice service by virtue of their vast coverage, and E-UTRAN primarily handles data traffic. While, in the second stage, E-UTRAN is fully developed, VoLTE/SRVCC will play the key role in voice traffic.

CSFB offers a solution to meet requirement of the first phase. To be specific, the UE works on E-UTRAN mode with high priority. When there is MO/MT voice call, the network moves the UE to 2G/3G by handover, CCO or redirection. When the voice call finishes, UE should return to E-UTRAN as soon as possible. Yet, the problem of CSFB is how to return to E-UTRAN swiftly. With regarding to GSM, according to the current procedure, upon hanging up Rel-10 and onwards UE will firstly takes 2~4 seconds to try to camp on the GSM cell, synchronizing to it and reading its system information, during which the UE is unreachable, i.e., it cannot be paged. And if the LA of the new cell is different from the stored one, UE takes another 3~4 unreachable seconds to perform LAU/RAU. Notice that if it is a Rel-9 or earlier UE, LAU/RAU should always be performed irrespective of whether LA is changed or not. Following that, UE has to undergo around 1 second unreachable time to search for and reselect to LTE cell. Then it performs TAU. Therefore, during the whole procedure, there is about 3~9 seconds unreachable time—an unacceptable degradation of user experience. An illustration of the procedure is shown in Fig.1 (As to UTRAN, the case is similar).

Fig. 1 illustration of unreachable time

To solve this problem, redirection on channel release message of GERAN (or RRC release message of UTRAN) is proposed, i.e., cell selection related information of E-UTRAN is conveyed in channel release message of GERAN(or RRC release message of UTRAN). When hanging up, the UE will directly search and try to camp on an E-UTRAN cell according to the received information from channel or RRC release.

However this redirection solution requires additional update of numerous BSCs or RNCs to support redirection function, which is costly and not in tune with operator’s evolution strategy, especially considering that CSFB is just as a transient solution holding merit merely in a short time. From operator point of view, a preferred solution should meet the following two requirements:

- UE should swiftly return to E-UTRAN after voice call finishes and the unreachable time should be limited to 1~2 seconds.

- The solution should just involve E-UTRAN and should not require modification in GERAN or UTRAN network, which means an uniform fast return solution applicable for both UTRAN->LTE and GSM->LTE cases.

Therefore, we propose that:

Proposal: RAN2 is kindly asked to consider only E-UTRAN side solutions for fast returning to E-UTRAN after CS fallback from GERAN or UTRAN.

Conclusions and Proposal

In this document, we describe the importance of CSFB in the initial deployment of E-UTRAN, and highlight the problem in switching back to E-UTRAN after CSFB. Then we make the following proposal:

Proposal: RAN2 is kindly asked to consider only E-UTRAN side solutions for fast returning to E-UTRAN after CS fallback from GERAN or UTRAN.