3GPP TSG RAN WG2 Meeting #80	R2-125712R2-12

# New Orleans, U.S.A., 12-16 November, 2012

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | CRNum | rev | - | Current version: | 8.17.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on measurements upon Re-establishment when MBSFN subframes are configured |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE-L23 |  |  |  |  |  |  | Date: |  | 2012-11-01 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | When UE experiences RLF and performs reestablishment on a neighbor cell, it continues to use the neighCellConfig configured by the first cell. If the first configured neighCellConfig = ‘10’, and the neighbor cell’s MBSFN subframe set is different from that of the first cell, the UE can derive a wrong set of subframes as MBSFN subframes at the first cell. This can result in incorrect RRM measurements of the first cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Actions at handover and reestablishment are changed so that UE sets neighCellConfig to ‘00’ at reestablishment. Impact Analysis Impacted functionality: Reestablishment procedure Inter-operability: No inter-operability issue is forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may perform (and report) RRM measurements in MBSFN subframes of neighbor cells upon reestablishment until RRC reconfiguration with the correct neighCellConfig is received. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.5.6	Measurement related actions

#### 5.5.6.1	Actions upon handover and re-establishment

E-UTRAN applies the handover procedure as follows:

-	when performing the handover procedure, as specified in 5.3.5.4, ensure that a measObjectId corresponding to the handover target carrier frequency is configured as a result of the procedures described in this sub-clause and in 5.3.5.4;

E-UTRAN applies the re-establishment procedure as follows:

-	when performing the connection re-establishment procedure, as specified in 5.3.7, ensure that a measObjectId corresponding to the target carrier frequency is configured as a result of the procedure described in this sub-clause and the subsequent connection reconfiguration procedure immediately following the re-establishment procedure;

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the triggerType is set to 'periodical':

3>	remove this measId from the measIdList within VarMeasConfig:

1>	if the procedure was triggered due to inter-frequency handover or successful re-establishment to an inter-frequency cell, update the measId values in the measIdList within VarMeasConfig as follows:

2>	if a measObjectId value corresponding to the target carrier frequency exists in the measObjectList within VarMeasConfig:

3>	for each measId value in the measIdList:

4>	if the measId value is linked to the measObjectId value corresponding to the source carrier frequency:

5>	link this measId value to the measObjectId value corresponding to the target carrier frequency;

4>	else if the measId value is linked to the measObjectId value corresponding to the target carrier frequency:

5>	link this measId value to the measObjectId value corresponding to the source carrier frequency;

2>	else:

3>	remove all measId values that are linked to the measObjectId value corresponding to the source carrier frequency;

- if the procedure was triggered due to a reestablishment

2> set neighCellConfig to ‘00’;

1>	remove all measurement reporting entries within VarMeasReportList;

1>	reset the periodical reporting timer or timer T321, whichever one is running, as well as associated information (e.g. timeToTrigger) for all measId;

1>	release the measurement gaps, if activated;

NOTE:	If the UE requires measurement gaps to perform inter-frequency or inter-RAT measurements, the UE resumes the inter-frequency and inter-RAT measurements after the E-UTRAN has setup the measurement gaps.