3GPP TSG-RAN WG2 Meeting #80	R2-125723

New Orleans, LA, USA,  Nov. 12th -  16th, 2012

Agenda item:	10.3.2

Source:		InterDigital Communications

Title:		Simplification of HARQ re-transmission scenario description in 25.321

Document for:	Discussion

# Introduction

At the last RAN2 meeting in Bratislavia, a number of documents on the impact of UL MIMO on the RAN2 specifications were discussed and a large number of agreements were made.  While good progress on the CRs to the MAC specifications was made during the meeting and the ensuing e-mail discussions, a number of details still remain to be discussed.

In this contribution, we discuss a few outstanding issues related to the draft CR to 25.321 in [1].

# Discussion

During the UL MIMO and 64QAM work item, RAN1 has determined a number of rules for operations of the HARQ for dual-stream transmission.  In an LS to RAN2 in [2], RAN1 has described its agreement and summarized the behavior in a table in the Annex B, reproduced below in Table 1 for convenience:

Table 1: HARQ re-transmission scenarios (from [2])

| CASE | USED/PREFERRED RANK |  | ACK/NACK |  | OTHERS | Behaviour |
| --- | --- | --- | --- | --- | --- | --- |
|  | Original Tx | Re-Tx | TB1 | TB2 | Power/Buffer* Limited | Re-Tx handling |
| 1 | 1 | 1 | nack | - | - | LEGACY |
| 2 | 1 | 2 | nack | - | - | Force re-transmission in Rank 1 |
| 3 | 2 | 1 | nack | nack | - | Re-transmit in Rank 2 |
| 4 | 2 | 2 | nack | nack | - | Re-transmit in Rank 2 |
| 5 | 2 | 1 | ack | nack | no | Re-transmit in Rank1 using primary stream |
| 6 | 2 | 2 | ack | nack | no | Re-transmit stream 2 and new data on stream 1 |
| 7 | 2 | 1 | nack | ack | no | Re-transmit in Rank1 |
| 8 | 2 | 2 | nack | ack | no | Re-transmit stream 1 and new data on stream 2 |
| 9 | 2 | 1 | ack | nack | Buffer/power | Re-transmit in Rank1 using primary stream |
| 10 | 2 | 2 | ack | nack | Buffer | Re-transmit in Rank1 using primary stream |
| 10b | 2 | 2 | ack | nack | Power | New data on primary stream, Re-Tx on the secondary stream if sufficient power available to select a new TB to be transmitted in parallel to the Re-Tx. Otherwise only the retransmission is sent on the primary stream. |
| 11 | 2 | 1 | nack | ack | Buffer/power | Re-transmit in Rank1 |
| 12 | 2 | 2 | nack | ack | Buffer | Re-transmit in Rank1 |
| 12b | 2 | 2 | nack | ack | power | New data on secondary stream, Re-Tx on the primary stream, if sufficient power available to select a new TB to be transmitted in parallel to the Re-Tx. Otherwise only the retransmission is sent on the primary stream. |

In an attempt to help the understanding of the UL MIMO HARQ operations, this table was inserted in the draft CR to TS25.321.  The table above, captures the retransmission behavior in case of power and buffer limitations, however, doesn’t capture the behavior in case of grant limitation, which also needs to be taken into account.  Additionally, the table refers to power and buffer limitations, which are not commonly used in any of the 3GPP Specifications and would add some complexity to explain what they mean.  During email discussion [3] it was proposed that we could replace the column on buffer/power/serving grant limited by “if E-TFC selection allows rank-2 transmission” .

With that in mind, a careful analysis of the table from a RAN2 perspective shows that a number of entries in the table can be merged, which may lead to further simplifications and better understanding.  .

Table 2 is a more streamlined summary of Table 1 that we feel is more suited to be included in the MAC specifications .  More specifically, the following simplications to Table 1 were made:

- Case 1 is legacy single stream behavior and therefore doesn’t need to be captured

- Case 2 is captured in table 2, to show that no new TB will be created if in that TTI if the corresponding HARQ process had been transmitted using rank-1, even if the maximum allowed rank is set to 2.

- Cases 3 and 4 result in the same outcome.  These two entries are summarized as “Case 2” of Table 2.

- Cases 5, 7, 9, 11 are all linked to the more general case where after a rank-2 transmission one stream is NACKed and needs to be retransmitted while the UE maximum rank is changed to “1”.  These 4 entries lead to the UE retransmitting using the primary stream and have been merged in “Case 3” of Table 2.

- Cases 6, 8, 10, 10b, 12 and 12b are all linked to the more general case where one stream is retransmitting when rank-2 transmission is allowed.  These 6 entries are summarized under “Case 4”  in the proposed table.  We group the buffer, power, and grant limitations and refer to the general concept of “rank-2 is allowed” pointing to the E-TFC selection procedure for more details.  In E-TFC selection, rank-2 will be allowed if the power, serving grant, and buffer allows the UE to meet the minimum requirements for rank-2 transmission.

- Finally, the “Other” column has been removed as it carried no meaning in the new table.

Table 2: Summary of UL MIMO HARQ re-transmission cases

| Case | Rank |  | ACK/NACK |  | Behavior upon retransmission |
| --- | --- | --- | --- | --- | --- |
|  | Actual rank upon transmission | Maximum allowed rank at the time of retransmission | First stream TB | Second stream TB |  |
| 1 | 1 | 2 | NACK | - | No new TB on secondary stream (e.g. re-transmit in rank1) |
| 2 | 2 | 2 or 1 | NACK | NACK | Re-transmit in rank2 |
| 3 | 2 | 1 | All other cases (e.g. one ACK one NACK) |  | Re-transmit in rank1 on primary stream. |
| 4 | 2 | 2 | All other cases (e.g. one ACK one NACK) |  | Re-transmit on NACK-ed stream and transmit new data on the other stream if rank-2 is allowed by E-TFC selection (e.g. section 11.8..4 x). Otherwise re-transmit in rank1 on primary stream. |

Proposal:	Agree to replace Table 11.8.1.1.x.1 in the Draft CR by Table 2.

# Conclusion

In this contribution, we propose a few changes to the draft CR for the Introduction of UL MIMO and 64QAM to TS25.321.  Further, we propose:

Proposal:	Agree to replace Table 11.8.1.1.x.1 in the Draft CR by Table 2.

# References

- R2-125064, DRAFT – Introduction of UL MIMO with 64QAM in TS 25.321

- R2-123244, “LS on RAN1 agreements on MIMO with 64QAM for HSUPA,” 3GPP TSG RAN WG2 #79, Qingdao, China, 13th -17th August, 2012.

- [79bis#10] [UMTS/64QAM] UL MIMO and 64 QAM CRs