3GPP TSG-RAN WG2 Meeting #82	R2-131649

Fukuoka, Japan, 20th to 24th May 2013

Agenda item:		5.1.2

Source:	ZTE

Title:	Discussion on reduce power consumption in WLAN scanning

Document for:		Discussion and Decision

- Introduction

After RAN2#81bis meeting, the 81bis#11 email discussion about WLAN scanning and power consumption was conducted. Many companies presented their points of view on 4 issues:

- Relation to ANDSF

- Solutions for UEs in IDLE and CONNECTED mode

- WLAN access network discovery assistance information

- How the network obtains information about WLAN APs

In this paper, we further discuss how to reduce power consumption in WLAN scanning.

- Discussions

There are two kinds of WLAN scan, one is active scan and the other is passive scan. The passive scan requires the device to listen on each channel it support to try to receive the Beacon frame that a WLAN AP broadcast. The active scan requires the device to send the Probe request frame on each channel it support and try to receive the Probe response frame that the WLAN AP replies. Both procedures need the whole band transmitting and/or listening. If the WLAN scaning process is not appropriate, the power consumption will be huge and the energy is wasted. After last RAN2#81bis meeting, there are 3 solution directions has been captured in the TR [1]. In these 3 directions, at least RAN could provide some assist information to the UE, and the 3rd issue in the 81bis#11 email discussion is about the WLAN access network discovery assistance information. In the email discussion, many companies give different WLAN related information that maybe help reducing UE’s power consumption in WLAN scanning. Here we present some useful WLAN related information and discuss the possible improvement, also with the possible changes for the current specification.

Scenario 1

Figure 1 3gpp and WLAN interworking network diagram example 1

In Figure 1, a 3gpp and WLAN interworking network diagram example 1 is showed: UE1 is in the coverage of AP1 and eNB, UE2 is in the coverage of eNB. Both UE support interaction with ANDSF, if the access network discovery information ANDSF give include two WLANs, AP1 and AP2, and the priority of AP2 is higher than AP1. Based on this information, UE1 may first use the active scan for AP2, and it is beyond AP2’s coverage, so all the trying for AP2 is wasted, then UE1 may scan for AP1. Or even UE1 already discovery AP1 and the signal is good enough, it may still scan for AP2. In this case, all the power consumption in scaning for AP2 is wasted. In the case of UE2, it will waste the enery try to find both AP1 and AP2 since it is beyond both AP’s coverage.

To solve the problem mentioned above, RAN may provide UE with some assist information to help optimize the WLAN scaning process. The eNB could get some WLAN property information from OAM or other way, the WLAN property information may include WLAN AP location, using channel information and the transmit power of WLAN AP. The eNB could predict the WLAN AP coverage based on these informations and the signal fading model. The eNB could provide the WLAN AP list with three enteries which are WLAN ID, WLAN channel information and the WLAN coverage (AP location and coverage radius) to the WLAN interworking enable UE. The UE could amend its scan range based on UE’s current position and the information received from RAN.

Also, for the above example of UE1, it may also help if we define that if UE find a fine WLAN, it do not need to scan for the better WLAN.

Observation 1: RAN should provide the WLAN ID, WLAN channel and the WLAN coverage information to the UE to assist the WLAN scanning.

Proposal1: UE could amend its scan range based on UE’s current position and the information received from RAN in order to save energy.

Scenario 2

Figure 2 3gpp and WLAN interworking network diagram example 2

In Figure2, another 3gpp and WLAN interworking network diagram example 2 is showed. UE1 is under the coverage of AP1, AP2 and eNB, it could get the service quality of WLAN1 and WLAN2, which include the signal and the load situation. In this example, WLAN2 is provide service to more device than AP2, the load in WLAN2 is relatively heavy and it can not provide good service to the new comer. As the UE2 moves, it may begin to scan for WLAN to access, and it may know there are two WLANs through the assist information received from eNB. Since the WLAN can not provide good service, the power consumption for scanning for WLAN2 will be wasted.

In order to solve the problem mentioned above, the best solution is for the eNB to tell the UE2 the load situation of different WLAN, or provide the priority of WLAN considering the load situation. If UE2 knows the load situation of each WLAN, it may skip scanning for WLAN2 and direct to scan for AP1.

Observation 2: RAN should provide the load information (actual load or load indicator or WLAN priority) of WLANs to the UE to assist the WLAN scanning.

Another thing need to be noted is that how the eNB get the load information of the WLAN. In the Beacon frame or the Probe response frame, there is a BSS load element in the frame, since the load information is dynamically changing and the OAM could provide the relatively static information about the WLAN to the eNB, so the convenient way is for the UE to report the load information of WLAN to the eNB. Although this may increase a specific UE’s power consumption, this may be benefit to the whole network. And the network needs to minimize the power consumption of the specific UE. If network knows that a specific UE has already access to a specific WLAN, it may configure this UE to report the load information about the WLAN, or if the network knows a UE is under a WLAN’s coverage or under many WLAN’s coverage, it may configure the UE to report the load information of one or many WLAN.

Proposal 2: eNB should configure some specific UE to report the load information of a specific WLAN.

As we discussed above, it will help if RAN provide the load information or priority of the different WLANs, since the ANDSF also may give UE a priority of different WLAN, it should be guarantee that RAN’s priority is not conflicted with ANDSF’s policy. In current network architecture, there is no interface between ANDSF and RAN, it’s best for the eNB to know the ANDSF’s policy, such as priority, blacklist or forbidden behavior etc. This should also need eNB configure some ANDSF function enabled UE to report these information.

Proposal 3: eNB should configure some UE to report the ANDSF information.

- Conclusions

Observation 1: RAN should provide the WLAN ID, WLAN channel and the WLAN coverage information to the UE to assist the WLAN scanning.

Proposal1: UE could amend its scan range based on UE’s current position and the information received from RAN in order to save energy.

Observation 2: RAN should provide the load information (actual load or load indicator or WLAN priority) of WLANs to the UE to assist the WLAN scanning.

Proposal 2: eNB should configure some specific UE to report the load information of a specific WLAN.

Proposal 3: eNB should configure some UE to report the ANDSF information.

- Reference

- RAN2#81bis Chairman Notes