3GPP TSG-RAN2 Meeting #82	R2-131805

# Fukuoka, Japan, 20-24 May 2013

# Source:	Samsung

# Title:	WLAN power consumption during scanning procedures

# Agenda item:	5.1.2

# Document for:	Discussion and Agreement

# 1. Introduction

From the SID [1] of the study item 'WLAN/3GPP Radio Interworking', RAN2 needs to take into account the unnecessary WLAN scanning problems which may drain UE battery resources. For this, we may need to investigate existing WLAN scanning mechanisms.

In this paper, we provide a brief overview of WLAN scanning procedure, and a proposal for a way forward.

# 2. Discussion

## 2.1 Passive scanning in WLAN

In WLAN, similar to transmission of System Information Block (SIB), the Beacon frame is transmistted by the Access Point (AP) periodically. The Beacon frame includes following information: Basic Service Set Identifier (BSSID), Service Set Identifier (SSID), capabilities of the AP, timestamp for synchronozation and so on. When a STA operates in a specific WLAN channel, the STA can hear the Beacon frames as well as all other frames from all APs which operate in the WLAN channel.

From this manner, passive scanning is a scanning mode that a STA listens to each WLAN channel passively, and the STA does not send any request for APs to send the Beacon frame. So, in the passive scanning mode, a STA has a possibty to miss to receive the Beacon frame if scanning time is not enough. Typical transmission periodicity of the Beacon frame is 102.4 msec: in order to receive all Beacon frames from APs in the channel, the STA needs to receive frames at least 102.4 msec. Further, since a STA has no idea the received frame is data/management/control frame until decoding the WLAN MAC header, the STA needs to receive ALL frames during the period, and perform decoding all received frames which results an increase of power consumption of an UE.

## 2.2 Active scanning in WLAN

On the contrary of passive scanning, WLAN also provides active scanning: active scanning involves the generation of Probe Request frames to request Probe Response frames (which is almost same as the Beacon frame), and the subsequent processing of received Probe Response frames [2].

Figure 1 Active scanning procedure [2]

Figure 1 shows an example of active scaning procedure in WLAN. When a STA performs active scanning, the STA shall use the following procedure [2]:

For each channel to be scanned: - Send the Probe Request frame to the broadcast destination address, with the SSID and BSSID. - Set to 0 and start a ProbeTimer. - If channel is not busy before the ProbeTimer reaches MinChannelTime, then scan the next channel. Else, process all received probe responses when ProbeTimer reaches MaxChannelTime, and then scan the next channel.

From above and the section 2.1, we can see that a STA in the active scanning mode needs to send at least one or more the Probe Request frame, and receives and processes ALL received frames during MaxChannelTime if channel is busy. In the hotspot area, many APs and STAs are likely to operate in each channel, and an UE may need to perform receiving/ processing unnecerary frames which is not destined for the UE from APs/STAs during the scanning procedure.

## 2.3 Power consumption during WLAN scanning procedures

Please note that the above procedure either of passive scanning and active scanning is for each channel, which means a STA needs to repeat the above procedure per each channel. There are several frequency bands for WLAN, e.g. 2.4 GHz, 5 GHz, and there are several frequency channels in each band [3].

- For 2.4 GHz which is used by IEEE 802.11b/g/n, there are 14 channels: but according to the regulations of each country, 13 channels are available in most of world, 11 channels are available in U.S.

- For 5 GHz which is used by IEEE 802.11a/h/j/n, there are about 40 channels, but each country applies its own regulations to both the allowable channels, allowed users and maximum power levels within these frequency bands.

Hence, if an UE supports both 2.4 GHz and 5 GHz WLAN, the UE needs to perform scanning across around 30 channels, which means the UE repeats passive/active scanning procedure 30 times. This leads considerable power consumption during WLAN scanning procedure in the UE side.

For the quantitative comparison, we can compare the power consumption in WLAN depending on the state with the power consumption in LTE.

Table 1 Power consumption depending on the MAC states of WLAN [4][5][6]

| States | Power Consumption | Description |
| --- | --- | --- |
| Off | 0 mW | Turns off the WLAN interface completely |
| Sleep | 40 mW | Turns off most parts of circuit except for critical circuit |
| Listen | 800 mW | Keeps sensing the carrier |
| Receive | 900 mW | Performs the receive operations such as demodulation |
| Transmit | 2,000 mW | Transmits the frame to the air |

Table 2 Power consumption depending on the RRC states of LTE [7]

| States | Power Consumption | Description |
| --- | --- | --- |
| RRC_IDLE | 594 mW | LTE DRX On RRC_IDLE |
| RRC_CONNECTED | 1060-3300 mW | Either transmits data or waits for data |

Table 1 and Table 2 show the power consumption of WLAN and LTE depending on the operating states, repectively. In WLAN, since a WLAN STA requires listening, receiving, and even transmistting frames during scanning procedure, the power consumption both in passive and active scanning could be considerably high.

Based on the above comparison, it can be conluded that the power consumption due to WLAN scanning procedure is not negligible, and therefore, it is desirable for an UE to reduce the number of WLAN scanning for its power saving. If (E-)UTRAN provides a mechanism for an UE to reduce the number of WLAN scanning, it would be very helpful to the power saving of the UE.

Proposal: It is proposed that (E-)UTRAN provides a mechanism for an UE to reduce power consumption during WLAN scanning procedure.

# 3. Conclusion

Proposal: It is proposed that (E-)UTRAN provides a mechanism for an UE to reduce power consumption during WLAN scanning procedure.

# 4. References

- RP-122038: New Study Item Proposal on WLAN/3GPP Radio Interworking, Intel Corporation

- IEEE Std 802.11-2012, Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications, 2012.

- "List of WLAN channels," http://en.wikipedia.org/wiki/List_of_WLAN_channels, Online Link

- Atheros Communications, Power Consumption and Energy Efficiency Comparisons of WLAN Products, white paper, April 2004.

- Youngkyu Choi and Sunghyun Choi, "Service Charge and Energy-Aware Vertical Handoff in Integrated IEEE 802.16e/802.11 Networks," in Proc. IEEE INFOCOM 2007, Anchorage, USA, May 6-12, 2007

- D. Halperin, B. Greenstein, A. Sheth, and D. Wetherall, "Demystifying 802.11n power consumption," in Proc. USENIX Workshop on Power Aware Computing and Systems, October 2010

- J. Huang, F. Qian, A. Gerber, Z. Morley Mao, S. Sen and O. Spatscheck, "A Close Examination of Performance and Power Characteristics of 4G LTE Networks," in Proc. the 10th international conference on Mobile systems, applications, and services (MobiSys'12), Ambleside, UK, June 25-29, 2012