3GPP TSG-RAN2 Meeting #84		R2-134119

11th to 15th November 2013, San Francisco, USA

Agenda Item		: 7.2.4

Source			: LG Electronics Inc.

Title			: LCP in dual connectivity

Document for		: Discussion and Decision

Introduction

In this contribution, we discuss how the LCP procedure is performed with dual connectivity, and address further issue in split bearer.

Discussion

In UP architecture 1A, radio bearers are not split; hence one radio bearer is only served by one eNB. Since each eNB manages the UL resources only for the radio bearers which are belonging to that eNB, the radio bearers served by different eNBs would not share the UL resources. Therefore, for 1A, it is straightforward that the legacy LCP procedure is independently performed for radio bearers served by the same eNB.

In UP architecture 3C, the situation is somewhat different because a radio bearer can be split into two RLC/MAC entities, and it can be served by both of the MeNB and the SeNB.

However, the LCP procedure is performed for ‘logical channels’, and it does not care about whether the radio bearer is split or not in the upper layer. From the logical channel point of view, it is clear to which eNB the data of the logical channel is transmitted.

Then, it is still straightforward that the legacy LCP procedure is applied to 3C, i.e. the LCP procedure is independently performed for logical channels whose data are to be transmitted to the same eNB.

- When the UE has a dual connectivity, the legacy LCP procedure is independently performed for logical channels whose data are to be transmitted to the same eNB.

For 3C, one issue with proposal 1 is PBR enforcement. In current specification, the PBR is configured per radio bearer not per logical channel. If we apply the legacy LCP procedure as it is, there may be a problem of excessive PBR enforcement, as addressed in [1]. Therefore, we may need a PBR partitioning method, but details can be discussed in WI phase.

- How to enforce the PBR for the split bearer in LCP procedure needs to be studied in WI phase.

Conclusions

In this contribution, we have discussed the LCP and addressed further issue on LCP. We propose as follow:

Proposal 1.	When the UE has a dual connectivity, the legacy LCP procedure is independently performed for logical channels whose data are to be transmitted to the same eNB.

Proposal 2.	How to enforce the PBR for the split bearer in LCP procedure needs to be studied in WI phase.

References

- R2-133536, LCP Impacts by Bearer Split, LG Electronics