3GPP TSG RAN WG2 #86						R2-142293

19th May to 23th May 2014, Seoul, Korea.

Agenda Item:		7.1.4

Souce:	Samsung

Title:	Text proposal for BSR in dual connectivity

Document  for:			Discussion

Introduction

In the RAN2 #83b and #85 meeting, RAN2 had agreed that BSR related to specific bearer toward the eNB which the bearer belongs to and the BSR is triggered only by triggering condition of the corresponding MAC. BSR related parameters should be independently configured for each MAC entity. In this contribution, we discuss remaining issues related to BSR parameter configuration and text changes in TS 36.321 specification for the BSR procedure.

Discussion

Agreement #83b

- For eNB-specific bearer, UE sends BSR information related to specific bearer towards the eNB for which corresponding bearer belongs to.

Agreement #85

- BSR configuration is independent per MAC entity. - When BSR triggering condition is met for a cell group, only corresponding MAC entity triggers BSR. FFS for PDCP data arrival in 3C case - LCG is defined per MAC entity - BS information is reported only to the corresponding eNB - Double scheduling of same PDCP data should be avoided.

In short, what have been agreed are

  - per-MAC entity BSR configuration

  - per-MAC entity BSR triggering

This contribution addresses remaining issues regarding BSR in dual connectivity as follow;

- Details of BSR configuration

- Details of BSR triggering/procedure text

Details of BSR configuration

Details of BSR configuration are shown in table 1 and 2 for MCG and SCG. BSR related parameters can be configured by each eNB as agreement. Thus, the BSR for MCG/MeNB is configured as legacy system like in table 1. Also, BSR for SCG/SeNB can be configured by the SeNB with helps of the MeNB since RRC function is only located on the MeNB. The SeNB determines BSR related parameters and sends it via X2 interface. How to deliver those parameters such as higher level IE and X2 message can be discussed in CP session. However, it is not necessary to introduce new value range for the SCG/SeNB and same IE should be used for an UE as noted in the table 2.

| BSR related IEs |  |  | Higher level IE (placeholder) |  |
| --- | --- | --- | --- | --- |
| periodicBSR-Timer | ON | {sf5, … , sf2560} | MAC-MainConfig | ON |
| retxBSR-Timer |  | {sf320, … , sf10240} | MAC-MainConfig | ON |
| extendedBSR-Sizes-r10 | OR | {setup} | mac-MainConfig-v1020 | ON |
| logicalChannelGroup |  | (0..3) | LogicalChannelConfig | OPTIONAL |

Table 1. BSR related parameters of MCG/MeNB

| BSR related IEs |  |  | Higher level IE (placeholder) |  |
| --- | --- | --- | --- | --- |
| periodicBSR-Timer | ? | SeNB determines the values and send via the corresponding X2 message. No need to define new value range. The same IEs should be reused. | CP should discuss which message/IE contains the parameters | CP should discuss the optionality of IEs |
| retxBSR-Timer | ? |  |  |  |
| extendedBSR-Sizes-r10 | ? |  |  |  |
| logicalChannelGroup | ? |  |  |  |

Table 2. BSR related parameters of SCG/SeNB

Proposal 1. RAN2 is asked to agree that no new BSR parameter and value range are needed and signaling detail for BSR parameters is discussed in CP session.

Details of BSR triggering/procedure text

In following text proposal based on current agreement, it is relatively straightforward such as replacing UE with a/the MAC entity for dual connectivity in text of section 5.4.5 (in TS 36.321). Furthermore, TP makes it more clear how BSR is triggered and transmitted by each MAC entity

--------------------------------------------- Text Proposal  ---------------------------------------------

5.4.5	Buffer Status Reporting

The Buffer Status reporting procedure is used to provide the serving eNB with information about the amount of data available for transmission in the UL buffers related to a MAC entity. RRC controls BSR reporting of a MAC entity by configuring the two timers periodicBSR-Timer and retxBSR-Timer and by, for each logical channel, optionally signalling logicalChannelGroup which allocates the logical channel to an LCG [8].

For the Buffer Status reporting procedure, the MAC entity shall consider, among the radio bearers mapped to the MAC entity, all radio bearers which are not suspended and may consider radio bearers which are suspended.

A Buffer Status Report (BSR) shall be triggered for a MAC entity if any of the following events occur:

-	UL data, for a logical channel which is mapped to the MAC entity and belongs to a LCG, becomes available for transmission in the RLC entity or in the PDCP entity (the definition of what data shall be considered as available for transmission is specified in [3] and [4] respectively) and either the data belongs to a logical channel with higher priority than the priorities of the logical channels which are mapped to the MAC entity and belong to any LCG and for which data is already available for transmission, or there is no data available for transmission for any of the logical channels which are mapped to the MAC entity and belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC control element plus its subheader, in which case the BSR is referred below to as "Padding BSR";

-	retxBSR-Timer expires for the MAC entity and the MAC entity has data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	periodicBSR-Timer expires for the MAC entity, in which case the BSR is referred below to as "Periodic BSR".

For Regular and Periodic BSR, a MAC entity shall:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Long BSR;

-	else report Short BSR.

For Padding BSR, a MAC entity shall:

-	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

-	if more than one LCG of the MAC entity has data available for transmission in the TTI where the BSR is transmitted: report Truncated BSR of the LCG with the highest priority logical channel with data available for transmission of the MAC entity;

-	else report Short BSR.

-	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader, report Long BSR.

If the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled in the MAC entity:

-	if the MAC entity has UL resources allocated for new transmission for this TTI:

-	instruct the Multiplexing and Assembly procedure to generate the BSR MAC control element(s);

-	start or restart periodicBSR-Timer except when all the generated BSRs are Truncated BSRs;

-	start or restart retxBSR-Timer.

-	else if a Regular BSR has been triggered:

-	if an uplink grant is not configured or the Regular BSR was not triggered due to data becoming available for transmission for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers:

-	a Scheduling Request shall be triggered.

A MAC PDU shall contain at most one MAC BSR control element, even when multiple events trigger a BSR by the time a BSR can be transmitted in which case the Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

A MAC entity shall restart retxBSR-Timer upon indication of a grant for transmission of new data on any UL-SCH mapped with the MAC entity.

All triggered BSRs of a MAC entity shall be cancelled in case the UL grant(s) in this subframe can accommodate all pending data available for transmission of the MAC entity but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader. All triggered BSRs of a MAC entity shall be cancelled when a BSR is included in a MAC PDU for transmission on any UL-SCH mapped with the MAC entity.

A MAC entity shall transmit at most one Regular/Periodic BSR in a TTI. If a MAC entity is requested to transmit multiple MAC PDUs in a TTI, it may include a padding BSR in any of the MAC PDUs which do not contain a Regular/Periodic BSR.

All BSRs generated by a MAC entity and transmitted in a TTI always reflect the buffer status after all MAC PDUs have been built for this TTI by the MAC entity. Each LCG shall report at the most one buffer status value per TTI and this value shall be reported by the MAC enity in all BSRs reporting buffer status for this LCG.

NOTE:	A Padding BSR is not allowed to cancel a triggered Regular/Periodic BSR. A Padding BSR is triggered for a specific MAC PDU only and the trigger is cancelled when this MAC PDU has been built.

-------------------------------------------- end of Text proposal --------------------------------------

Proposal 2. RAN2 is asked to agree that the text proposal is considered for TS 36.321 text change.

Conclusion

Proposal 1. RAN2 is asked to agree that no new BSR parameter and value range are needed and signaling detail for BSR parameters is discussed in CP session.

Proposal 2. RAN2 is asked to agree that the text proposal is considered for TS 36.321 text change.