3GPP TSG-RAN WG2 Meeting #86			R2-142687

Seoul, Korea, May 19 – May 23, 2014

Agenda item:	7.8

Source: 	Huawei, HiSilicon

Title: 	Discussion on HARQ RTT Timer in TDD-FDD CA

Document for:	Discussion and Decision

Introduction

According to the LS R1-141827 [1] on TDD-FDD CA outcome, RAN1 has agreed on HARQ timing for both FDD PCell and TDD PCell cases. In this contribution we analyze how the description of the HARQ RTT Timer in TS36.321 is impacted based on the RAN1 agreements.

Discussion

As we know, HARQ RTT Timer is per HARQ process of DL transmission, so we only focus on the potential impact caused by HARQ Timing of PDSCH. Based on LS R1-141827 [1], we conclude the HARQ Timing of PDSCH if TDD-FDD CA is configured, as shown in the following table:

Table 1 DL HARQ timing for TDD-FDD CA

| Cases |  | HARQ Timing of PDSCH |
| --- | --- | --- |
| FDD PCell | For FDD PCell | Follow PCell’s timing |
|  | For self-scheduled TDD SCell | Follow PCell’s timing |
|  | For cross-scheduled TDD SCell | Follow PCell’s timing |
| TDD PCell | For TDD PCell | Follow PCell’s timing |
|  | For self -scheduled FDD SCell | Follow a new table as provided in LS R1-141827 |
|  | For cross -scheduled FDD SCell | Follow PCell’s timing |

For a give cell when DRX and TDD-FDD CA are configured, the HARQ RTT Timer should be set according to the HARQ Timing of PDSCH as concluded in above table. However, in current specification, it simply states that “For FDD the HARQ RTT Timer is set to 8 subframes. For TDD the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission and the transmission of associated HARQ feedback, as indicated in Table 10.1.3.1-1 of [2]”. Obviously current description cannot be applied to TDD-FDD CA case anymore.

If we want to modify the description of the HARQ RTT Timer in TS36.321 to be applied to these two cases accurately, the updated descriptions would be most probably like as the following:

For all serving cells when FDD cell is the PCell, the HARQ RTT Timer is set to 8 subframes. For TDD PCell or cross-carrier scheduled cell when TDD cell is the PCell, the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission on the Pcell and the transmission of associated HARQ feedback, as indicated in Table 10.1.3.1-1 of [2]. For self-carrier scheduled FDD cell when TDD cell is the PCell, the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission on the FDD cell and the transmission of associated HARQ feedback, as indicated in Table 10.1.3A-1 of [2].

Obviously this kind of modification to HARQ RTT Timer as shown above is too complicated to be accepted. On the contrary, we don’t think that it is a good idea to simply delete the whole description of HARQ RTT Timer from TS 36.321or just to say “refer to …”. As a tradeoff, we propose the following descriptions for giving some guidline of HARQ RTT Timer setting in MAC specification while avoiding too much complexity for details.

For all serving cells when FDD cell is the PCell, the HARQ RTT Timer is set to 8 subframes. For all serving cells when TDD cell is the PCell, the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission on the HARQ timing reference cell and the transmission of associated HARQ feedback, as indicated in Table 10.1.3.1-1 or Table 10.1.3A-1 of [2].

Therefore, we propose that,

Proposal 1: change the descriptions of the HARQ RTT Timer in section 7.7 of TS 36.321 to:

For all serving cells when FDD cell is the PCell, the HARQ RTT Timer is set to 8 subframes. For all serving cells when TDD cell is the PCell, the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission on the HARQ timing reference cell and the transmission of associated HARQ feedback, as indicated in Table 10.1.3.1-1 or Table 10.1.3A-1 of [2] , and for an RN configured with rn-SubframeConfig [8] and not suspended, as indicated in Table 7.5.1-1 of [11].

Since this change applies to TDD-FDD CA case, we need a Rel-12 CR to capture it if it is agreed. The corresponding CR is provided in R2-142682.

Conclusion

We analyzed how the description of the HARQ RTT Timer in TS36.321 is impacted by the DL HARQ Timing if TDD-FDD CA is configured, and propose:

Proposal 1: change the descriptions of the HARQ RTT Timer in section 7.7 of TS 36.321 to:

For all serving cells when FDD cell is the PCell, the HARQ RTT Timer is set to 8 subframes. For all serving cells when TDD cell is the PCell, the HARQ RTT Timer is set to k + 4 subframes, where k is the interval between the downlink transmission on the HARQ timing reference cell and the transmission of associated HARQ feedback, as indicated in Table 10.1.3.1-1 or Table 10.1.3A-1 of [2] , and for an RN configured with rn-SubframeConfig [8] and not suspended, as indicated in Table 7.5.1-1 of [11].

Moreover, the corresponding CR is provided in R2-142682.

Reference

R1-141827, “LS on RAN1 TDD-FDD CA outcome”, RAN1

[2]    3GPP TS 36.213 Physical layer procedures