3GPP TSG-RAN WG2 #86	R2-142706

Seoul, Republic of Korea, 19th May – 23rd May 2014

Agenda Item:	10.1.2.1

Source: 	Qualcomm Incorporated

Title:  	On allowing switching from EUL to R99 DCH

Document for:	Discussion

# Introduction

During RAN#62, a Work Item on Further EUL enhancements was agreed. One of the objectives listed under the WID [1] is to introduce coverage improvements by means of enhancements to Uplink Power Headroom (UPH) measurements and TTI switching mechanism:-

- EUL coverage improvements for single and multi RAB, by means of TTI switching and UPH measurements improvements;

Whilst the discussion thus far has focused on the switching between 2ms and 10ms E-DCH TTI, one could also consider enabling faster switching between EUL and R99 DCH as a means to improve coverage. In this contribution these aspects are discussed.

# R99 DCH vs. 10ms E-DCH TTI Coverage

In this section, we present results comparing the coverage of R99 DCH and 10ms E-DCH TTI in a lab setup. The lab test procedure and parameters are listed in the Annex A.

## Single-cell

Figure 1 shows the uplink coverage comparison in a single-cell scenario.

Figure 1: Uplink Coverage with Single-cell Setup

The difference in coverage between two configurations is determined by comparing the UL attenuation (path loss) at 5% residual BLER It can be seen that UL coverage improves by ~4Db for PA3 and VA30 channels in a single-cell scenario with R99 DCH compared to E-DCH with 10ms TTI.

## SHO

Figure 2 shows the uplink coverage comparison in a SHO scenario.

Figure 2: Uplink Coverage with SHO Setup

It can be seen that UL coverage improves by ~5Db for PA3 and VA30 channels in SHO scenario with R99 DCH compared to E-DCH with 10ms TTI.

# Enabling fast TTI switch from E-DCH to R99 DCH

Based on results shown in section 2, we observe a 4-5 Db improvement in UL coverage with R99 DCH compared to E-DCH with 10ms TTI. Similar to the fast 2ms and 10ms TTI switching mechanism being introduced for E-DCH, a mechanism to switch faster from E-DCH to R99 DCH can help exploit the coverage improvement offered by R99 DCH. The agreed mechanism for fast switch between 2ms and 10ms TTI for E-DCH can largely be re-used for this purpose with the following additions/modifications:-

- UE will need to be provided R99 DCH pre-configuration in addition to 2ms and 10ms E-DCH pre-configuration

- New HS-SCCH order to switch from E-DCH to R99 DCH will need to be defined

- A mechanism to inform the non-serving NodeB of a switch from E-DCH to R99 DCH would need to be defined. The current working assumption of UE transmitting a special 18-bit E-TFC after the switch would not work as there is no such TFC defined for R99 DCH.

# Conclusion

In this contribution, we have discussed the use case of enabling faster switching between E-DCH and R99 DCH. A 4-5Db coverage improvement is observed for R99 DCH compared to 10ms TTI E-DCH. It is noted that with some modifications/additions, the current agreed mechanism for faster switching between 2ms and 10ms E-DCH TTI can be reused for this purpose. The following is proposed:-

Proposal: EUL coverage improvements should also address the use case of switching from E-DCH to R99 DCH. For this purpose, agreed mechanism and signalling for switching between 2ms and 10ms E-DCH TTI should be re-used where possible, with updates and modifications as necessary.

# References

RP-132078, Further EUL Enhancements Work Item Description

# Annex A

## Lab Setup

The coverage of R99 DCH and 10ms E-DCH TTI was compared in a lab setup.

Test Procedure:

- Configure the system with 2 Ues.

  - Set up one UE with 10ms TTI

  - Set up the other UE R99 DCH

- Set a low attenuator level: 0Db (explained below)

- Set up the PA3 channel conditions for both UE1 & UE2

- Establish PS data calls on both UE1 and UE2.

- Collect logs for 5 minutes

Repeat previous step with different uplink attenuator levels in 1Db steps so that we get 2 points with EUL residual BLER = 1% and up to high residual BLER levels or until the call cannot be sustained.

Repeat previous steps for VA30 channel.

Test Parameters:

| Parameter | Value |
| --- | --- |
| Rx diversity, UE | Off |
| UE Receiver | Rake |
| Rx diversity, Node-B | On |
| Node-Bs | 1 or 2 (For SHO) |
| Cells | 1 or 2 (For SHO); one in each Node-B |
| UEs in the system | 2 |
| UL set point | -32 to -6.7dB |
| RLC mode for PS traffic | UM |
| CQI feedback cycle | 8ms |
| CQI repetition factor | 1 |
| UL Payload for E-DCH 10ms TTI | TBS: 331, Target BLER = 1%, Target Number of Transmission = 1, Max Number of Transmissions = 2 |
| UL Payload for R99 DCH | TBS: 246, Target BLER = 1% |

Test Metric: The difference in coverage between two configurations is determined by comparing the UL attenuation (path loss) at 5% residual BLER