3GPP TSG-RAN WG2 Meeting #89	 R2-150546

09.02. to 13.02.2015, Athens, Greece

Agenda Item:	6.2.11.1

Source: 	CMCC

Title:  	Clarification for the UE behavior on cell reselection

Document for:	Discussion and Decision

Introduction

During drive test of our LTE network, cell reselection ping-pong was discovered and when we looked into it, we found that different UEs have different behaviors in the ping-pong area. It seems that different chipset vendors have different understanding of cell reselection procedure.

In this paper, we firstly rebuilt drive-testing environment and show the phenomenon, and then we would like RAN2 to make some clarification on this issue.

Discussion

In LTE, a cell specific parameter, Qrxlevmin, is defined as the minimum required RX level in the cell for UE performing cell (re)selection [1], which will be configured a different value in different cell according to the types of eNB, e.g., Macro eNB or Micro eNB,  Eight-antenna eNB or Two-antenna eNB, as well as the each cell’s traffic load taken into account. At the same time, independent Qrxlevmin is required to be configured in SIB3, SIB5, and SIB6/7, respectively, for intra-frequency, inter-frequency, and inter-RAT neighboring cells, which is a frequency specific parameter. However, it is not clear how to configure this frequency specific parameter, when there are multiple cells on one frequency with different Qrxlevmin, and some configuration set seems to cause different understanding in UE, and hence inconsistent UE behavior. Here is the case that we rebuilt according to the drive test.

There are two cells, E1 and E2. The detailed parameters of the two cells are listed in Table I. For convenience of tracing of the UE behavior in cell reselection, in the test, we have constructed an inter-frequency scenario where cell on each frequency is configured with different TAC. In the test, three UE equipped with different chipset were used.

Table I. Parameters of the Test Case

| Carrier frquency | Cell | TAC | Frequency priority | UE position | Qrxlevmin |  | RSRP(dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | SIB1 | SIB5 | Initialization | Trigger |
| 2620~2640MHz | E1 | 1 | 7 | Camping cell | -60 | -60 | -100 | -100dBm |
| 2640~2660MHz | E2 | 2 | 7 | Neighboring cell | -22 | -60 | Inactivation | Activation,-90dBm |
| Other common parameters: q-Hyst =5dB, q-offsetFreq=0, q-offsetCell=0, s-NonIntraSearch = 31 |  |  |  |  |  |  |  |  |
| Note: Cell E2 fulfills S criterion according to the Qrxlevmin in SIB5 of cell E1 |  |  |  |  |  |  |  |  |

In this test case, observation is summarized in table II as below:

Table II. Observation

| Chipset | Action in Test |
| --- | --- |
| Vendor 1 | No action in cell E2 |
| Vendor 2 | TAU in cell E2 firstly, enter idle mode immediately after TAU, and then reselect to cell E1 after checking MIB and SIB1 again, ping-pong cell reselection |
| Vendor 3 | TAU in cell E2 firstly, enter idle mode immediately after TAU, and then reselect to cell E1 after checking MIB and SIB1 again, ping-pong cell reselection |

From this observation, chipset of Vendor 1 abort cell reselection though it seems that cell E2 is more suitable to be camped according to the parameters in SIB5 of cell E1. It demonstrates this chipset has done some verification of the configuration both in SIB5 of cell E1 and SIB1 in cell E2 before deciding to camp on.

However, chipsets of Vendor 2 and Vendor 3 have triggered cell reselection to cell E2, which demonstrates that these two chipsets have only checked the configuration in cell E1 while ignoring the configuration in cell E2 at the moment of performing cell reselection. Due to cell E1 and cell E2 belongs to different TAC, UE triggers TAU when it decides to camp on cell E2, following which, UE enters idle mode and perform cell reselection back to cell E1 when it find out that cell E2 is not suitable to be camped on in fact. Ping-pong cell reselection is observed since the trigger condition is not changed.

We check with the specification, it says in [1] that “The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE not to be CSG member cells.” when the UE decides which cell to be reselected. However where the parameters of criterion colored in red are coming from is unclear, i.e., camping cell’s SIB3/5 or target cell’s SIB1. Therefore we ask RAN2 to clarify this problem.

Some may argue that it is unreasonable to configure different Qrxlevmin values in SIB3/5 in one cell and in SIB1 in the corresponding neighbor cell. But in fact the Qrxlevmin in SIB1 is cell-specific and in SIB3/5 is frequency-specific. In some scenarios, we have the requirement of configure different Qrxlevmin value for different cell, such as for different eNB type or different traffic loads.

Conclusions and Proposal

In this document, we presented our observation of UE behavior in our test of some Qrxlevmin configuration set, which shows inconsistent UE behaviors.

Proposal: RAN2 is asked to clarify where the parameters of cell selection criterion S applied in cell reselection coming from.

- [1] TS 36.304 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode