3GPP TSG-RAN WG2 Meeting #89bis 	R2-151158

# Bratislava, Slovakia, 20th – 24th April 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR |  | rev |  | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SL-DCH transmission for the autonomous resource allocation mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Panasonic |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_D2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2015-04-08 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Description of the UE behaviour for the SL-DCH transmission in the autonomous resource allocation mode is misleading. According to current version of TS36.321 UE selects randomly a complete sidelink grant from the configured resource pool, which is not the intended UE behaviour. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.14.1.1 it’s clarified that a UE determines for the autonomous resource allocation mode a sidelink grant based on a random selection of the time and frequency (position) resources used for SL-SCH Data and transmission resources for SCI from the resource pool configured by upper layers. The random function shall be such that each of the allowed selections can be chosen with equal probability. Impact analysis Impacted functionality: SL-DCH transmission functionality Inter-operability: There are no inter-operability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Description of SL-DCH transmission for the autonomous resource allocation mode remains ambigious in the specification, which may lead to some unintended and unefficient UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.14	SL-SCH Data transfer

### 5.14.1	SL-SCH Data transmission

#### 5.14.1.1	SL Grant reception and SCI transmission

In order to transmit on the SL-SCH the MAC entity must have a sidelink grant. The sidelink grant is selected as follows:

-	if the MAC entity is configured to receives a sidelink grant dynamically on the PDCCH or EPDCCH and more data is available in STCH than can be transmitted in the current SC period, the MAC entity shall:

-	using the received sidelink grant determine the set of subframes in which transmission of SCI and transmission of first transport block occur according to subclause 14.2.1 of [2];

-	consider the received sidelink grant to be a configured sidelink grant occurring in those subframes starting at the beginning of the first available SC Period which starts at least 4 subframes after the subframe in which the sidelink grant was received, overwriting a previously configured sidelink grant occurring in the same SC period, if available;

-	clear the configured sidelink grant at the end of the corresponding SC Period;

-	else, if the MAC entity is configured by upper layers to transmit using a pool of resources as indicated in subclause 5.14.4 of [8] and more data is available in STCH than can be transmitted in the current SC period and if the MAC entity does not have a configured sidelink grant, the MAC entity shall:

-	determine a sidelink grant based on a random selection of the time and frequency (position) resources used for SL-SCH Data and  transmission resources for SCI from the resource pool configured by upper layers. The random function shall be such that each of the allowed selections [2] can be chosen with equal probability;

-	use the determined sidelink grant to define the set of subframes in which transmission of SCI and transmission of first transport block occur according to subclause 14.2.1 of [2];

-	consider the determined sidelink grant to be a configured sidelink grant occurring in those subframes starting at the beginning of the first available SC Period which starts at least 4 subframes after the subframe in which the sidelink grant was determined;

-	clear the configured sidelink grant at the end of the corresponding SC Period;

NOTE:	Retransmissions on SL-SCH cannot occur after the configured sidelink grant has been cleared.

The MAC entity shall for each subframe:

-	if the MAC entity has a configured sidelink grant occuring in this subframe:

-	if the configured sidelink grant corresponds to transmission of SCI:

-	instruct the physical layer to transmit SCI corresponding to the configured sidelink grant.

-	else if the configured sidelink grant corresponds to transmission of first transport block:

-	deliver the configured sidelink grant and the associated HARQ information to the Sidelink HARQ Entity for this subframe.