3GPP TSG-RAN WG2 Meeting #90	R2-152292

# Fukuoka, Japan, May 25 – May 29, 2015

Agenda item:		7.9 (LTE_dualC_enh-Core)

Source:	LG Electronics Inc.

Title: 	PDCP reordering enhancement

Document for:	Discussion and Decision

1.	Introduction

In R12 DC, PDCP reordering is performed for split bearer (and MCG bearer if split bearer is reconfigured to MCG bearer). This document discusses potential enhancement from R12 PDCP reordering.

2.	Discussion

Figure 1. Example of R12 PDCP reordering

The Figure 1 shows an example of R12 PDCP reordering for split bearers. In this example, PDCP PDUs 3, 4, 6, 7 are received from MCG RLC and PDCP PDUs 1, 5, 8, 9 are received from SCG RLC. The PDCP PDU 2 is not received due to, e.g. X2 loss or PDCP discard. Then, the UE PDCP stores PDUs 3~9 in the reordering buffer, and delivers them to upper layer only when the reordering timer expires.

This R12 PDCP reordering behavior can be enhanced if we consider that AM RLC entity always delivers RLC SDUs in-sequence and PDCP would not retransmit PDCP PDUs without PDCP status report.

If a PDU with SN=X is received from an AM RLC entity, the UE can know that missing PDUs with SN < X will not be received from the AM RLC entity. For split bearers, if a PDU with SN=X1 is received from AM RLC 1 and a PDU with SN=X2 is received from AM RLC 2, then the UE can know that missing PDUs with SN < X1 and X2 will not be received from any of the AM RLC entities. This behavior is similar to R12 in-order delivery function where when a PDU with SN=X is received, all the missing PDUs with SN < X are considered as lost.

In the above example, the UE PDCP can know that PDU 2 is never received when PDU 5 is received from SCG RLC. The reordering can be stopped at this time and there should be no further reordering delay in delivering PDUs 3, 4, and 5.

Proposal: Discuss whether it is worth to enhance PDCP reordering mechanism for split bearers in R13.

3.	Proposal

This paper shows that R12 PDCP reordering can be enhanced considering the PDCP and AM RLC characteristics. We would like to check whether companies want to enhance R12 PDCP reordering mechanism.

Proposal: Discuss whether it is worth to enhance PDCP reordering mechanism for split bearers in R13.