3GPP TSG-RAN WG2 Meeting #90	R2-152316

Fukuoka, Japan, 25 - 29 May 2015

Source: 	Nokia Networks

Title: 					Considerations on dual PUCCH activation for non UL aligned PUCCH SCell

Agenda Item: 	7.2.3.2

Document for:	Approval

# Introduction

A revised WI for LTE Carrier Aggregation Enhancement Beyond 5 Carriers was agreed in RAN plenary in [1]. One part of the objective of the WI is to introduce the support of PUCCH on SCell:

- For Rel-12 CA configurations, specify and complete the support of PUCCH on SCell for UEs supporting uplink Carrier Aggregation.

In addition, it is listed that UE and eNB performance requirements should be specified if such are identified:

- Specify the necessary (if any) UE and eNodeB performance requirements for PUCCH on SCell for LTE Carrier Aggregation for the scenarios addressed in RAN1 and RAN2 specifications.

In this paper, we will discuss the UE and eNB impact of introduction of PUCCH on SCell  - and activating of such a cell when it is not UL time aligned.

# Discussion

When introducing the PUCCH on a SCell our understanding is, that the SCell with PUCCH is introduced using the basic CA procedures as baseline. In our view such SCell is configured by network in the same way as an existing SCell, of course with additional PUCCH configuration included. We assume same behaviour will be applied to a SCell independently whether it is configured without or with PUCCH. i.e. the SCell will be configured by the PCell and activated by the PCell. Additionally it seem logical that any UE PUCCH related UL traffic for the SCell with PUCCH will be carriered on that SCells PUCCH. This includes among other the CSI reporting for the SCell.

When an SCell has been configured it is well known that it needs to be followed by a network activation command in case the network want the UE to monitor the SCell PDCCH to enable active use of the SCell. Currently RAN4 has defined UE requirements related to SCell activation delay.

In brief summary of the activation delay requirements the UE shall be able to transmit a valid CSI report and apply actions related to the activation command for the SCell being activated no later than 24ms or 34ms depending on the conditions (e.g. related when UE sent measurement report realted to activated SCell). It should be noted that UE shall send out-of-range value CSI report already from no later than 8ms after activation command and UE may start sending valid CSI values as soon as it is ready in order to indicate NW that now I’m ready to rock.

In current CA the CSI feedback is transmitted on PCell. The understanding behind the existing requirements is that the UE will have valid UL timing on the PCell at the time of receiving the SCell activation. UE will have valid UL timing due to the need for PCell signaling the activation command and therefore the UE can always transmit the CSI report used as baseline in the activation delay requirements.

One aspect different from baseline CA is that for SCell with PUCCH this baseline assumption of having valid UL timing in the cell in which to transmit the CSI report may no longer be true.

In such a situation where the UE does not having valid UL timing in the SCell with PUCCH, the UE would not be able to transmit CSI report on PUCCH for the SCell, and it is unclear what would be the UE behavior on which to base the UE requirements.

In next section, we elaborate a bit further on the issue.

## SCell with PUCCH without valid TA

The basic difference introduced when introducing the SCell with PUCCH is that the PUCCH no longer is only carried on PCell. This includes in our view also the initial CSI report to be sent by the UE when SCell is activated. Additionally it must be assumed that the PCell and SCell with PUCCH can be non-colocated and therefore may have different UL timing (different TA). This is illustrated in figure 1 (collocated and non-colocated scenario).

Figure 1 Illustration of SCell with PUCCH with and without same TA as PCell

In one example the PCell and SCell are co-located and UL timing is likely the same and can be re-used. Therefore the TA from the PCell can be readily used also for the newly activated SCell for UL transmissions. In case the UE can re-use the TA from PCell in the activated SCell the UE can also send the CSI report according to current requirements.

In the other example, the SCell is not collocated with the PCell. In this case the UL timing can be different than the one applied in the PCell. In such situation the TA from the PCell is not valid in the SCell the UE cannot transmit the CSI

Currently the UE has to indicate to eNB that the SCell has been activated and UE is applying the actions related to an activated SCell. This requirement is assuming that the CSI report is transmitted on the PCell and UE has valid UL timing on PCell. While PUCCH SCell has PUCCH available it may not always be situation that UE is UL time aligned with the PUCCH SCell.

In order to achieve time alignment it seems likely that one needs to perform RACH. NW can already now initiate PDCCH order to start RACH, but problem is that NW will not know when UE is actually activated if UE is able to do activation faster than n+24/34. If the small delay caused of this uncertainty is not an issue then NW can initiate PDCCH order on PUCCH SCell after NW is sure that it is activated and thus start the procedure for obtaining timing advance.

Observation: NW can use existing PDCCH order to start RACH on PUCCH SCell

But if we would like to promote faster UE implementations it may be necessary to allow such a UEs to gain from their better implementation by allowing RACH to start sooner in this case:

Question: Should we introduce mechanism that allows quick RACH starting to get UL time alignment for PUCCH SCell

# Conclusions

In this paper we have discussed the UE and eNB impact of introduction of PUCCH on SCell with regard to UE requirements and eNB expectation. Specifically, we have the following observation and proposal.

Observation: NW can use existing PDCCH order to start RACH on PUCCH SCell

Question: Should we introduce mechanism that allows quick RACH starting to get UL time alignment for PUCCH SCell

# References

- RP-150277, LTE Carrier Aggregation Enhancement Beyond 5 Carriers, Nokia Corporation, NTT DoCoMo Inc., Nokia Networks