3GPP TSG-RAN WG2 Meeting #90	R2-152433

Fukuoka, Japan, 25th – 29th May 2015

Agenda item:	7.4.3

Source:		Sony

Title:		Reduced Mobility support

Document for:	Discussion and Decision

# Introduction

Complexity reduction and coverage enhancement are the main objectives of Rel.13 WID “Further LTE Physical Layer Enhancements for MTC” [1]. It is also stated that reduced mobility support can be considered if this is needed to fulfil the objectives.

In WG1 #79 Meeting in San Francisco, there have been some discussions related to the MTC mobility support. The outcome so far is that RAN1 recommends RAN2 to consider limiting support of mobility for Rel-13 low complexity UEs to reduce SIB size at least in enhanced coverage [2].

Sony’s opinion [3] is that MTC Release-13 should support two broad classes of device:

- Smart meter type devices: SM-LTE devices

- Consumer electronic type devices: CE-LTE devices

MTC should not be limited to a smart meter or stationary device. But, it should also support non-stationary devices which may require more mobility consideration. In our previous contribution [4], it has been shown that the intra RAT mobility functionalities of radio resource management (e.g. idle mode and connected mode) only introduce negligible UE power consumption.

This contribution describes Sony’s view on reduced mobility support for MTC devices.

# Discussion

## SM-LTE and CE-LTE Devices

Table 1 lists the high level characteristics of CE-LTE and SM-LTE devices. Considering that at least these two types of device will be supported by MTC, MTC should be able to support very different mobility aspects from static / low mobility (SM-LTE) to high mobility (CE-LTE) device.

Table 1 – High Level Characteristics of CE-LTE and SM-LTE devices

| Requirement | CE-LTE device | SM-LTE device |
| --- | --- | --- |
| Cost | Low cost | Very low cost |
| Data rate | 100kbps – 1Mbps range | 10 – 100bps |
| Latency | Approx. 250ms to 5 seconds | 24 hours |
| Frequency of UL Data Transmissions | Every 10 seconds to 30 minutes | 24 hours |
| Coverage | Standard LTE coverage | Coverage extension to basements |
| Form factor | Very small | Standard |
| Battery capacity | Up to 300 mAh = 1Wh | Up to 7000mAh (2 AA cells) = 10Wh |
| Battery lifetime | 1 to 2 weeks | Up to 10 years |
| Mobility | Required | Slow mobility acceptable |
| Roaming | Required | Required |

Table 2 (derived from [3]) identifies some communicating consumer electronics devices that could benefit from the Release 13 LTE MTC specifications, considering the mobility aspects of these devices. It can be identified that some CE-LTE devices can require high mobility. For example, a user with a smart watch riding a car or train would be in a high mobility situation.

Table 2 – Example Consumer Electronics Devices

| Consumer electronics device | Mobility Classifications |
| --- | --- |
| Smart watch | High mobility (user in vehicle) |
| Fitness tracker | Medium mobility (running, cycling) |
| Health monitor | High mobility (monitored patient on transport) |
| Baby monitor | Low mobility |
| Panic alarm | High mobility (user on public transport) |
| Pet tracker | Medium mobility (pet running away from owner) |
| Headphones | High mobility (user in vehicle) |
| Toy car | Low mobility (local use) |
| Robot lawn mower | Low mobility (local use) |

The usage of SM-LTE is well known. Even tough, the device can be semi-permanently placed in a location, it may still require mobility. As an example, an SM-LTE device in a parking meter can occasionally be hidden behind a bus which obstructs the radio wave propagation from its normal serving cell. In such situations, an SM-LTE device may perform cell-reselection to a neighbour cell. Another example, SM-LTE device in the train vending machine also obviously requires mobility support.

## Reduced Mobility Support for MTC devices

The reduced mobility support as written in the WI proposal [1] has not been clearly defined. Based on the latest discussion from NTT Docomo contribution [5], the SIB seems to be the limiting factor of mobility support for MTC devices. It also states one possible approach to make the new SIB be efficient is to focus on very low mobility scenario. If a support for mobility is limited, design for SIB can be simplified from viewpoint of both physical and higher layers. More specifically, if mobility support for the UE in the idle mode, i.e., cell reselection, is limited, SIB3 -SIB8 would be simplified [5].

A legacy LTE UE has a broad range of mobility support. For examples: roaming across countries/operators, hand-over to other cells, hand-over to other radio access technology (e.g. GSM, WCDMA, CDMA), or even the speed of UE physical movement. In this document, we discuss some potential mobility supports for MTC that can be removed, so that it can fulfil WI proposal objectives (i.e. complexity reduction and coverage enhancement).

For complexity and cost reasons, a Release-13 LTE MTC UE may be designed with a single RAT. Single RAT operation will reduces the mobility support as the device is no longer required to perform inter-RAT mobility. Single-RAT operation will have the following impact:

- UE measurements related to other RATs are no longer needed.

- MTC UE is no longer required to decode the broadcast SIB information that related to other RATs (i.e. SIB6-8).

Proposal 1: Reduced complexity MTC device only needs to support intra-RAT mobility.

For the cost and power consumption reduction consideration, a Release-13 LTE MTC UE is expected to support fewer bands (potentially in the region of 2-4 bands) than a full capability Release-12 UE. A reduction in the number of bands reduces the number of inter-frequency measurements required at the UE. Consequently, less number of measurements brings benefit that MTC UE consumes less power than a legacy LTE UE.

Considering the supported bands are limited and there is on-going discussion to reduce the SIB size, one possibility is to reduce the number of instances of inter-frequency neighbour frequencies, cells and blacklisted cells. We can also consider reducing the number of instances of intra-frequency neighbour cells and blacklisted cells.

The current specification specifies the number of instances of inter-frequency neighbour cells is up to 8 cells on up to 8 frequencies, and intra-frequency neighbour cells is up to 16 cells. The number of instances can be reduced in an MTC SIB so that the total transport block size is less than 1000 bits.

Proposal 2: Considering the supported bands are limited and we need to reduce the SIB size, we can consider SIB with reduced size of intra-frequency and inter-frequency neighbour lists.

Our analysis indicates no evidence that we need to specifically constrain the MTC mobility in terms of velocity (e.g. MTC device for low mobility) as pointed out in [5]. This also applies to the UE in enhanced coverage but may depend on the level of coverage enhancement provided/needed. In an extreme case (i.e. 15 dB coverage enhancement) where a large number of SIB repetitions are required, we expect the target device is located in a basement (i.e. a stationary device) and no mobility support is needed for such a device. However, a device such as a smartwatch which may need coverage enhancement in order to compensate for a reduction in performance due to complexity reductions will require mobility support, for example to support coverage enhancement of 3dB.

Proposal 3: The number of SIB repetitions is configurable by the eNB depending on the intended coverage enhancement level.

In addition, it is beneficial to allow the UE to access the cell using different levels of coverage enhancement. Of course, it is better for a UE that is able to access the cell without using repetitions to use normal coverage mode – hence we believe it is also beneficial for a UE to access the cell using e.g. 3dB coverage enhancement (less repetition – potentially more features supported such as inter-frequency mobility) even if it supports up to 20dB coverage enhancement. This can result in a UE supporting a high level of coverage enhancement being able to use mobility when it is only just out of normal coverage, and then disabling mobility in favour of obtaining some level of service when a high level of coverage enhancement is needed. It means that not all SIBs (e.g. SIB5) need to be transmitted using the maximum amount of repetition in order to always provide the maximum level of coverage enhancement and it allows for the different use-cases which we provided above to be supported. The level of coverage enhancement can be selected simply by providing a threshold (Qrxlevmin) for each level of coverage enhancement provided, and the UE selects the level necessary based on the measured downlink Srxlev.

Proposal 4: The eNB may configure multiple levels of coverage enhancement and the UE selects the level of coverage enhancement based on the downlink Srxlev/Squal measurement.

Proposal 4a: Mobility SIBs may be provided at some level of coverage enhancement (e.g. 3dB) and not at other levels of coverage enhancement (e.g. 15dB)

This also allows for different device types to be implemented. For example a smart watch type device may need only to support 3dB coverage enhancement due to less well performing antenna in order to reach the same level of mobility performance as a smartphone, while a smart meter in a basement doesn’t need to support mobility, but would need to support the maximum level of coverage enhancement.

Proposal 5: A UE may support one of a number of maximum coverage enhancement levels.

Finally, as we have observed, the limiting factor in support of mobility is the ability to receive SIBs in coverage enhancement. In many cases the UE will have received mobility information in normal coverage, so we should not exclude the support of mobility if this is the case but rather define what happens in the case that UE is unable to receive mobility related SIBs. The various mobility scenarios are summarised below in figure 1.

Figure 1: Mobility scenarios when enhanced coverage is used.

A: In this case the UE moves from a cell using only normal coverage mode, into the enhance coverage region of another cell. In this case, the reselection rules and parameters should ensure that the UE prioritises normal coverage over enhanced coverage, so will remain camped on the first cell.

B: Now the UE moves into normal coverage mode of both of the cells. The reselection thresholds will cause the UE to reselect to the second cell once the second cell meets the reselection criteria.

C: The UE moves from normal coverage to enhanced coverage mode of this cell.

-	UE has already stored the system information received in normal coverage mode.

Proposal 6: In enhanced coverage the UE shall continue to perform measurements and reselection using stored system information received in normal coverage.

Proposal 6a: Discuss whether UE shall still continue to use stored mobility information even after system information change while the SIBs are not transmitted in enhanced coverage mode.

D: The UE moves into a zone which is covered using enhanced coverage by the current cell and another cell. The UE reselects to the new cell, once reselection criteria is met.

Proposal 7: Discuss whether UE may use neighbour information provided from previous cell in case not available in coverage enhanced mode of the new cell.

E: The UE moves from enhanced coverage to normal coverage of that cell.

Proposal 8: Upon moving from enhanced coverage to normal coverage, the UE shall trigger reading of system information, and information received in SIBs shall replace any stored information.

F: UE moves from a cell supporting coverage enhancement to a cell not supporting coverage enhancement. The UE uses normal reselection procedure

G: The UE moves from being completely out of coverage into a coverage-enhanced region of a cell, or selects the cell using cell selection procedure (E.g. at power on).

H: The UE moves from the coverage enhanced region of a cell into another cell. If the UE was able to obtain the reselection information in the previous step, then UE may reselect to the new cell. Otherwise the UE eventually loses coverage of the coverage enhanced cell, and performs cell selection.

Proposal 9: Only in case the UE does not have stored system information, is reselection not supported – in this case cell selection is used if coverage is lost.

# Conclusion

In this contribution, we have elaborated the definition of reduced mobility for MTC in order to meet MTC rel.13 objectives. MTC should not be limited to a smart meter or stationary device. It should also support non-stationary devices which may require more mobility consideration. This creates more use-cases for MTC devices.

Our proposals are summarized as follows:

Proposal 1: Reduced complexity MTC device only needs to support intra-RAT mobility.

Proposal 2: Considering the supported bands are limited and we need to reduce the SIB size, we can consider SIB with reduced size of intra-frequency and inter-frequency neighbour lists.

Proposal 3: The number of SIB repetitions is configurable by the eNB depending on the intended coverage enhancement level.

Proposal 4: The eNB may configure multiple levels of coverage enhancement and the UE selects the level of coverage enhancement based on the downlink Srxlev/Squal measurement.

Proposal 4a: Mobility SIBs may be provided at some level of coverage enhancement (e.g. 3dB) and not at other levels of coverage enhancement (e.g. 15dB)

Proposal 5: A UE may support one of a number of maximum coverage enhancement levels.

Proposal 6: In enhanced coverage the UE shall continue to perform measurements and reselection using stored system information received in normal coverage.

Proposal 6a: Discuss whether UE shall still continue to use stored mobility information even after system information change while the SIBs are not transmitted in enhanced coverage mode.

Proposal 7: Discuss whether UE may use neighbour information provided from previous cell in case not available in coverage enhanced mode of the new cell.

Proposal 8: Upon moving from enhanced coverage to normal coverage, the UE shall trigger reading of system information, and information received in SIBs shall replace any stored information.

Proposal 9: Only in case the UE does not have stored system information, is reselection not supported – in this case cell selection is used if coverage is lost.

# References

RP-141660, “New WI Proposal: Further LTE Physical Layer Enhancements for MTC”

R2-150006, “LS on simultaneous reception requirements and SIBs for MTC UEs”

R1-144050, “Complexity Reduction Targets and General Considerations for MTC UE Types”

R1-145017, “Mobility support impact on power consumption for MTC devices”

R1-144962, “Features for Rel-13 Low Cost MTC with UE Complexity Reduction”