3GPP TSG-RAN WG2 Meeting #90 	R2-152835

# Fukuoka, Japan, 25 - 29 May 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 0782 | rev | - | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Prohibit timer for SR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12, LTE-L23 |  |  |  |  |  |  | Date: |  | 2015-05-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The purpose of the SR prohibit timer is to delay the triggering of an SR with a time period equal to the length of the SR prohibit timer. However, in the current version of the MAC specification the delay until triggering an SR may be shorter than the time duration of the SR prohibit timer if data for the logical channel becomes available for transmission while this timer is running. What this means is that there will be situations when an SR is triggered by the UE during time periods when the SR prohibit timer happens to be running when new UL data becomes available for the logical channel. This causes unnecessary load on the SR channel and unnecessary battery usage by the UE because the UE needs to send the SR and the UE needs to be in DRX active state in the time period from sending an SR and until receiving a grant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the logicalChannelSR-ProhibitTimer is running when data becomes available for transmission for a logical channel for which the timer applies, the timer is restarted. This will ensure there will always be a minimum delay until the UE triggers an SR for UL data that becomes available for transmission belonging to a logical channel for which the timer applies. Impact Analysis Impacted functionality: The change is limited to the SR triggering functionality. Inter-operability: If the CR is implemented by the UE no interoperability problems are foreseen. The CR does not impact the eNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Using the SR prohibit timer for a logical channel where the eNB uses periodic scheduling will be less efficient because it will cause unnecessary SRs to be sent and result in unnecessary battery consumption in the UE. |  |  |  |  |  |  |  |  |
| NS |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--- Parts omitted ---

### 5.4.5	Buffer Status Reporting

The Buffer Status reporting procedure is used to provide the serving eNB with information about the amount of data  available for transmission in the UL buffers associated with the MAC entity. RRC controls BSR reporting by configuring the three timers  periodicBSR-Timer, retxBSR-Timer and logicalChannelSR-ProhibitTimer and by, for each logical channel, optionally signalling logicalChannelGroup which allocates the logical channel to an LCG [8].

For the Buffer Status reporting procedure, the MAC entity shall consider all radio bearers which are not suspended and may consider radio bearers which are suspended.

A Buffer Status Report (BSR) shall be triggered if any of the following events occur:

-	UL data, for a logical channel which belongs to a LCG, becomes available for transmission in the RLC entity or in the PDCP entity (the definition of what data shall be considered as available for transmission is specified in [3] and [4] respectively) and either the data belongs to a logical channel with higher priority than the priorities of the logical channels which belong to any LCG and for which data is already available for transmission, or there is no data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	UL resources are allocated and number of padding bits is equal to or larger than the size of the Buffer Status Report MAC control element plus its subheader, in which case the BSR is referred below to as "Padding BSR";

-	retxBSR-Timer expires and the MAC entity has data available for transmission for any of the logical channels which belong to a LCG, in which case the BSR is referred below to as "Regular BSR";

-	periodicBSR-Timer expires, in which case the BSR is referred below to as "Periodic BSR".

For Regular BSR:

-	if the BSR is triggered due to data becoming available for transmission for a logical channel for which logicalChannelSR-ProhibitTimer is configured by upper layers:

-	start or restart the logicalChannelSR-ProhibitTimer;

-	else:

-	if running, stop the logicalChannelSR-ProhibitTimer.

For Regular and Periodic BSR:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Long BSR;

-	else report Short BSR.

For Padding BSR:

-	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Truncated BSR of the LCG with the highest priority logical channel with data available for transmission;

-	else report Short BSR.

-	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader, report Long BSR.

If the Buffer Status reporting procedure determines that at least one BSR has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI:

-	instruct the Multiplexing and Assembly procedure to generate the BSR MAC control element(s);

-	start or restart periodicBSR-Timer except when all the generated BSRs are Truncated BSRs;

-	start or restart retxBSR-Timer.

-	else if a Regular BSR has been triggered and logicalChannelSR-ProhibitTimer is not running:

-	if an uplink grant is not configured or the Regular BSR was not triggered due to data becoming available for transmission for a logical channel for which logical channel SR masking (logicalChannelSR-Mask) is setup by upper layers:

-	a Scheduling Request shall be triggered.

A MAC PDU shall contain at most one MAC BSR control element, even when multiple events trigger a BSR by the time a BSR can be transmitted in which case the Regular BSR and the Periodic BSR shall have precedence over the padding BSR.

The MAC entity shall restart retxBSR-Timer upon indication of a grant for transmission of new data on any UL-SCH.

All triggered BSRs shall be cancelled in case the UL grant(s) in this subframe can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader. All triggered BSRs shall be cancelled when a BSR is included in a MAC PDU for transmission.

The MAC entity shall transmit at most one Regular/Periodic BSR in a TTI. If the MAC entity is requested to transmit multiple MAC PDUs in a TTI, it may include a padding BSR in any of the MAC PDUs which do not contain a Regular/Periodic BSR.

All BSRs transmitted in a TTI always reflect the buffer status after all MAC PDUs have been built for this TTI. Each LCG shall report at the most one buffer status value per TTI and this value shall be reported in all BSRs reporting buffer status for this LCG.

NOTE:	A Padding BSR is not allowed to cancel a triggered Regular/Periodic BSR. A Padding BSR is triggered for a specific MAC PDU only and the trigger is cancelled when this MAC PDU has been built.

--- Parts Omitted ---