3GPP TSG-RAN WG2 Meeting #91	R2-153150

Beijing, China, 24th – 28st August 2015

Agenda item:		7.6.2.1

Source:	Samsung

Title:	UE behaviours of different UE and Network conditions in LTE-WLAN integrated Network Environment

Document for:		Discussion and Decision

1	Introduction

Since RAN2#67, RAN-level aggregation of LTE and WLAN is studied as a working item [1], for benefits such as dynamic resource allocation, centralized network control and management, real-time load balancing, and seamless mobility support.

In this contribution, we investigate the practical UE behaviors in an LTE-WLAN integrated network environment, considering the UE/network conditions and user preference on LTE-WLAN interworking / aggregation.

2	Discussion

So far, RAN2 discussed that E-UTRAN only configure the UE for LTE-WLAN interworking / aggregation. Consequently, the UE may provide information to E-UTRAN for such configuration.

In this contribution, we provide possible UE behaviour with different user preference and conditions. The UE behaviour can be configured by E-UTRAN with such UE capability information regarding user preference and conditions.

2.1 Considering LTE-WLAN environment and aggregation methods

Figure 1 depicts the considerable LTE-WLAN interworking / aggregation in a heterogeneous network consists of LTE eNBs, enterprise WLAN APs, and private APs. Note that the solid (and dotted) orange lines denote current (and possible) traffic flows through the path.

Figure 1 An example of LTE-WLAN interworking/integration methods in a heterogeneous network
consists of an eNB, an enterprise AP, and a private AP

Although the above Figure 1 did not depict all the cases such as LTE or WLAN only, we expect that those examples are enough for discussing the following issues on LTE-WLAN interworking and aggregation.

Since the ‘LTE-WLAN interworking / aggregation’ and ‘simultaneous LTE and private WLAN usage’ utilize both LTE and WLAN modules simultaneously, the throughput and power consumption of those two methods are higher than that of the LTE-WLAN interworking method.

2.2 Possible UE behaviour

In a considered network environment, each UE with both LTE and WLAN modem can associate and communicate with any eNB or AP. In this subclause, we observe the possible UE behaviour with respect to the user preference and different conditions of UE or networks.

Considered assumptions on the following UE behaviors are as follows: 
 * UE knows the following conditions and user preferences such as 
	+ Whether the WLAN modem is on or off 
	+ Whether the user allows the UE to utilize LTE-WLAN interworking / aggregation or not
	+ Whether there is a list of user preferred APs
	+ Whether the user allows the network (or the LTE modem) to turn on the WLAN modem of the UE or not
 * A user preferred AP is always considered to have higher priority to associate, than any not-preferred APs.
 * User intervention or indication is not described in detail.

Following table categorizes the UE behaviour with respect to different user preference, and UE and network conditions.
(LWA: LTE-WLAN interworking / aggregation)

| UE and network conditions |  |  |  | UE Behavior |
| --- | --- | --- | --- | --- |
| 1. WLAN modem in the UE is ON | 1.1. There is no user preferred AP (The list of user preferred APs is empty) | 1.1.1. User does not allow the UE to utilize LWA |  | Do nothing ※ Do not perform WLAN scanning also, since the preferred AP list is empty. |
|  |  | 1.1.2. User allows the UE to utilize LWA | 1.1.2.1. When there is no nearby LWA AP (LWA AP: AP which can utilize LWA) | Perform WLAN scanning until an AP which can utilize LWA appears. |
|  |  |  | 1.1.2.2. When there are nearby LWA APs | Associate with the nearest/ or eNB designated AP for LWA ※ Do not perform WLAN scanning since the preferred AP list is empty. |
|  | 1.2. There is user preferred AP (The list of user preferred APs is not empty) | 1.2.1. User does not allow the UE to utilize LWA |  | Perform WLAN scanning until a preferred AP appears. |
|  |  | 1.2.2. User allows the UE to utilize LWA | 1.2.2.1. When there is no nearby preferred AP or AP which can utilize LTE-WLAN interworking / aggregation | Perform WLAN scanning until a preferred AP or an AP which can utilize LWA appears. |
|  |  |  | 1.2.2.2. When there are APs which can utilize LWA | Associate with the nearest/ or eNB designated AP for LTE-WLAN interworking / aggregation and simultaneously, perform WLAN scanning until a preferred AP appears, since the preferred AP list is not empty. |
|  |  |  | 1.2.2.3. When there are preferred APs nearby, without any AP which can utilize LWA | Associate with the nearest/ or the most preferred AP |
|  |  |  | 1.2.2.4. When there are both preferred APs and APs which can utilize LWA | Associate with the nearest/ or the most preferred AP ※ Because the priority of user preferred AP is high |
|  |  |  |  | Associate with the nearest / or eNB designated AP for LWA ※ When the user allows it |
|  |  |  |  | Opportunistically associate and communicate with the nearest / or eNB designated AP for LWA, when there is no traffic between the UE and the associated preferred AP ※ May utilize the LWA using IDC schemes |
| 2.WLAN modem in the UE is OFF |  | 2.1. User does not allow the UE to utilize LWA, or the user does not allow UE/ network to turn-on the WLAN modem |  | Do nothing ※ UE/network cannot turn-on the WLAN modem in the UE. |
|  |  | 2.2. User allows the UE to utilize LWA and the user allows UE/ network to turn-on the WLAN modem |  | Turn-on the WLAN modem and utilize LWA by associating with a nearby LWA AP, following: 1. Instruction of the associated eNB via wireless signal. 2. UE’s setting of when to turn on the WLAN modem. ※ After the WLAN modem is turned-on, UE behavior is equal to 2.1.2. |

With the UE behaviour scenarios, it is possible for a UE to utilize LTE-WLAN aggregated network consists of LTE eNBs, APs which support LTE-WLAN interworking / aggregation, and private APs. As ‘Observation 1’, utilizing LTE-WLAN interworking / aggregation may increase the performance, but will consume more power compared with utilizing a single module (LTE or WLAN) only. Moreover, users will experience different costs according to their charging policy.

Therefore, the user should have the authority to control both LTE and WLAN modem in its UE, especially using LTE-WLAN interworking / aggregation. Hence, the user preference of allowing the UE to use LTE-WLAN interworking / aggregation or not, shall be indicated in the UE and the E-UTRAN should be informed of the indication, also.

Proposal 1-1: The UE can send ‘WLAN Status message’ to indicate user preference on AP association to the network whenever it is changed.

The UE can configure its preference of APs based on user input. Then, the UE can indicate this information (such as WLAN modem status, surrounding APs, RSSIs, and list of preferred APs) to E-UTRAN apriori and the E-UTRAN can accordingly decide whether to configure the LTE-WLAN interworking / aggregation to the UE or not. This indication message might carry information such as WLAN modem status, surrounding APs and their Received Signal Strengths, and preferred AP list, etc.

Proposal 1-2: The UE can send ‘WLAN Status message’ based on the request from E-UTRAN before E-UTRAN commands the UE to configure the AP for aggregation.

The UE can send this ‘WLAN Status message’ based on the request from E-UTRAN before E-UTRAN commands the UE to configure the AP for aggregation. For example, if a UE has indicated that it is connected with a preferred AP, then the E-UTRAN can configure the LTE-WLAN interworking/ aggregation without worrying about the WLAN Status at the UE. If the UE dis-associate with the preferred AP and wants to associate with the LTE-WLAN integrated/aggregated AP, then a new ‘WLAN Status message’ might be sent to the network for LTE-WLAN interworking / aggregation.

Proposal 1-3: The UE can send ‘WLAN Status message’ when the user accepts or rejects the aggregation, for example, based on user interface.

Alternatively, if the UE does not configures and indicates its preference of APs while E-UTRAN sends the command to aggregate with the indicated LTE-WLAN aggregated AP, then the UE may prompts the user to accept or reject the aggregation for example based on user interface.

Moreover, the above UE behaviours bring out new issues such as:

1. Whether to allow an LTE-WLAN aggregated eNB or a UE to turn-on the WLAN modem to associate with an LTE-WLAN aggregated AP?

2. Whether to allow the UE to opportunistically utilize the LTE-WLAN aggregation, when there is no traffic between the associated preferred AP.

With the above issues, we have the following proposals:

Proposal 2: UE or network may turn-on the WLAN modem to associate with an LTE-WLAN aggregated AP by following:

- An LTE-WLAN aggregated eNB indicated,

- The network conditions met the UE setting,

when the user allows the UE to utilize LTE-WLAN interworking / aggregation and the user also allows UE/ network to turn-on the WLAN modem

In order to turn-on the turned-off WLAN modem, the user should not prohibit the LTE-WLAN interworking / aggregation to turn-on the user’s UE’s WLAN modem. Moreover, merits of the LTE-WLAN interworking / aggregation must be clear since the WLAN modem is turned-off by the user. If the merit is clear and the user approves, it would be better to turn-on the WLAN modem to utilize the LTE-WLAN interworking / aggregation.

There might be more than one condition to trigger and turn-on the WLAN modem in a UE. An eNB might indicate to turn-on the WLAN modem, or the UE turns-on the WLAN modem when the pre-set condition is met, such as the RSSI of the LTE-WLAN aggregated AP is bigger than a threshold, battery remnant is bigger than a threshold, and so on. All the decisions to turn-on the WLAN modem may utilize network information acquired by UE itself or through the LTE-WLAN aggregated network.

About the user preference, how to reflect a user preference in a UE is an implementation issue. In this contribution, we use the ‘preferred AP list’ as the result of user preference reflection in a UE.

Proposal 3: UE may utilize the LTE-WLAN interworking / aggregation, when there is no traffic between the associated private AP

Although the UE is connected with a preferred AP, there might be an LTE-WLAN aggregated AP nearby. When there is such an LTE-WLAN aggregated AP, it would be possible that the UE may utilize the LTE-WLAN aggregated AP when the UE’s connection with the private AP is not having any traffic to send or receive.

3	Conclusion

Proposal 1: The UE can send ‘WLAN Status message’ in the following situations.

- To indicate user preference on AP association to the network whenever it is changed.

- Based on the request from eNB before the eNB commands the UE to configure the AP for aggregation.

- When the user accepts or rejects the aggregation for example based on user interface.

Proposal 2: UE or network may turn-on the WLAN modem to associate with an LTE-WLAN aggregated AP by following:

- An LTE-WLAN aggregated eNB indicated,

- The network conditions met the UE setting,

when the user allows the UE to utilize LTE-WLAN interworking / aggregation and the user also allows UE/ network to turn-on the WLAN modem.

Proposal 3: UE may utilize the LTE-WLAN interworking / aggregation, when there is no traffic between the associated private AP.

4	References

- RP-150481	“New WI Proposal: LTE-WLAN Radio Level Integration and Interworking Enhancement, ” Intel Corporation, China Telecom, Qualcomm Incorporated.