3GPP TSG-RAN WG2 Meeting #91	R2-153163

Beijing, China, 24 August – 28 august, 2015	revision of R2-152362

Agenda Item	: 	7.1.3 (LTE_LAA-Core)

Source	: 	LG Electronics Inc.

Title	:	Consideration on DL MAC for unlicensed band

Document for	:	Discussion and Decision

1.	Introduction

In Study Item on Licensed-Assisted Access using LTE (LAA) [RP-141817], RAN2 have discussed what needs to be supported for LAA by taking the characteristics/requirement in using the unlicensed band into account. As a result, RAN2 starts Work Item on Licensed-Assisted Access to Unlicensed Spectrum [RP-151045] focusing only on downlink transmission.

As the unlicensed band is to be used unless it is occupied by others while the maximum occupancy time is limited to a certain time duration, using a cell of unlicensed band (U-Cell) would be quite different from using a cell of licensed band (L-Cell) in terms of scheduling. For example, the eNB may schedule on U-Cell only for a short time in an opportunistic manner.

MAC should be configured properly so that the UE can be scheduled with a reasonable delay and battery consumption. In this sense, the MAC configuration appropriate for U-Cell might not be the same as that for L-Cell. However, currently most of the MAC configurations do not consider cell-specific characteristics because there have not been much differences between the cells so far.

This contribution presents a possible consideration on MAC configuration by taking different characteristics of U-Cell and L-Cell into account.

2.	Discussion

In RRC, MAC-MainConfig contains MAC configuration parameters for PHR, BSR, SR, uplink timing maintenance, LCP, DRX, Activation/Deactivation, and HARQ. Among them, we would look into DRX and Activation/Deactivation because those functions are mainly related to the downlink scheduling of a cell.

2.1 DRX

For DRX, onDurationTimer, drx-InactivityTimer, drx-RetransmissionTimer, mac-ContentionResolutionTimer, longDRX-CycleStartOffset, shortDRX-Cycle, and drxShortCycleTimer are signalled.

In RAN2 #89 meeting, RAN2 agreed to apply common DRX unless a very short DRX cycle or a very long Active Times are required. In RAN2 #89bis meeting, a possibility of getting benefits by introducing separate DRX was discussed again. The concern in using common DRX was that with common DRX, the UE might not be able to fully utilize the U-Cell or the UE might consume more battery to fully utilize the U-Cell. However, it was not clear whether/how separate DRX can provide significant gains, hence, RAN2 decided to stick to the last agreement, i.e., to use common DRX.

Common DRX means that the same Active Time applies to all cells while not necessarily mandating configuring the same values for all cells. Then, one simple way that hopefully resolves the concern would be to configure two different values for one timer, i.e., one for L-Cell and the other one for U-Cell.

For example, with two separate timer values for drx-InactivityTimer, when the UE is scheduled on L-Cell the UE starts drx-InactivityTimer with the value for L-Cell while when the UE is scheduled on U-Cell the UE starts drx-InactivityTimer with the value for U-Cell. The benefit would come from controlling the Active Time with different values for one timer depending on the cell on which the UE mainly expects to transfer data.

Figure 1. Example of applying two values for drx-InactivityTimer

In the meantime, by supporting asynchronous HARQ for UL in U-Cell, the maximum number of consecutive PDCCH-subframe(s) until a DL retransmission is received would be different between U-Cell and L-Cell. Hence, two different values for drx-RetransmissionTimer might be needed for LAA.

For this purpose, we think drx-InactivityTimer and drx-RetransmissionTimer could be considered to have two different values – one for U-Cell and the other one for L-Cell.

Observation 1: For U-Cell, a longer Active Time might be preferred compared to L-Cell.

Proposal 1: RAN2 is kindly asked to consider configuring two separate values for drx-InactivityTimer and drx-RetransmissionTimer, one is for U-Cell and the other one is for L-Cell.

2.2 Activation/Deactivation

For Activation/Deactivation, sCellDeactivationTimer is signaled.

Given that occupancy of U-Cell would change from time to time and even not predictable, the UE may need to be activated longer on U-Cell than on L-Cell. Accordingly the eNB may want to configure a longer value of sCellDeactivationTimer for U-Cell in order to keep the UE in activated status longer for data transmission on U-Cell.

Observation 2: For U-Cell, a longer sCellDeactivationTimer might be preferred compared to L-Cell.

Proposal 2: RAN2 is kindly asked to consider configuring two values for sCellDeactivationTimer, one is for U-Cell and the other one is for L-Cell.

3.	Conclusion

In this contribution, a possible consideration on MAC configuration for downlink is presented by taking different characteristics of U-Cell and L-Cell into account. Based on the observations, it is proposed:

Observation 1: For U-Cell, a longer Active Time might be preferred compared to L-Cell.

Observation 2: For U-Cell, a longer sCellDeactivationTimer might be preferred compared to L-Cell.

Proposal 1: RAN2 is kindly asked to consider configuring two values for drx-InactivityTimer, drx-RetransmissionTimer, mac-ContentionResolutionTimer, one is for U-Cell and the other one is for L-Cell.

Proposal 2: RAN2 is kindly asked to consider configuring two values for sCellDeactivationTimer, one is for U-Cell and the other one is for L-Cell.

4.	Reference

[1] RP-141817, Study on Licensed-Assisted Access to Unlicensed Spectrum