3GPP TSG-RAN WG2 Meeting #91                           R2-153209

24.8. to 28.8.2015, Beijing, China

Source: 	CMCC

Title: 			   Discussion on LAA DRS design

Agenda Item:	7.1.2

Document for:	Discussion

- Introduction

In LAA study item phase, it was considered to use Rel-12 discovery reference signals (DRS) for RRM as a baseline. However, the Rel-12 DRS mechanism may need some modifications to support LAA RRM functionalities.

In this contribution, we discuss the potential solutions of DRS design to support LAA RRM functionalities.

- Constraints of Rel-12 DRS mechanism

In Rel-12, DRS is designed to support cell discovery and RRM with small cell on/off operation. DRS comprise PSS/SSS, CRS and optionally CSI-RS. The transmissions of DRS occur in DRS occasions that may have a periodicity of 40, 80 or 160ms. The UE is configured with a discovery measurement timing configuration (DMTC) which is a time window that UE can expect DRS to be received.

There is one configured DMTC in one period. The duration of a DMTC is 6ms. A DRS occasion is configurable from 1ms (2ms for TDD) to 5ms and it may occur anywhere in the DMTC. Figure 1 shows the Rel-12 DRS transmission mechanism.

Figure 1: Rel-12 DRS transmission

In LAA study item phase, it was considered to use Rel-12 DRS as a starting point for providing RRM functionalities. The Rel-12 DRS only allows periodically DRS transmission (40ms/80ms/160ms), which means only one DRS transmission opportunity (i.e. DRS occasions) in each period.

Due to some regulatory requirements, transmission on unlicensed spectrum has some restrictions, e.g. mandatory Listen-before-talk (LBT) mechanism, restricted maximum transmission time [1], which will lead to unpredictable discontinuous transmission. Due to the discontinuous transmission, eNB may miss the configured DRS transmission opportunities.

As illustrated in Figure 2, if the unlicensed carrier is busy during the configured DRS occasion, DRS cannot be transmitted successfully, which means the periodical DRS transmission cannot be guaranteed anymore on unlicensed carrier.

Figure 2: Rel-12 DRS transmission on unlicensed carrier

According to TS 36.133, the specified cell identification time and measurement time period of Rel-12 DRS are associated with both “DMTC period length” and “number of DMTC period”. The intra-frequency cell identification and measurement period requirements are provided as follows:

- TMeasurement_Period_intra_FDD_CRS=5* TDMTC_periodicity

- TIdentify_Intra_SCE= 12*TDMTC_periodicity + TMeasurement_Period_intra_FDD_CRS

TMeasurement_Period_intra_FDD_CRS indicates the required time to measure a cell, which needs 5 DMTC periods. TIdentify_Intra_SCE indicates the required time to detect a new cell (includes 1 measurement period), which needs 17 DMTC periods.

Since DRS transmission on unlicensed carrier is opportunistic, it may happen that consecutive configured DRS occasions are missed and cannot transmit DRS. In this case, the existing measurement requirements cannot be fulfilled. The cell identification and measurement delay will be increased depending on the channel condition on unlicensed carrier.

- Solutions for LAA DRS design

In this section, we discuss the potential solutions to support RRM functionalities on unlicensed carrier. The basic idea is to increase the DRS transmission opportunities in order to subject to the discontinuous transmission on unlicensed carrier.

Solution 1: allow shorter DRS period for unlicensed carrier.

The periodicity of Rel-12 DRS is 40ms, 80ms and 160ms. In order to increase the DRS transmission opportunities, one solution is to allow shorter DRS period, e.g. 20ms or shorter.

The potential problem with this solution is the throughput loss, since 6ms DMTC duration is required to receive DRS in each period. For the current DMTC configuration with DMTC duration=6ms and period=40ms, the throughput loss is 15% (6/40). If shorter DRS period is introduced, the throughput loss will be increased, e.g. 20ms period will lead to 30% throughput loss.

Solution 2: allow DRS to be transmitted in at least one of different time positions within a single DMTC duration (6ms).

Rel-12 DRS only allows one DRS transmission opportunity in one DMTC duration. The DMTC duration is 6ms and DRS duration is configurable from 1ms (2ms for TDD) to 5ms. The duration between two successive DRS transmissions is fixed (40ms/80ms/160ms).

As shown in Figure 3, solution 2 allows DRS to be transmitted in multiple time positions within the configured DMTC. More specifically, if the DRS is not able to be transmitted in the first attempt of DRS transmission, eNB will keep trying to transmit DRS in the left subframes of the configured DMTC. The duration between two successive DRS transmissions is not fixed anymore.

Figure 3: Solution 2 of LAA DRS design

In a DMTC duration (6ms), the first and last 0.5ms are left for RF re-tuning. So the available time for measurement is 6ms-2*0.5ms=5ms. A DRS duration is configurable from 1ms (2ms for TDD) to 5ms. If 1ms DRS is configured, up to 5 opportunities (4 for TDD） can be used in one DMTC. But if 5ms DRS is configured, there is only one DRS transmission opportunity in one DMTC, which is same as Rel-12 DRS mechanism. Hence, the available DRS transmission opportunities are related to DRS duration time, only short DRS duration can benefit from solution 2.

Solution 3: allow DRS to be transmitted in at least one of different time positions within one DMTC period (40ms/80ms/160ms).

As we discussed, solution 2 has some restrictions on the DRS duration time. In order to address this issue, solution 3 allows multiple DRS transmission opportunities in one DMTC period (40ms/80ms/160ms).

More specifically, multiple DRS occasions are configured in one DMTC period. In each DRS occasion, if DRS is transmitted successfully, eNB will stop transmitting DRS in the left occasions in this period; if DRS is not able to be transmitted, eNB will continue to transmit DRS in the next occasion.

Figure 4 shows one example of the proposed DRS design. Four DRS occasions are configured in one period. In the 1st DRS occasion, the channel is busy, DRS is not transmitted. Then eNB continue to transmit DRS in the 2nd DRS occasion and succeed. The 3rd and 4th DRS occasion are invalid and will not be used for DRS transmission.

Figure 4: Solution 3 of LAA DRS design

Solution 3 can also be combined with solution 2. Multiple DRS transmission opportunities can be allowed both in each DMTC (6ms) and in each DMTC period (40ms/80ms/160ms) in order to guarantee at least one successful DRS transmission in one period as much as possible.

- Conclusion

In this contribution, we discuss the constraints of Rel-12 DRS mechanism and potential solutions of DRS design to support LAA RRM functionalities. In order to subject to the discontinuous transmission on unlicensed carrier, three possible solutions are provided:

- Solution 1: allow shorter DRS period for unlicensed carrier.

- Solution 2: allow DRS to be transmitted in at least one of different time positions within a single DMTC duration (6ms).

- Solution 3: allow DRS to be transmitted in at least one of different time positions within one DMTC period (40ms/80ms/160ms).

Based on the above analysis, we have the following proposal:

Proposal 1: RAN2 is requested to select a proper solution for the DRS design to support the LAA RRM functionalities.

- Reference

- TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum (Release 13).