3GPP TSG RAN WG2 #91                               R2-153216

24.8. to 28.8.2015, Beijing, China

Agenda Item:	  	7.1.2

Source:	  		CMCC

Title:		  		RRM measurements and reporting

Document for:  	Discussion and decision

- Introduction

In the RAN2#89bis meeting [1], the following was agreed:

Agreements: RSRP/RSRQ: 1 RRM measurements (e.g. RSRP/RSRQ) on LAA cell can be used to configure, activate, de-configure LAA cell (as for SCells on licensed carriers)

In the RAN2#90 meeting [2], the following was agreed further:

RSRP/RSRQ: 7 RAN2 assumes that the UE’s physical layer will only report valid RSRP/RSRQ measurement samples to RRC (i.e., L1 should not provide samples when DRS transmission was blocked by LBT).

In terms of RAN2’s work, this contribution would discuss the issues related with RRM measurements and reporting.

- Discussion

- Measurement configuration

The Rel-12 Discovery Reference Signals (DRS) can be used as a starting point for providing RRM functionality including cell identification. The transmissions of the DRS occur in DRS occasions that may have a periodicity of 40, 80 or 160 ms. The signals comprising the DRS include the PSS, SSS, CRS and optionally the CSI-RS. The UE is configured with a discovery measurement timing configuration (DMTC) which is a time window within which the UE can expect the DRS to be received. While the DRS occasion may occur anywhere in the DMTC, the UE may expect the DRS to be transmitted from a given cell so that the duration between successive DRS transmissions is fixed (40, 80 or 160 ms) [3].

The use of the Rel-12 DRS on a carrier in unlicensed spectrum has some new constraints.  DRS design for LAA should allow DRS transmissions on an LAA SCell to be subject to LBT (Listen-before-talk). If LBT is applied to DRS transmissions, there will be some instances where the DRS is not able to be transmitted in a periodic manner as in the case of the Rel-12 DRS transmitted on a cell in licensed spectrum. The following two alternatives (i.e. Alt 1 and Alt 2) may then be considered for DRS design for LAA.

- Subject to LBT, DRS is transmitted in fixed time position within the configured DMTC

- Subject to LBT, DRS is allowed to be transmitted in at least one of different time positions within the configured DMTC

Figure 1  LAA DRS design alternatives

For Alt 1, there is only one chance for the eNB to send DRS in one DMTC period. If the DRS is blocked due to the LBT, the eNB has to wait for another 40/80/160ms. The uncertainty of the channel availability increases the time on measurements. Therefore if Alt 1 is applied, modifications on DRS configuration are required, for example, by reducing the DMTC period or extending the DMTC window.

For Alt 2, if the DRS on the first time position is blocked by the LBT, it can be sent on the second time position. However the efficiency of Alt 2 depends on the configuration of DRS duration. If a 4ms DRS duration is configured, there is only one DRS transmission opportunity in a DMTC window. In a word only short DRS duration can benefit from Alt 2. With large DRS duration (e.g., 4ms or above) Alt 2 would have the same problem with Alt 1.

Based on the above analysis, the existing DRS configuration (i.e., 6ms DMTC window and 40/80/160ms DMTC period) are no more applicable for LAA. To reduce the impact of LBT or increase the DRS transmission opportunities, modifications on DRS configuration are required, for example, by reducing the DMTC period or increasing in frequency of DMTC duration.

- The existing DRS configuration (i.e., 6ms DMTC window and 40/80/160ms DMTC period) are not appropriate for LAA.

As currently defined, DMTC causes service interruptions on all serving cells of the UE in both uplink and downlink. Accordingly modified DRS configuration (i.e., DMTC window extension or frequent DMTC duration) may result in longer service interruption which will deteriorate the user experience. Given that in many cases a separate RF chain from the licensed band is used to support unlicensed bands at the UE, it makes sense to define a band-specific DMTC. If separate DMTC can be configured for LAA RF branch, the interruption on licensed carrier can be avoided when modified DRS are configured for LAA band.

- If separate DMTC can be configured for LAA RF branch, the service interruption on licensed carrier can be eased even if the modified DRS (i.e., DMTC window extension or frequent DMTC duration) is configured for LAA band.

- Separate DMTC should be supported at least for LAA band to ease the service interruption.

- Measurement reporting

- Periodic measurement reporting

When periodic measurement reporting is configured, the UE is required to report measurement results at fixed time interval. The measurement results shall include at least ones of PCell and may optionally include those of SCell and neighbor cells.

Figure 2  Measurement model

Since it was agreed in RAN2#90 that “the UE’s physical layer will only report valid RSRP/RSRQ measurement samples to RRC (i.e., L1 should not provide samples when DRS transmission was blocked by LBT)”, it is natural to consider the Layer 3 filtering and the evaluation of reporting criterion using the same logic. If the UE cannot acquire valid RSRP/RSRQ measurements at D, the UE may just not include anything in the measurement results for the involved LAA cell. Therefore, current periodical reporting could also work for LAA RRM measurements.

- Current periodical reporting could also work for LAA RRM measurements by allowing not including anything in the measurement results for the unreachable LAA cell.

- Event triggered measurement reporting

The measurement reporting events supported in the existing RRM measurement framework are listed as follows:

Event A1 (Serving becomes better than threshold);

Event A2 (Serving becomes worse than threshold);

Event A3 (Neighbour becomes offset better than PCell/PSCell);

Event A4 (Neighbour becomes better than threshold);

Event A5 (PCell/PSCell becomes worse than threshold1 and neighbour becomes better than threshold2);

Event A6 (Neighbour becomes offset better than SCell);

Event B1 (Inter RAT neighbour becomes better than threshold);

Event B2 (PCell becomes worse than threshold1 and inter RAT neighbour becomes better than threshold2);

Event C1 (CSI-RS resource becomes better than threshold);

Event C2 (CSI-RS resource becomes offset better than reference CSI-RS resource).

Since it was agreed in RAN2#89bis that “RRM measurements (e.g. RSRP/RSRQ) on LAA cell can be used to configure, activate, de-configure LAA cell (as for SCells on licensed carriers)”, and a LAA Cell cannot be configured as PCell, the measurement events A3/A5/B1/B2 shall not be configured for LAA Cells [4]. Therefore the measurement reporting events A1/A2/A4/A6/C1/C2 could be useful to configure, activate, de-configure LAA SCells. More specifically, the event A1 can be used to inform the eNB that the current LAA SCell is efficient in terms of channel quality, the event A2 can be used to de-configure a serving LAA SCell, and the events A4/A6/C1/C2 could be used to configure LAA SCells.

- Event A1 can be used to inform the eNB that the current LAA SCell is efficient in terms of channel quality, event A2 can be used to de-configure a serving LAA SCell, and events A4/A6/C1/C2 could be used to configure LAA SCells.

To sum up, we have the following proposal:

- The existing measurement reporting events are sufficient to support LAA SCell configuration/de-configuration.

- Conclusions

Based on the discussion, our proposals are provided as follows:

- The existing DRS configuration (i.e., 6ms DMTC window and 40/80/160ms DMTC period) are not appropriate for LAA.

- If separate DMTC can be configured for LAA RF branch, the service interruption on licensed carrier can be eased even if the modified DRS (i.e., DMTC window extension or frequent DMTC duration) is configured for LAA band.

- Separate DMTC should be supported at least for LAA band to ease the service interruption.

- Current periodical reporting could also work for LAA RRM measurements by allowing not including anything in the measurement results for the unreachable LAA cell.

- Event A1 can be used to inform the eNB that the current LAA SCell is efficient in terms of channel quality, event A2 can be used to de-configure a serving LAA SCell, and events A4/A6/C1/C2 could be used to configure LAA SCells.

- The existing measurement reporting events are sufficient to support LAA SCell configuration/de-configuration.

- Reference

- RAN2-89bis Bratislava - Chairman Notes. April 2015.

- RAN2-90 Fukuoka - Chairman Notes, May 2015.

- TR 36.889-051.

- R2-152218, Considerations of Measurement Issues in LAA, Huawei, HiSilicon