3GPP TSG-RAN WG2 Meeting #91	R2-153760

# Beijing, P.R. China, 24th – 28th August 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | 0143 | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDCP data transfer procedure for uplink bearer split |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_dualC_enh-Core |  |  |  |  |  |  | Date: |  | 2015-08-14 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | This CR introduces changes to support uplink bearer split in PDCP. A new threshold is introduced. When the PDCP buffer exceeds the threshold, then data is reported and submitted to both MCG and SCG MAC entities. When the buffer is under threshold, then data is reported and submitted to only the configured MAC entity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes needed to support uplink bearer split |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the change is not approved there is no support for uplink bearer split |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.5, 5.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

### 4.2.2	PDCP entities

The PDCP entities are located in the PDCP sublayer. Several PDCP entities may be defined for a UE. Each PDCP entity carrying user plane data may be configured to use header compression.

Each PDCP entity is carrying the data of one radio bearer. In this version of the specification, only the robust header compression protocol (ROHC), is supported. Every PDCP entity uses at most one ROHC compressor instance and at most one ROHC decompressor instance.

A PDCP entity is associated either to the control plane or the user plane depending on which radio bearer it is carrying data for.

Figure 4.2.2.1 represents the functional view of the PDCP entity for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in [2].

For RNs, integrity protection and verification are also performed for the u-plane.

For split bearers, routing is performed in the transmitting PDCP entity, and reordering is performed in the receiving PDCP entity. When submitting PDCP PDUs to lower layers, the transmitting PDCP entity shall:

-	if uplinkSplitThreshold is configured:

-	if data available for transmission is larger than uplinkSplitThreshold:

-	submit the PDCP PDUs to the associated AM RLC entities configured for MCG and SCG when the lower layer entities requests data;

-	else if ul-DataSplitDRB-ViaSCG is set to TRUE by upper layers [3]:

-	submit the PDCP PDUs to the associated AM RLC entity configured for SCG when the lower layer entities associated with the SCG request data;

-	else:

-	submit the PDCP PDUs to the associated AM RLC entity configured for MCG when the lower layer entities associated with the SCG request data.

-	else if ul-DataSplitDRB-ViaSCG is set to TRUE by upper layers [3]:

-	submit the PDCP PDUs to the associated AM RLC entity configured for SCG;

-	else:

-	submit the PDCP PDUs to the associated AM RLC entity configured for MCG.

Next change

## 4.5	Data available for transmission

For the purpose of MAC buffer status reporting, the UE shall consider PDCP Control PDUs, as well as the following as data available for transmission in the PDCP layer:

For SDUs for which no PDU has been submitted to lower layers:

- the SDU itself, if the SDU has not yet been processed by PDCP, or

- the PDU if the SDU has been processed by PDCP.

In addition, for radio bearers that are mapped on RLC AM, if the PDCP entity has previously performed the re-establishment procedure, the UE shall also consider the following as data available for transmission in the PDCP layer:

For SDUs for which a corresponding PDU has only been submitted to lower layers prior to the PDCP re-establishment, starting from the first SDU for which the delivery of the corresponding PDUs has not been confirmed by the lower layer, except the SDUs which are indicated as successfully delivered by the PDCP status report, if received:

- the SDU, if it has not yet been processed by PDCP, or

- the PDU once it has been processed by PDCP.

For split bearers, when indicating the data available for transmission to the MAC entity for BSR triggering and Buffer Size calculation, the UE shall:

-	if uplinkSplitThreshold is configured and data available for transmission is larger than uplinkSplitThreshold:

-	indicate the data available for transmission to both the MAC entity configured for SCG and the MAC entity configured for MCG;

-	else if ul-DataSplitDRB-ViaSCG is set to TRUE by upper layer [3]:

-	indicate the data available for transmission to the MAC entity configured for SCG only;

-	else:

-	indicate the data available for transmission to the MAC entity configured for MCG only.

Next change

### 5.1.1	UL Data Transfer Procedures

At reception of a PDCP SDU from upper layers, the UE shall:

-	start the discardTimer associated with this PDCP SDU (if configured);

For a PDCP SDU received from upper layers, the UE shall:

-	associate the PDCP SN corresponding to Next_PDCP_TX_SN to this PDCP SDU;

NOTE:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

NOTE:	For split bearers, when uplinkSplitThreshold is configured, discarding a PDCP SDU already associated with a PDCP SN causes an SN gap in the transmitted PDCP PDUs, which increases PDCP reordering delay in the receiving PDCP entity. How to prevent an SN gap in the transmitted PDCP PDUs after PDCP SDU discard is left up to UE implementation.

-	perform header compression of the PDCP SDU (if configured) as specified in the subclause 5.5.4;

-	perform integrity protection (if applicable), and ciphering (if applicable) using COUNT based on TX_HFN and the PDCP SN associated with this PDCP SDU as specified in the subclause 5.7 and 5.6, respectively;

-	increment Next_PDCP_TX_SN by one;

-	if Next_PDCP_TX_SN > Maximum_PDCP_SN:

-	set Next_PDCP_TX_SN to 0;

-	increment TX_HFN by one;

-	submit the resulting PDCP Data PDU to lower layer.

End of changes