3GPP TSG-RAN WG2 Meeting #91bis	R2-154437

# Malmo, Sweden, Oct. 5 – Oct. 9, 2015                                            Revision of R2-153450

Agenda item:	7.7

Source:	LG Electronics Inc.

Title: 	RS-SINR measurement condition triggered by RSRQ measurement for load balancing and a TP

Document for:	Discussion and Decision

Introduction

In the RAN2 meeting #89bis, RAN2 discussed requirements targeted in the WI [1] titled “Multicarrier Load Distribution” and agreed followings:

7) Maximize user throughput and network capacity (in terms of system throughput, connection establishment,

RA, (inter-frequency) mobility related signalling) for UEs in CONNECTED.

In order to maximize UE throughput, RAN2 considers RS-SINR measurement for load balancing in RRC_CONNECTED. In the RAN2 meeting #89bis, many companies addressed limitation of RS-SINR measurements and RAN2 sent a LS [2] to check RS-SINR measurement feasibility. Based on the response from RAN4 [3], it appears feasible from a power consumption and complexity perspective, and the combination of the RS-SINR and RSRQ provides more information of downlink quality than RSRQ alone, which could be beneficial for predicting the achievable user throughput. In this contribution, we further discuss the additional considerations on RS-SINR measurement for load balancing and propose RS-SINR measurement condition triggered by RSRQ measurement.

Discussion

In the several proposals [4]-[7], many companies addressed that the current RSRQ measurement does not provide good representation of achievable throughput in the higher SINR region. As a result, RAN2 considers RS-SINR measurement for load balancing. Since different types of RS-SINR measurements (narrow band or wideband) provide different level of measurement accuracy and it is difficult to support inter-frequency RS-SINR measurement with measurement gap for non-serving cells, the RS-SINR measurement should be carefully considered for load balancing mechanism.

Since the current RSRQ measurement provides good representation of user throughput in lower SINR region and has low complexity, it is beneficial to consider the current RSRQ measurement for load balancing in the lower SINR region. When we consider the difficulty to support the inter-frequency measurement, the dynamic range of RSRQ in terms of SINR can be a good factor to determine which type of measurements is useful for load balancing mechanism in a given SINR region.

Since RS-SINR has the complexity in implementation compared to RSRP and the combination of the RS-SINR and RSRQ could be beneficial for predicting the achievable user throughput, we propose a RS-SINR measurement condition triggered by the RSRQ measurement. For load balancing, UE determines whenever it needs to additionally measure the RS-SINR measurement as well as RSRP/RSRQ. With a threshold to specify a boundary of the dynamic range of RSRQ in terms of SINR, the UE needs to additionally measure the RS-SINR in higher SINR region. The threshold may be set by the curve specifying the characteristics between SINR and RSRQ as shown in the Figure 1.

Figure 1. RSRQ-SINR Curve

UE is required to measure RS-SINR if the following condition is satisfied:

N_RSRQ > Th_RSRQ

where N_RSRQ and Th_RSRQ are the RSRQ measurement value of neighbor cell and the threshold specifying the dynamic range of RSRQ, respectively. If RSRQ measurement of a neighbor cell is better than threshold specifying dynamic range of RSRQ, UE performs RS-SINR measurements of the neighbor cell. If RSRQ measurement of a neighbor cell is less than the threshold, the UE does not perform the RS-SINR measurement of the cell or UE is allowed to not perform the RS-SINR measurement of the cell.

Proposal: RS-SINR measurement of a cell (PCell) should be triggered when RSRQ measurement of the cell is beyond the threshold when applying the load balancing mechanism to maximize user throughput.

Text Proposal

Beginning of Text Proposal

#### 5.2.4.2	Measurement rules for cell re-selection

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

- 	UE capable of RS-SINR measurement shall perform RS-SINR measurement only if a cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Qqualmeas > ThreshX, SINR

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

  - UE capable of RS-SINR measurement shall perform RS-SINR measurement only if a cell of a lower or equal priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Qqualmeas > ThreshX, SINR

#### 5.2.4.3	Mobility states of a UE

Besides Normal-mobility state a High-mobility and a Medium-mobility state are applicable if the parameters (TCRmax, NCR_H, NCR_M and TCRmaxHyst) are sent in the system information broadcast of the serving cell.

State detection criteria:

Medium-mobility state criteria:

- If number of cell reselections during time period TCRmax exceeds NCR_M and not exceeds NCR_H

High-mobility state criteria:

- If number of cell reselections during time period TCRmax exceeds NCR_H

The UE shall not count consecutive reselections between same two cells into mobility state detection criteria if same cell is reselected just after one other reselection.

State transitions:

The UE shall:

-	if the criteria for High-mobility state is detected:

-	enter High-mobility state.

-	else if the criteria for Medium-mobility state is detected:

-	enter Medium-mobility state.

-	else if criteria for either Medium- or High-mobility state is not detected during time period TCRmaxHyst:

-	enter Normal-mobility state.

If the UE is in High- or Medium-mobility state, the UE shall apply the speed dependent scaling rules as defined in subclause 5.2.4.3.1.

##### 5.2.4.3.1	Scaling rules

UE shall apply the following scaling rules:

-	If neither Medium- nor Highmobility state is detected:

-	no scaling is applied.

-	If High-mobility state is detected:

-	Add the sf-High of "Speed dependent ScalingFactor for Qhyst” to Qhyst if sent on system information

-	For E-UTRAN cells multiply TreselectionEUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionEUTRA” if sent on system information

-	For UTRAN cells multiply TreselectionUTRA by the sf-High of "Speed dependent ScalingFactor for TreselectionUTRA” if sent on system information

-	For GERAN cells multiply TreselectionGERA by the sf-High of "Speed dependent ScalingFactor for TreselectionGERA state” if sent on system information

-	For CDMA2000 HRPD cells Multiply TreselectionCDMA_HRPD by the sf-High of "Speed dependent ScalingFactor for TreselectionCDMA_HRPD” if sent on system information

-	For CDMA2000 1xRTT cells Multiply TreselectionCDMA_1xRTT by the sf-High of "Speed dependent ScalingFactor for TreselectionCDMA_1xRTT” if sent on system information

-	If Medium-mobility state is detected:

-	Add the sf-Medium of "Speed dependent ScalingFactor for Qhyst for medium mobility state" to Qhyst if sent on system information

-	For E-UTRAN cells multiply TreselectionEUTRA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionEUTRA" if sent on system information

-	For UTRAN cells multiply TreselectionUTRA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionUTRA" if sent on system information

-	For GERAN cells multiply TreselectionGERA by the sf-Medium of "Speed dependent ScalingFactor for TreselectionGERA" if sent on system information

-	For CDMA2000 HRPD cells Multiply TreselectionCDMA_HRPD by the sf-Medium of "Speed dependent ScalingFactor for TreselectionCDMA_HRPD" if sent on system information

-	For CDMA2000 1xRTT cells Multiply TreselectionCDMA_1xRTT by the sf-Medium of "Speed dependent ScalingFactor for TreselectionCDMA_1xRTT" if sent on system information

In case scaling is applied to any TreselectionRAT parameter the UE shall round up the result after all scalings to the nearest second.

#### 5.2.4.4		Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in subclause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in subclause 5.2.4.5, the UE shall check if the access is restricted according to the rules in subclause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in subclause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell as a candidate for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the [8]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

#### 5.2.4.5		E-UTRAN Inter-frequency and inter-RAT Cell Reselection criteria

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT; or

-	A cell of a higher priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a higher priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

- More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority E-UTRAN frequency shall be based on ranking for Intra-frequency cell reselection as defined in sub-clause 5.2.4.6.

If threshServingLowQ is provided in SystemInformationBlockType3 and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority EUTRAN or UTRAN FDD RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT; or

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority UTRAN TDD, GERAN or CDMA2000 RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority E-UTRAN frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP  during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a higher priority RAT/ frequency shall take precedence over a lower priority RAT/ frequency, if multiple cells of different priorities fulfil the cell reselection criteria.

The UE shall not perform cell reselection to UTRAN FDD cells for which the cell selection criterion S is not fulfilled.

For cdma2000 RATs, Srxlev is equal to -FLOOR(-2 x 10 x log10 Ec/Io) in units of 0.5 dB, as defined in [18], with Ec/Io referring to the value measured from the evaluated cell.

For cdma2000 RATs, ThreshX, HighP and ThreshX, LowP are equal to -1 times the values signalled for the corresponding parameters in the system information.

In all the above criteria the value of TreselectionRAT is scaled when the UE is in the medium or high mobility state as defined in subclause 5.2.4.3.1. If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an E-UTRAN frequency, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria according to section 5.2.4.6;

-	If the highest-priority frequency is from another RAT, a cell ranked as the best cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

Cell reselection to another RAT, for which Squal based cell reselection parameters are broadcast in system information, shall be performed based on the Squal criteria if the UE supports Squal (RSRQ) based cell reselection to E-UTRAN from all the other RATs provided by system information which UE supports. Otherwise, cell reselection to another RAT shall be performed based on Srxlev criteria.

#### 5.2.4.6		Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s + QHyst - Qoffsettemp

Rn = Qmeas,n - Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in [3] |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CSG cells that are known by the UE not to be CSG member cells.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to subclause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

#### 5.2.4.7		Cell reselection parameters in system information broadcasts

Cell reselection parameters are broadcast in system information and are read from the serving cell as follows:

cellReselectionPriority

This specifies the absolute priority for E-UTRAN frequeny or UTRAN frequency or group of GERAN frequencies or band class of CDMA2000 HRPD or band class of CDMA2000 1xRTT.

Qoffsets,n

This specifies the offset between the two cells.

Qoffsetfrequency

Frequency specific offset for equal priority E-UTRAN frequencies.

Qoffsettemp

This specifies the additional offset to be used for cell selection and re-selection. It is temporarily used in case the RRC Connection Establishment fails on the cell as specified in [3].

Qhyst

This specifies the hysteresis value for ranking criteria.

Qqualmin

This specifies the minimum required quality level in the cell in dB.

Qrxlevmin

This specifies the minimum required Rx level in the cell in dBm.

TreselectionRAT

This specifies the cell reselection timer value. For each target E-UTRA frequency and for each RAT (other than E-UTRA) a specific value for the cell reselection timer is defined, which is applicable when evaluating reselection within E-UTRAN or towards other RAT (i.e. TreselectionRAT for E-UTRAN is TreselectionEUTRA, for UTRAN TreselectionUTRA for GERAN TreselectionGERA, for TreselectionCDMA_HRPD, and for TreselectionCDMA_1xRTT).

Note: 	TreselectionRAT is not sent on system information, but used in reselection rules by the UE for each RAT.

TreselectionEUTRA

This specifies the cell reselection timer value TreselectionRAT for E-UTRAN. The parameter can be set per E-UTRAN frequency [3].

TreselectionUTRA

This specifies the cell reselection timer value TreselectionRAT for UTRAN.

TreselectionGERA

This specifies the cell reselection timer value TreselectionRAT for GERAN.

TreselectionCDMA_HRPD

This specifies the cell reselection timer value TreselectionRAT for CDMA HRPD.

TreselectionCDMA_1xRTT

This specifies the cell reselection timer value TreselectionRAT for CDMA 1xRTT.

ThreshX, HighP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshX, HighQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a higher priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN FDD might have a specific threshold.

ThreshX, LowP

This specifies the Srxlev threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN, each group of GERAN frequencies, each band class of CDMA2000 HRPD and CDMA2000 1xRTT might have a specific threshold.

ThreshX, LowQ

This specifies the Squal threshold (in dB) used by the UE when reselecting towards a lower priority RAT/ frequency than the current serving frequency. Each frequency of E-UTRAN and UTRAN FDD might have a specific threshold.

ThreshServing, LowP

This specifies the Srxlev threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshServing, LowQ

This specifies the Squal threshold (in dB) used by the UE on the serving cell when reselecting towards a lower priority RAT/ frequency.

ThreshX, SINR

This specifies RSRQ threshold specifying the dynamic range of RSRQ when performing RS-SINR measurements of the neighbor cell.

SIntraSearchP

This specifies the Srxlev threshold (in dB) for intra-frequency measurements.

SIntraSearchQ

This specifies the Squal threshold (in dB) for intra-frequency measurements.

SnonIntraSearchP

This specifies the Srxlev threshold (in dB) for E-UTRAN inter-frequency and inter-RAT measurements.

SnonIntraSearchQ

This specifies the Squal threshold (in dB) for E-UTRAN inter-frequency and inter-RAT measurements.

End of Text Proposal

Conclusion

Proposal: RS-SINR measurement of a cell (PCell) should be triggered when RSRQ measurement of the cell is beyond the threshold when applying the load balancing mechanism to maximize user throughput.

References

[1] RP-150491, “New WI proposal for Multicarrier Load Distribution of UEs in LTE,” Verizon, China Telecom, et al.

[2] R2-151785, “LS on a new measurement quantity for Multicarrier Load Distribution,” RAN2

[3] R2-154022, “Reply LS on a new measurement quantity for Multicarrier Load Distribution,” RAN WG4

[4] R2-151156, “Measurement quantities for load balancing,” Nokia Networks

[5] R2-151057, “Limitations of the current measurement qualities for load balancing in multiple carrier deployments,” NTT DOCOMO, CMCC, et al.

[6] R2-151213, “Discussion on introducing RS-SINR measurement,” CMCC

[7] R2-151089, “Consideration on RS-SINR measurement,” Qualcomm Inc. NTT DOCOMO