3GPP TSG-RAN WG2 Meeting #91bis	R2-154410

# Malmo, Sweden, October 5 – October 9, 2015

# Agenda Item: 7.2.1

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of enhanced CA in PDCP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. (PDCP rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_enh_b5C-Core |  |  |  |  |  |  | Date: |  | 2015-10-05 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Intention of this CR is to capture agreements made for enhanced CA in Rel-13. The main agreements potentially impacting PDCP specification are listed below. In RAN2#91 => Take 23bits PDCP SN as a baseline. => The PDCP SN of 23 bit applies only for AM bearers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 23bits PDCP SN is introduced for AM DRBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDCP specification does not support enhanced CA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.6, 6.2.11(new), 6.3.2, 6.3.9, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

6.2.6	PDCP Control PDU for PDCP status report

Figure 6.2.6.1 shows the format of the PDCP Control PDU carrying one PDCP status report when a 12 bit SN length is used, Figure 6.2.6.2 shows the format of the PDCP Control PDU carrying one PDCP status report when a 15 bit SN length is used, and Figure 6.2.6.3 shows the format of the PDCP Control PDU carrying one PDCP status report when a 23 bit SN length is used. This format is applicable for DRBs mapped on RLC AM.

Figure 6.2.6.1: PDCP Control PDU format for PDCP status report using a 12 bit SN

Figure 6.2.6.2: PDCP Control PDU format for PDCP status report using a 15 bit SN

Figure 6.2.6.3: PDCP Control PDU format for PDCP status report using a 23 bit SN

6.2.11	User plane PDCP Data PDU with further extended PDCP SN (23 bits)

Figure 6.2.11.1 shows the format of the PDCP Data PDU when a 23 bit SN length is used. This format is applicable for PDCP Data PDUs carrying data from DRBs mapped on RLC AM.

Figure 6.2.11.1: PDCP Data PDU format for DRBs using a 23 bit SN

6.3.2	PDCP SN

Length: 5, 7, 12, 15, 16, or 23 bits as indicated in table 6.3.2.1.

Table 6.3.2.1: PDCP SN length

| Length | Description |
| --- | --- |
| 5 | SRBs |
| 7 | DRBs, if configured by upper layers (pdcp-SN-Size [3]) |
| 12 | DRBs, if configured by upper layers (pdcp-SN-Size [3]) |
| 15 | DRBs, if configured by upper layers (pdcp-SN-Size [3]) |
| 16 | SLRBs |
| 23 | DRBs, if configured by upper layers (pdcp-SN-Size [3]) |

6.3.9	FMS

Length: 12 bits when a 12 bit SN length is used, 15 bits when a 15 bit SN length is used, and 23 bits when a 23 bit SN length is used

PDCP SN of the first missing PDCP SDU.

7.3	Constants

a) Reordering_Window

Indicates the size of the reordering window. The size equals to 2048 when a 12 bit SN length is used, 16384 when a 15 bit SN length is used, or 4194304 when 23 bit SN length is used, i.e. half of the PDCP SN space, for radio bearers that are mapped on RLC AM.

b) Maximum_PDCP_SN is:

-	8388607 if the PDCP entity is configured for the use of 23 bits SNs

-	65535 if the PDCP entity is configured for the use of 16 bits SNs

-	32767 if the PDCP entity is configured for the use of 15 bits SNs

-	4095 if the PDCP entity is configured for the use of 12 bit SNs

-	127 if the PDCP entity is configured for the use of 7 bit SNs

-	31 if the PDCP entity is configured for the use of 5 bit SNs