3GPP TSG-RAN WG2 Meeting #91bis	R2-154434

# Malmo, Sweden, Oct. 5 – Oct. 9, 2015			                     Revision of R2-153340

Agenda item:	7.7

Source:	LG Electronics Inc.

Title: 	Measurement rules supporting Cell Specific Priority

Document for:	Discussion and Decision

Introduction

In the last RAN2 meeting [1], RAN2 agreed followings regarding cell level load distribution:

Agreements from RAN2 #91 [1]

- We increase the value range of the existing field

- Adopt cell specific priorities as baseline

In this contribution, we propose measurement rules supporting cell level load distribution mechanism (e.g. cell specific priority (CSP)) to be captured in the TS 36.304 [2].

Discussion

Cell specific priorities (CSP) is currently considered as a potential solution enabling load distribution among individual cells rather than only a carrier level. If a UE is configured with CSP for a list of cells, the UE needs to detect such cells and then to evaluate whether reselection towards the cell needs to be done. The detection of the CSP cells can be done by measuring the frequency of cells.

According to the current measurement rules specified in TS 36.304 [2], whether UE performs measurements of a frequency is controlled by relative priority difference between serving frequency and target frequency and signal strength/quality of the serving cell. It is difficult to detect and measure the CSP cells by the current measurement rules such as adjusting the relative frequency priorities or reselection measurement thresholds.

To enable UE to detect and measure the cells for which CSP is applicable, UE propose to apply CSP for the measurements of the frequencies for which CSPs are configured. With the priority replacement, UE can follow legacy measurement rules.

Proposal: If UE is configured with a cell list with cell specific priority for a frequency and the cell specific priority of any cell in the list is higher than the original reselection priority for that frequency, UE shall follow legacy measurements except that UE shall consider that frequency to have the cell specific priority for measurement purpose.

Conclusions

Proposal: If UE is configured with a cell list with cell specific priority for a frequency and the cell specific priority of any cell in the list is higher than the original reselection priority for that frequency, UE shall follow legacy measurements except that UE shall consider that frequency to have the cell specific priority for measurement purpose.

References

[1] R2-15xxxx_draft_report_RAN2_91_Beijing_v0.1

[2] TS 36.304 v12.5.0, “User Equipment (UE) procedures in idle mode (Release 12)”