3GPP TSG-RAN WG2 meeting #92				R2-156359

Anahaim, USA, 16th – 20th Nov 2015

Source: 	ZTE

Title: 	TP of evaluation results for TTI redution

Agenda item:	7.10.2

Document for: Discussion and Decision

Introduction

This submission for the evaluations to be included in TR 36.881 is based on the RAN2#92 ZTE Tdoc R2-15xxxx.

The evaluation results on the gains provided by the shortened TTI of 1/2 OFDM symbol and 0.5ms are presented in section 2, and are proposed to be added to the evaluation section in TR 36.881, including related simulation assumption descriptions.

Evaluation of potential gains from TTI reduction

2.1	Simulation assumptions

The details of the TBS (Transport Block Size) calculation assumptions for short TTI duration are given in Annex A1.x. The detailed simulation assumptions including the header bits for each layer are given in Annex A1.x. The performance of 1ms TTI duration is the baseline. The L1 loss rate for the 1ms TTI duration is assumed as 8.3%, which is calculated as given in Annex A1.x. Only small file size (etc. 0.5Mbyte) is evaluated. The same RLC parameters are applied to different TTI durations.

2.2	Simulation results on UPT gain

Figure x: UPT

As the figure given above, we can see that the shorter TTI duration shows a better performance on UPT.

Considering the DL data transmission, a shorter TTI duration could have relatively smaller TBS could bring more segments by RLC. However the number of downlink OFDM symbols used for data transmission is similar between different TTI duration, while the L1 overhead loss is the same. Then the time used for the downlink data transmission is similar for different TTI duration. The extra latency caused by more segments is the HARQ reordering delay. Given that the BLER is less than 10%, the extra HARQ reordering delay for segments is not too much. Considering the UL data transmission, as the TCP ACK/NACK packet is relatively small, the RLC segments are hardly caused by TCP ACK packet even through the TBS of short TTI duration is small. The transmission latency for the UL TCP ACK/NACK can be largely reduced, according the UL transmission latency calculation model given in Annex A1.1. Thus the TCP RTT which deceases with the reduction of TTI duration brings a better performance on the slow start process of TCP.

Observation 1: The UPT increases with the reduction of TTI duration.

2.2	Simulation results on overhead gain

Header

The header overhead rate is the total header bits (from the TCP layer to the MAC layer) divided by the total TBS(s) used for data transmission. The header overheads in the RLC/HARQ retransmission are not considered for the calculation.

Figure x: Header overhead rate

As the figure given above, the header overheads generally increase with the reduction of the TTI duration, as the shorter TTI duration leads to smaller TBS which in turn results in more segments in the RLC layer, and brings more header bits in RLC and MAC.

Observation 2: The header overheads increase with the reduction of TTI duration.

Padding

The padding rate is the total padding bits divided by the total TBS(s) used for data transmission. Only the downlink padding rate is calculated below.

Figure x: Padding rate

As the figure given above, the padding rate decreases while the TTI duration decreases. If the TCP segment is large (This is most likely as a larger TCP segment normally increases the user throughput, especially for small FTP files), it is difficult for the network to schedule the TCP segment to fit within a transport block. However with short TTI duration, the eNB can have more granularities on TBS which gives the flexibilities to the eNB to fit the TCP segments and RLC segments into different transport block size.

Observation 3: The padding bits decreases with the reduction of TTI duration.

Total overhead

Only the downlink transmission is considered. The total overhead rate is calculated as follows:

The total overhead rate = (the total TBS(s) used for data transmission – the total transmitted bits of FTP file) / the total TBS(s) used for data transmission.

Figure x: Overhead Rate

As the figure given above, the RLC may trigger the RLC retransmission even through the corresponding RLC PDU is still being transmitted via the corresponding HARQ process. As the RLC PDU is smaller in shorter TTI, the overhead for the RLC retransmission is less in shorter TTI if the RLC retransmission configurations are not scaled with the TTI duration.

Observation 4: The RLC retransmission configurations should be scaled with the TTI duration to reduce the number of RLC retransmissions. Otherwise the larger TTI duration causes more retransmission overheads of the duplicated RLC PDU(s).

Summary

In this submission, an evaluation of potential gains from the shortened TTI is presented. In this evaluation, the following observations are made:

Observation 1: The UPT increases with the reduction of TTI duration.

Observation 2: The header overheads increase with the reduction of TTI duration.

Observation 3: The padding bits decreases with the reduction of TTI duration.

Observation 4: The RLC retransmission configurations should be scaled with the TTI duration to reduce the number of RLC retransmissions. Otherwise the larger TTI duration causes more retransmission overheads of the duplicated RLC PDU(s).

It is proposed to:

Capture the evaluation results including descriptions as outlined in section 2 and A1.x in TR 36.881 as described in e-mail discussion ‘[91#29][LTE/Latency] Evaluation results’.

Annex <A>:
Simulation assumptions

A1.x	Simulation x

The UL transmission latency and the HARQ RTT are based on the assumptions given in A1.1.

Simulation assumptions

| Parameter | Assumption |
| --- | --- |
| Simulation time | 20s(Ftp model 2) |
| TTI duration | 1, 2, 7 OFDM symbol; 1ms (legacy baseline) |
| System bandwidth | 10 MHz |
| Duplex mode | FDD |
| Carrier frequency | 2GHz |
| Cell layout | Hexagonal grid, 7 sites, 3 cells per site |
| UE distribution | Random distribution; average number of UE per cell: 2 |
| Inter-site distance | 500m |
| HARQ RTT | 8TTI |
| L1 overhead | For 1, 2, 7 OFDM symbol: 5%, 10%, 20%, 30%, 50% |
| UE mobility | 0 Km/h |
| Reserved OFDM symbol | 0, 1 for legacy PDCCH |
| One-way CN Delay | 1ms |
| Target BLER | 10% |
| Downlink scheduler | Proportional fair in time and frequency |
| Antenna configuration | 2 tx , 2 rx (CELL) ; 1 tx, 1 rx (UE) |
| Maximum Tx Power of eNodeB | 46 dBm |
| UE power class | 23dBm (200mW) |
| Channel model | Large Fading channel model |
| TCP parameters | Initial TCP window: 1460 MSS: 1460 Ssthresh: 65535 |
| FTP traffic model | FTP Traffic Model 2 [x]: - File size: 0.5 Mbytes - Reading Time: Exponential Distribution, Mean = 1s |
| RLC parameters | t-PollRetransmit: 500 ms pollPDU: 128 pollByte: infinity maxRetxThreshold: 4 t-Reordering: 80 ms t-StatusProhibit: 5 ms |
| Overheads of headers | TCP header: 20 bytes IP header: 20 bytes PDCP header: 2 bytes TCP+IP header overheads after RoHC: 8 bytes RLC header: 2 bytes MAC header: 2 bytes |

TBS calculation assumption

The TBS (Transport Block Size) of each short TTI PDSCH is calculated as follows:

TBSshort=TBSlegacy×NshortNlegacy×1-Lshort1-Llegacy

TBSshort: The TBS of short TTI PDSCH

TBSlegacy: The TBS [x] of legacy PDSCH which uses the same MCS and PRB(s) as the short TTI PDSCH.

Nshort: The number of REs in short TTI PDSCH per PRB

Nlegacy: The number of REs in legacy PDSCH per PRB

Lshort: The loss rate of PHY layer within the short TTI PDSCH due to the REs not used for data transmission

Llegacy: The loss rate of PHY layer within the legacy PDSCH due to the REs not used for data transmission

Resource mapping of short TTI

In Figure x, the resource map above is the legacy resource mapping per PRB in one subframe, considering 2 Antenna ports and 2 OFDM symbols control field. In Figure x, the resource map below is the short TTI resource mapping, considering 2 OFDM symbols used for the control field in order to ensure the backward compatibility. The loss rates (Llegacy, e.g. 5% - 50%) of the PHY layer in short TTI duration are assumed.

Figure x: Resource mapping per PRB in each subframe

TBS Calculation of short TTI

According to the resource mapping and the TBS calculation formula given above, the loss rate of PHY layer for legacy PDSCH is calculated as follows:

Llegacy =the number of reference symbols within PDSCH the number of REs within PDSCH= 12144= 8.3%

For different short TTI duration, The TBS of short TTI PDSCH is calculated as the following table:

Table x: TBS calculation for different TTI duration

| TTI Duration | TBS of short TTI PDSCH (TBSshort) |
| --- | --- |
| 7 OFDM symbol | First time slot: TBSshort=TBSlegacy×60144×1-Lshort1-8.3% |
|  | Second time slot: TBSshort=TBSlegacy×84144×1-Lshort1-8.3% |
| 2 OFDM symbol | TBSshort=TBSlegacy×24144×1-Lshort1-8.3% |
| 1 OFDM symbol | TBSshort=TBSlegacy×12144×1-Lshort1-8.3% |