3GPP TSG-RAN WG2 meeting #92				R2-157031

Anahaim, USA, 16th – 20th Nov 2015

Source: 	ZTE Corporation, Nokia Networks

Title: 	TP of evaluation results for L2 overhead

Agenda item:	7.10.2

Document for: Discussion and Decision

# Introduction

This submission for the evaluations is to be included in TR 36.881. The text proposal 1 in section 2.1 is based on ZTE’s Tdoc R2-156340 [1]. The text proposal 2 in section 2.2 is based on Nokia Network’s Tdoc R2-156278 [2].

# Text Proposals

## 2.1 Text Proposal 1

### 9.1.x	Simulation x

TTI reduction gain with additional L2 overhead [x]

#### 9.1.x.1	Simulation assumptions

The details of the TBS (Transport Block Size) calculation assumptions for short TTI duration are given in Annex A1.x. The detailed simulation assumptions including the header bits for each layer are given in Annex A1.x. The performance of 1ms TTI duration is the baseline. The L1 loss rate for the 1ms TTI duration is assumed as 8.3%, which is calculated as given in Annex A1.x. Only small file size (etc. 0.5Mbyte) is evaluated.

#### 9.1.x.2	Evaluation results

The header overhead rate is the total header bits (from the TCP layer to the MAC layer) divided by the total TBS(s) used for data transmission. The header overheads in the RLC/HARQ retransmission are not considered for the calculation.

Figure 9.1.x-1: Header overhead rate

As the figure given above, the header overheads generally increase with the reduction of the TTI duration, as the shorter TTI duration leads to smaller TBS which in turn results in more segments in the RLC layer, and brings more header bits in RLC and MAC.

Observation x: The header overheads increase with the reduction of TTI duration.

## Annex A1.x Simulation x [x]

The UL transmission latency and the HARQ RTT are based on the assumptions given in A1.x.

Simulation assumptions

| Parameter | Assumption |
| --- | --- |
| Simulation time | 20s(Ftp model 2) |
| TTI duration | 1, 2, 7 OFDM symbol; 1ms (legacy baseline) |
| System bandwidth | 10 MHz |
| Duplex mode | FDD |
| Carrier frequency | 2GHz |
| Cell layout | Hexagonal grid, 7 sites, 3 cells per site |
| UE distribution | Random distribution; average number of UE per cell: 2 |
| Inter-site distance | 500m |
| HARQ RTT | 8TTI |
| L1 overhead | For 1, 2, 7 OFDM symbol: 5%, 10%, 20%, 30%, 50% |
| UE mobility | 0 Km/h |
| Reserved OFDM symbol | 0, 1 for legacy PDCCH |
| One-way CN Delay | 1ms |
| Target BLER | 10% |
| Downlink scheduler | Proportional fair in time and frequency |
| Antenna configuration | 2 tx , 2 rx (CELL) ; 1 tx, 1 rx (UE) |
| Maximum Tx Power of eNodeB | 46 dBm |
| UE power class | 23dBm (200mW) |
| Channel model | Large Fading channel model |
| TCP parameters | Initial TCP window: 1460 MSS: 1460 Ssthresh: 65535 |

| FTP traffic model | FTP Traffic Model 2 File size: 0.5 Mbytes Reading Time: Exponential Distribution, Mean = 1s |
| --- | --- |
| Overheads of headers | TCP header: 20 bytes IP header: 20 bytes PDCP header: 2 bytes TCP+IP header overheads after RoHC: 8 bytes RLC header: 2 bytes MAC header: 2 bytes |

TBS calculation assumption

The TBS (Transport Block Size) of each short TTI PDSCH is calculated as follows:

TBSshort=TBSlegacy×NshortNlegacy×1-Lshort1-Llegacy

TBSshort: The TBS of short TTI PDSCH

TBSlegacy: The TBS of legacy PDSCH which uses the same MCS and PRB(s) as the short TTI PDSCH.

Nshort: The number of REs in short TTI PDSCH per PRB

Nlegacy: The number of REs in legacy PDSCH per PRB

Lshort: The loss rate of PHY layer within the short TTI PDSCH due to the REs not used for data transmission

Llegacy: The loss rate of PHY layer within the legacy PDSCH due to the REs not used for data transmission

Resource mapping of short TTI

In Figure A1.x.1 the resource map above is the legacy resource mapping per PRB in one subframe, considering 2 Antenna ports and 2 OFDM symbols control field. In Figure A1.5.1 the resource map below is the short TTI resource mapping, considering 2 OFDM symbols used for the control field in order to ensure the backward compatibility. The loss rates (Llegacy, e.g. 5% - 50%) of the PHY layer in short TTI duration are assumed.

Figure A1.x.1: resource mapping per PRB in one subframe

TBS Calculation of short TTI

According to the resource mapping and the TBS calculation formula given above, the loss rate of PHY layer for legacy PDSCH is calculated as follows:

Llegacy =the number of reference symbols within PDSCH the number of REs within PDSCH= 12144= 8.3%

For different short TTI duration, The TBS of short TTI PDSCH is calculated as the following table:

Table A1.x.1: TBS calculation for different TTI duration

| TTI Duration | TBS of short TTI PDSCH (TBSshort) |
| --- | --- |
| 7 OFDM symbol | First time slot: TBSshort=TBSlegacy×60144×1-Lshort1-8.3% |
|  | Second time slot: TBSshort=TBSlegacy×84144×1-Lshort1-8.3% |
| 2 OFDM symbol | TBSshort=TBSlegacy×24144×1-Lshort1-8.3% |
| 1 OFDM symbol | TBSshort=TBSlegacy×12144×1-Lshort1-8.3% |

## 2.2 Text Proposal 2

### 9.1.x	Simulation x

TTI reduction gain with additional L2 overhead [x]

#### 9.1.x.1	Simulation assumptions

In our analysis, TBS of legacy 14 OFDM symbol TTI length from 36.213 [x] is used as reference for calculating TBS for 7 symbols and 2 symbols TTI length, assuming same L1 overhead and same MCS.

Minimum L2 overhead is assumed with 1 byte PDCP header, 1 byte RLC header, 1 byte MAC header and 3 byte CRC, i.e. in total 6 bytes for a TB.

Table 9.1.x-1 to 9.1.x-5 and Figure 9.1.x-1 to 9.1.x-5 illustrate the L2 overhead varies with TBS for TTI length of 14, 7 and 2 OFDM symbols, with 50 PRB and 5 PRB allocated.

#### 9.1.x.2	Evaluation results

Table 9.1.x-1: L1 overhead with 50PRBs

| ITBS | TBS_14sym | TBS_7sym | TBS_2symb | L2 OH_14sym | L2 OH_7sym | L2 OH_2sym |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 1384 | 680 | 177 | 0.034 | 0.068 | 0.239 |
| 1 | 1800 | 888 | 237 | 0.026 | 0.053 | 0.184 |
| 2 | 2216 | 1096 | 296 | 0.021 | 0.043 | 0.150 |
| 3 | 2856 | 1416 | 387 | 0.017 | 0.033 | 0.117 |
| 4 | 3624 | 1800 | 497 | 0.013 | 0.026 | 0.092 |
| 5 | 4392 | 2184 | 607 | 0.011 | 0.022 | 0.076 |
| 6 | 5160 | 2568 | 717 | 0.009 | 0.019 | 0.065 |
| 7 | 6200 | 3088 | 865 | 0.008 | 0.015 | 0.054 |
| 8 | 6968 | 3472 | 975 | 0.007 | 0.014 | 0.048 |
| 9 | 7992 | 3984 | 1121 | 0.006 | 0.012 | 0.042 |
| 10 | 8760 | 4368 | 1231 | 0.005 | 0.011 | 0.038 |
| 11 | 9912 | 4944 | 1395 | 0.005 | 0.010 | 0.034 |
| 12 | 11448 | 5712 | 1615 | 0.004 | 0.008 | 0.029 |
| 13 | 12960 | 6468 | 1831 | 0.004 | 0.007 | 0.026 |
| 14 | 14112 | 7044 | 1995 | 0.003 | 0.007 | 0.024 |
| 15 | 15264 | 7620 | 2160 | 0.003 | 0.006 | 0.022 |
| 16 | 16416 | 8196 | 2325 | 0.003 | 0.006 | 0.020 |
| 17 | 18336 | 9156 | 2599 | 0.003 | 0.005 | 0.018 |
| 18 | 19848 | 9912 | 2815 | 0.002 | 0.005 | 0.017 |
| 19 | 21384 | 10680 | 3034 | 0.002 | 0.004 | 0.016 |
| 20 | 22920 | 11448 | 3254 | 0.002 | 0.004 | 0.015 |
| 21 | 25456 | 12716 | 3616 | 0.002 | 0.004 | 0.013 |
| 22 | 27376 | 13676 | 3890 | 0.002 | 0.004 | 0.012 |
| 23 | 28336 | 14156 | 4027 | 0.002 | 0.003 | 0.012 |
| 24 | 30576 | 15276 | 4347 | 0.002 | 0.003 | 0.011 |
| 25 | 31704 | 15840 | 4509 | 0.002 | 0.003 | 0.011 |
| 26 | 36696 | 18336 | 5222 | 0.001 | 0.003 | 0.009 |

Figure 9.1.x-1: L2 overhead with 50PRB

Table 9.1.x-2: L1 overhead with 5PRBs

| ITBS | TBS_14sym | TBS_7sym | TBS_2symb | L2 OH_14sym | L2 OH_7sym | L2 OH_2sym |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 120 | 48 | 0 | 0.333 | 0.667 | 1.000 |
| 1 | 176 | 76 | 5 | 0.240 | 0.480 | 1.000 |
| 2 | 208 | 92 | 9 | 0.207 | 0.414 | 1.000 |
| 3 | 256 | 116 | 16 | 0.171 | 0.343 | 1.000 |
| 4 | 328 | 152 | 26 | 0.136 | 0.273 | 0.955 |
| 5 | 424 | 200 | 40 | 0.107 | 0.214 | 0.750 |
| 6 | 504 | 240 | 51 | 0.091 | 0.182 | 0.636 |
| 7 | 584 | 280 | 63 | 0.079 | 0.158 | 0.553 |
| 8 | 680 | 328 | 77 | 0.068 | 0.136 | 0.477 |
| 9 | 776 | 376 | 90 | 0.060 | 0.120 | 0.420 |
| 10 | 872 | 424 | 104 | 0.054 | 0.107 | 0.375 |
| 11 | 1000 | 488 | 122 | 0.047 | 0.094 | 0.328 |
| 12 | 1128 | 552 | 141 | 0.042 | 0.083 | 0.292 |
| 13 | 1256 | 616 | 159 | 0.038 | 0.075 | 0.263 |
| 14 | 1416 | 696 | 182 | 0.033 | 0.067 | 0.233 |
| 15 | 1544 | 760 | 200 | 0.031 | 0.061 | 0.214 |
| 16 | 1608 | 792 | 209 | 0.029 | 0.059 | 0.206 |
| 17 | 1800 | 888 | 237 | 0.026 | 0.053 | 0.184 |
| 18 | 1992 | 984 | 264 | 0.024 | 0.048 | 0.167 |
| 19 | 2152 | 1064 | 287 | 0.022 | 0.044 | 0.154 |
| 20 | 2344 | 1160 | 314 | 0.020 | 0.041 | 0.142 |
| 21 | 2472 | 1224 | 333 | 0.019 | 0.038 | 0.135 |
| 22 | 2664 | 1320 | 360 | 0.018 | 0.036 | 0.125 |
| 23 | 2856 | 1416 | 387 | 0.017 | 0.033 | 0.117 |
| 24 | 2984 | 1480 | 406 | 0.016 | 0.032 | 0.112 |
| 25 | 3112 | 1544 | 424 | 0.015 | 0.031 | 0.107 |
| 26 | 3752 | 1864 | 515 | 0.013 | 0.025 | 0.089 |

Figure 9.1.x-2: L2 overhead with 5PRB

Observation x: L2 overhead increases significantly for shorter TTI length with smaller TBS.

Observation x: L2 overhead is very high when the number of PRBs used for shorter TTI is limited and when low MCS is used, for some cases the TBS is not even enough just for the L2 headers and CRC.

# Summary

In this submission, the evaluation of potential gains from the shortened TTI is presented. In this evaluation,

It is proposed to:

Capture the Text Proposals as outlined in section 2 in TR 36.881.

# Reference

[1] R2-156340	Performance evaluation on short TTI	ZTE Corporation	discussion

[2] R2-156278	L2 overhead for shorter TTI	Nokia Networks	discussion