3GPP TSG-RAN WG2 Meeting #93	R2-161440

St. Julian's, Malta, Feb 15th - 19th, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 0835 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on sidelink related description in TS36.300 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eD2D_Prox-Core |  |  |  |  |  |  | Date: |  | 2016-1-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | - In section 3.2, the abbreviations of S-RSRP and SD-RSRP are missing. - In section 5.6.6, the description of SD-RSRP is missing. - In section 8.3, the 8 bit destiantion ID forward to physical layer is named as Group Destination ID, which is not suitable for sidelink unicast communication (e.g., between Remote UE and UE-NW relay). - In section 23.10.2.1, the transmission priority over PC5 and Uu is not suitable during the Sidelink Discvoery Gap which is introduced in Rel-13. - In section 23.10.2.1, ProSe Layer-2 Group ID is used which is not suitable for unicast. - In section 23.10.2.2, the Control-Plane protocol stack for one-to-one sidelink communication is missing. - In section 23.10.4, the minimum and/or a maximum Uu link quality threshold(s) are RSRP, not RSRQ. - In section 23.10.4, the description of “respecting these broadcasted threshold(s)” is unclear in the sentence “If the eNB does not broadcast transmission resource pool for ProSe-UE-to-Network Relay discovery, then UE can initiate request for ProSe-UE-to-Network Relay discovery resources by dedicated signalling, respecting these broadcasted threshold(s)”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In section 3.2, the abbreviations of S-RSRP and SD-RSRP are added. - In section 5.6.6, add the description of SD-RSRP. - In section 8.3, change “Group Destination ID” to “Destiantion Layer-1 ID” . - In section 23.10.2.1, the transmission priority over PC5 and Uu is modified considering the impact of Sidelink Discovery Gap. - In section 23.10.2.1, change the “ProSe Layer-2 Group ID” to “Destination Layer-2 ID”. - In section 23.10.2.2, the Control-Plane protocol stack for one-to-one sidelink communication is added. - In section 23.10.4, the RSRQ is deleted. - In section 23.10.4, it is clarified if the eNB does not broadcast transmission resource pool for ProSe UE-to-Network Relay discovery, then UE, satisfying the Uu link quality (RSRP) if configured, can initiate requrest for ProSe-UE-to-Network Relay discovery resource by dedicated signalling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Some descriptions in TS 36.300 remain incorrect. Impact analysis Impacted functionality: No functionality is impacted. Inter-operability: 1. If the network is implemented according to the CR and the UE is not, there is no inter-operability problem. 2. If the UE is implemented according to the CR and the network is not, there is no inter-operability problem. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.6.6, 8.3, 23.10.1, 23.10.2.1, 23.10.2.2, 23.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

First change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xCSFB	Circuit Switched Fallback to 1xRTT

ABS	Almost Blank Subframe

ACK	Acknowledgement

ACLR	Adjacent Channel Leakage Ratio

AM	Acknowledged Mode

AMBR	Aggregate Maximum Bit Rate

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

ARQ	Automatic Repeat Request

ARP	Allocation and Retention Priority

AS	Access Stratum

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BSR	Buffer Status Report

C/I	Carrier-to-Interference Power Ratio

CAZAC	Constant Amplitude Zero Auto-Correlation

CA	Carrier Aggregation

CBC	Cell Broadcast Center

CC	Component Carrier

CG	Cell Group

CIF	Carrier Indicator Field

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

CP	Cyclic Prefix

CoMP	Coordinated Multi Point

C-plane	Control Plane

C-RNTI	Cell RNTI

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Cell Range Extension

CRS	Cell-specific Reference Signal

CSA	Common Subframe Allocation

CSG	Closed Subscriber Group

CSI	Channel State Information

CSI-IM	CSI interference measurement

CSI-RS	CSI reference signal

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCN	Dedicated Core Network

DeNB	Donor eNB

DFTS	DFT Spread OFDM

DL	Downlink

DMTC	Discovery Signal Measurement Timing Configuration

DRB	Data Radio Bearer

DRS	Discovery Reference Signal

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

DwPTS	Downlink Pilot Time Slot

EAB	Extended Access Barring

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

EMM	EPS Mobility Management

E-CID	Enhanced Cell-ID (positioning method)

eIMTA	Enhanced Interference Management and Traffic Adaptation

eHRPD	enhanced High Rate Packet Data

eNB	E-UTRAN NodeB

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

E-RAB	E-UTRAN Radio Access Bearer

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GSM	Global System for Mobile communication

GBR	Guaranteed Bit Rate

GP	Guard Period

GRE	Generic Routing Encapsulation

G-RNTI	Group RNTI

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identifier

GWCN	GateWay Core Network

HARQ	Hybrid ARQ

(H)eNB	eNB or HeNB

HO	Handover

HPLMN	Home Public Land Mobile Network

HRPD	High Rate Packet Data

HSDPA	High Speed Downlink Packet Access

H-SFN	Hyper System Frame Number

ICIC	Inter-Cell Interference Coordination

IDC	In-Device Coexistence

IP	Internet Protocol

ISM	Industrial, Scientific and Medical

KPAS	Korean Public Alert System

LAA	Licensed-Assisted Access

LB	Load Balancing

LBT	Listen Before Talk

LCG	Logical Channel Group

LCR	Low Chip Rate

LCS	LoCation Service

LIPA	Local IP Access

LHN	Local Home Network

LHN ID	Local Home Network ID

LMU	Location Measurement Unit

LPPa	LTE Positioning Protocol Annex

L-GW	Local Gateway

LTE	Long Term Evolution

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE WLAN Radio Level Integration with IPsec Tunnel

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBR	Maximum Bit Rate

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCCH	Multicast Control Channel

MCE	Multi-cell/multicast Coordination Entity

MCG	Master Cell Group

MCH	Multicast Channel

MCS	Modulation and Coding Scheme

MDT	Minimization of Drive Tests

MeNB	Master eNB

MIB	Master Information Block

MIMO	Multiple Input Multiple Output

MME	Mobility Management Entity

MSA 	MCH Subframe Allocation

MSI	MCH Scheduling Information

MSP	MCH Scheduling Period

MTCH	Multicast Traffic Channel

NACK	Negative Acknowledgement

NAS	Non-Access Stratum

NCC	Next Hop Chaining Counter

NH	Next Hop key

NNSF	NAS Node Selection Function

NR	Neighbour cell Relation

NRT	Neighbour Relation Table

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OPI	Offload Preference Indicator

OTDOA	Observed Time Difference Of Arrival (positioning method)

P-GW	PDN Gateway

P-RNTI	Paging RNTI

PA	Power Amplifier

PAPR	Peak-to-Average Power Ratio

PBCH	Physical Broadcast CHannel

PBR	Prioritised Bit Rate

PCC	Primary Component Carrier

PCCH	Paging Control Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PCH	Paging Channel

PCI	Physical Cell Identifier

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PDCP	Packet Data Convergence Protocol

PDN	Packet Data Network

PDU	Protocol Data Unit

PHICH	Physical Hybrid ARQ Indicator CHannel

PHY	Physical layer

PLMN	Public Land Mobile Network

PPPP	ProSe Per-Packet Priority

PMCH	Physical Multicast CHannel

PRACH	Physical Random Access CHannel

PRB	Physical Resource Block

ProSe	Proximity based Services

PSBCH 	Physical Sidelink Broadcast CHannel

PSC	Packet Scheduling

PSCCH	Physical Sidelink Control CHannel

PSCell	Primary SCell

PSDCH	Physical Sidelink Discovery CHannel

PSM	Power Saving Mode

PSSCH	Physical Sidelink Shared CHannel

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared CHannel

pTAG	Primary Timing Advance Group

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QCI	QoS Class Identifier

QoS	Quality of Service

R-PDCCH	Relay Physical Downlink Control CHannel

RA-RNTI	Random Access RNTI

RAC	Radio Admission Control

RACH	Random Access Channel

RAT	Radio Access Technology

RB	Radio Bearer

RBC	Radio Bearer Control

RCLWI	RAN Controlled LTE-WLAN Interworking

RF	Radio Frequency

RIBS	Radio-interface based synchronization

RIM	RAN Information Management

RLC	Radio Link Control

RN	Relay Node

RNC	Radio Network Controller

RNL	Radio Network Layer

RNTI	Radio Network Temporary Identifier

RMTC	RSSI Measurement Timing Configuration

ROHC	Robust Header Compression

RRC	Radio Resource Control

RRM	Radio Resource Management

RU	Resource Unit

S-GW	Serving Gateway

S-RSRP	Sidelink Reference Signal Received Power

S1-MME	S1 for the control plane

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SD-RSRP	Sidelink Discovery Reference Signal Received Power

SeNB	Secondary eNB

SI	System Information

SIB	System Information Block

SIPTO	Selected IP Traffic Offload

SIPTO@LN	Selected IP Traffic Offload at the Local Network

SI-RNTI	System Information RNTI

S1-U	S1 for the user plane

SAE	System Architecture Evolution

SAP	Service Access Point

SC-FDMA	Single Carrier – Frequency Division Multiple Access

SCH	Synchronization Channel

SC-MCCH	Single Cell Multicast Control Channel

SC-MTCH	Single Cell Multicast Transport Channel

SC-PTM	Single Cell Point To Multiploint

SCTP	Stream Control Transmission Protocol

SDF	Service Data Flow

SDMA	Spatial Division Multiple Access

SDU	Service Data Unit

SeGW	Security Gateway

SFN	System Frame Number

S-GW	Serving GateWay

SBCCH	Sidelink Broadcast Control Channel

SL-BCH	Sidelink Broadcast Channel

SL-DCH	Sidelink Discovery Channel

SL-RNTI	Sidelink RNTI

SL-SCH	Sidelink Shared Channel

STCH	Sidelink Traffic Channel

SPID	Subscriber Profile ID for RAT/Frequency Priority

SR	Scheduling Request

SRB	Signalling Radio Bearer

SU	Scheduling Unit

sTAG	Secondary Timing Advance Group

TA	Tracking Area

TAG	Timing Advance Group

TB	Transport Block

TCP	Transmission Control Protocol

TDD	Time Division Duplex

TDM	Time Division Multiplexing

TEID	Tunnel Endpoint Identifier

TFT	Traffic Flow Template

TM	Transparent Mode

TNL	Transport Network Layer

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

UM	Unacknowledged Mode

UMTS	Universal Mobile Telecommunication System

U-plane	User plane

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

UpPTS	Uplink Pilot Time Slot

VRB	Virtual Resource Block

WLAN	Wireless Local Area Network

WT	WLAN Termination

X2 GW	X2 GateWay

X2-C	X2-Control plane

X2-U	X2-User plane

Xw-C	Xw-Control plane

Xw-U	Xw-User plane

Next change

5.6.6	Physical layer measurements definition

For measurement on the sidelink, two basic UE measurement quantities shall be supported:

-	Sidelink reference signal received power (S-RSRP).

-    Sidelink discovery reference signal received power (SD-RSRP).

Next change

8.3	Sidelink communication related identities

The following identities are used for sidelink communication:

-	Source Layer-2 ID: Identifies the sender of the data in sidelink communication. The Source Layer-2 ID is 24 bits long and is used together with Destination Layer-2 ID and LCID for identification of the RLC UM entity and PDCP entity in the receiver;

-	Destination Layer-2 ID: Identifies the target of the data in sidelink communication. The Destination Layer-2 ID is 24 bits long and is split in the MAC layer into two bit strings:

- One bit string is the LSB part (8 bits) of Destination Layer-2 ID and forwarded to physical layer as Destination Layer-1 ID. This identifies the target of the intended data in sidelink control information and is used for filtering of packets at the physical layer.

- Second bit string is the MSB part (16 bits) of the Destination Layer-2 ID and is carried within the MAC header. This is used for filtering of packets at the MAC layer.

No Access Stratum signalling is required for group formation and to configure Source Layer-2 ID, Destination Layer-2 ID and Destination Layer-1 ID in the UE. These identities are either provided by higher layer or derived from identities provided by higher layer. In case of groupcast and broadcast, the ProSe UE ID [62] provided by higher layer is used directly as the Source Layer-2 ID and the ProSe Layer-2 Group ID [62] provided by higher layer is used directly as the Destination Layer-2 ID in the MAC layer. In case of one-to-one communications, higher layer provides Source Layer-2 ID and Destination Layer-2 ID.

Next change

23.10.1	General

Sidelink communication is a mode of communication whereby UEs can communicate with each other directly over the PC5 interface [62]. This communication mode is supported when the UE is served by E-UTRAN and when the UE is outside of E-UTRA coverage. Only those UEs authorised to be used for public safety operation can perform sidelink communication.

In order to perform synchronisation for out of coverage operation UE(s) may act as a synchronisation source by transmitting SBCCH and a synchronisation signal. SBCCH carries the most essential system information needed to receive other sidelink channels and signals. SBCCH along with a synchronisation signal is transmitted with a fixed periodicity of 40ms. When the UE is in network coverage, the contents of SBCCH are derived from the parameters signalled by the eNB. When the UE is out of coverage, if the UE selects another UE as a synchronisation reference, then the content of SBCCH is derived from the received SBCCH; otherwise UE uses pre-configured parameters. SIB18 provides the resource information for synchronisation signal and SBCCH transmission. There are two pre-configured subframes every 40ms for out of coverage operation. UE receives synchronisation signal and SBCCH in one subframe and transmit synchronisation signal and SBCCH on another subframe if UE becomes synchronisation source based on defined criterion [16].

UE performs sidelink communication on subframes defined over the duration of Sidelink Control period. The Sidelink Control period is the period over which resources allocated in a cell for sidelink control information and sidelink data transmissions occur. Within the Sidelink Control period the UE sends sidelink control information followed by sidelink data. Sidelink control information indicates a Layer 1 ID and characteristics of the transmissions (e.g. MCS, location of the resource(s) over the duration of Sidelink Control period, timing alignment).

The UE performs transmission and reception over Uu and PC5 with the following decreasing priority order in case of no Sidelink Discovery Gap:

-	Uu transmission/reception (highest priority);

-	PC5 sidelink communication transmission/reception;

-	PC5 sidelink discovery announcement/monitoring (lowest priority).

The UE performs transmission and reception over Uu and PC5 with the following decreasing priority order in case of Sidelink Discovery Gap:

-	Uu transmission/reception for RACH;

-	PC5 sidelink communication transmission/reception;

-	PC5 sidelink discovery announcement/monitoring.

-	Uu transmission/reception except for RACH.

Next change

23.10.2.1	User plane

Figure 23.10.2.1-1 shows the protocol stack for the user plane, where PDCP, RLC and MAC sublayers (terminate at the other UE) perform the functions listed for the user plane in subclause 6.

The Access Stratum protocol stack in the PC5 interface consists of PDCP, RLC, MAC and PHY as shown below in Figure 23.10.2.1-1.

# Figure 23.10.2.1-1: User-Plane protocol stack for sidelink communication

User plane details of sidelink communication:

-	There is no HARQ feedback for sidelink communication;

-	RLC UM is used for sidelink communication;

-	A receiving UE needs to maintain at least one RLC UM entity per transmitting peer UE;

-	A receiving RLC UM entity used for sidelink communication does not need to be configured prior to reception of the first RLC UMD PDU;

-	ROHC Unidirectional Mode is used for header compression in PDCP for sidelink communication.

A UE may establish multiple logical channels. LCID included within the MAC subheader uniquely identifies a logical channel within the scope of one Source Layer-2 ID and Destination Layer-2 ID combination. Parameters for logical channel prioritization are not configured. The Access stratum (AS) is provided with the PPPP of protocol data unit transmitted over PC5 interface by higher layer. There is a PPPP associated with each logical channel.

Next change

23.10.2.2	Control plane

A UE does not establish and maintain a logical connection to receiving UEs prior to one-to-many a sidelink communication. Higher layer establish and maintain a logical connection for one-to-one sidelink communication including ProSe UE-to-Network Relay operation.

The Access Stratum protocol stack for SBCCH in the PC5 interface consists of RRC, RLC, MAC and PHY as shown below in Figure 23.10.2.2-1.

# Figure 23.10.2.2-1: Control-Plane protocol stack for SBCCH

The control plane for establishing the logical connection for one-to-one sidelink communication is shown in Figure 23.10.2.2-2.

# Figure 23.10.2.2-2: Control-Plane protocol stack for one-to-one sidelink communication

Next change

23.10.4	Sidelink Communication via ProSe UE-to-Network Relay

ProSe UE-to-Network Relay provides generic L3 forwarding function that can relay any type of IP traffic between the Remote UE and the network. One-to-one sidelink communication is used between the Remote UE and the ProSe UE-to-Network Relay. The Remote UE is authorised by upper layer and can be in-coverage or out-of-coverage of EUTRAN for UE-to-Network Relay discovery, (re)selection and communication. The ProSe UE-to-Network Relay is always in-coverage of EUTRAN. ProSe UE-to-Network Relay and the Remote UE performs sidelink communication and sidelink discovery as described in section 23.10 and 23.11 respectively.

The eNB controls whether the UE can act as a ProSe UE-to-Network Relay:

-	If the eNB broadcast any information associated to ProSe UE-to-Network Relay operation, then ProSe UE-to-Network Relay operation is supported in the cell;

-	The eNB may provide:

-	Transmission resources for ProSe UE-to-Network Relay discovery using broadcast for RRC_IDLE state and dedicated signalling for RRC_CONNECTED state;

-	Monitoring resources for ProSe UE-to-Network Relay discovery using broadcast signalling;

-	The eNB may broadcasts a minimum and/or a maximum Uu link quality (RSRP) threshold(s) that the ProSe UE-to-Network Relay needs to respect to autonomously start or stop the UE-to-Network Relay discovery procedure;

-	If the eNB does not broadcast transmission resource pool for ProSe-UE-to-Network Relay discovery, then UE, satisfying the Uu link quality (RSRP) if configured, can initiate request for ProSe-UE-to-Network Relay discovery resources by dedicated signalling.

-	If the ProSe-UE-to-Network Relay is initiated by broadcast signalling, it can perform ProSe UE-to-Network Relay discovery when in RRC_IDLE. If the ProSe UE-to-Network Relay is initiated by dedicated signalling, it can perform relay discovery as long as it is in RRC_CONNECTED.

Change end