| Issue | Brief description | Proposed solution/Comment | Prio. | Status/ Comment |
| --- | --- | --- | --- | --- |
|  |  | Huawei/HiSilicon |  |  |
| 100 | # In RRCConnectionRelease ::= CHOICE {}, we have # vb50NonCriticalExtensions SEQUENCE { # rrcConnectionRelease-vb50ext # RRCConnectionRelease-vb50ext-IEs, # nonCriticalExtensions SEQUENCE {} OPTIONAL # } OPTIONAL, in which there is eutra-TargetFreqInfoList EUTRA-TargetFreqInfoList-vb50ext OPTIONAL, while for RRCConnectionRelease-CCCH ::= CHOICE {}, there is no EUTRA frequency extension related IE included. | Not sure if it was the conclusion at that time, or it was just carelessly missing. | 1 | Closed |
|  |  | Comments: It looks like this was not a conscious decision, but an oversight. QC: We think it should be fixed from REL-11, as NW cannot use CCCH version to redirect UE to an extended E-UTRA frequency. - Huawei will bring separate CR package to fix from REL-11. |  |  |
| 101 | In ASN.1 for UE-Positioning-AddPos-MeasuredResults-element ::=SEQUENCE { # timestamp UTCTime OPTIONAL,……}, in the coresponding tabular, the type and reference is “ASN.1 UTCTime expressed as a Visible String (SIZE(13)) with format YYMMDDhhmmssZ”. # Not sure if it is clear enough in the ASN.1 with just “UTCTime”. | To explicitly use “Visible String (SIZE(13)”? | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 315. - We will just use UTCTime in the Type field, and then add a clarification in the Semantic description field. |  |  |
| 102 | # UE-Positioning-WLANMeasurementList-element ::= SEQUENCE { # Here the Tab should be Blank. | UE-Positioning-WLANMeasurementList-element ::= SEQUENCE { | 3 |  |
|  |  | Comments: |  |  |
| 103 | UE-Positioning-WLANMeasurementList ::= SEQUENCE (SIZE (1..maxWLANs))OF # One more “Blank” before SEQUENCE | UE-Positioning-WLANMeasurementList ::= SEQUENCE (SIZE (1..maxWLANs))OF | 3 |  |
|  |  | Comments: |  |  |
| 104 | wlanRSSI INTEGER(-127..128) OPTIONAL, wlanRTTvalue INTEGER(0..16777215) OPTIONAL, | Should use “Tab” before OPTIONAL | 3 |  |
|  |  | Comments: |  |  |
| 105 | UE-Positioning-MBSMeasurementList-element ::= SEQUENCE { transmitterId INTEGER (0..32767), codePhase INTEGER (0..2097151), codePhaseRmsError INTEGER (0..63) # } # For the corresponding tabular, it says “Enumerated(range index 0..range index 63)” | Better to be aligned | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 425. - We will update the Tabular to align with the ASN.1 |  |  |
| 106 | UE-Positioning-PositionEstimateInfo-vdxyext ::= SEQUENCE { locationTimeStamp UTCTime OPTIONAL # } # Should use “Tab” before OPTIONAL | Should use “Tab” before OPTIONAL | 3 |  |
|  |  | Comments: |  |  |
| 107 | # In 10.3.6.153 Power Control Algorithm 3, all the IEs, “TPC slot position”,” TPC step size”,” Decimation factor”, are OP. PowerControlAlgorithm3 ::= SEQUENCE { tPCSlotPosition INTEGER (0..4) OPTIONAL, tPC-StepSize TPC-StepSizeFDD OPTIONAL, decimationFactor ENUMERATED {slots3, slots5} OPTIONAL } # Questions were raised that they have to be MP, because you have to provide values for the decimation factor, TPC step size, and slot position; on the other hand, when a Alg 3 RL is reconfigured, if only TPC slot position is needed, TPC step size and decimation factor are not touched, the simple way is to put them as OP, while for the first time, the network has to include some value | Either we leave it as OP or, we update them to be MP. | 2 | Closed No Update needed |
|  |  | Comments: Huawei: It was an issue raised during the mail discussion by Nokia Networks. Nokia: We are fine to leave as OP, as long as captured correctly in Procedure text. |  |  |
| 108 | # In 10.2.27 RADIO BEARER RECONFIGURATION, for the IE below: >>Added or Reconfigured TrCH information list; OP; 1 to <maxTrCH>. >>>Added or Reconfigured DL TrCH information; MP; Added or Reconfigured DL TrCH information 10.3.5.1. # The corresponding ASN.1 is: DL-AddReconfTransChInfoList-r13 ::= SEQUENCE (SIZE (1..maxTrCHpreconf)) OF # DL-AddReconfTransChInformation-r13 # Theorectically, here it should be “maxTrch”. | For UL, since R6, it has been updated to 32: -- The maximum allowed size of UL-AddReconfTransChInfoList sequence is 16 UL-AddReconfTransChInfoList ::= SEQUENCE (SIZE (1..maxTrCHpreconf)) OF UL-AddReconfTransChInformation -- The maximum allowed size of UL-AddReconfTransChInfoList-r6 sequence is 32 UL-AddReconfTransChInfoList-r6 ::= SEQUENCE (SIZE (1..maxTrCH)) OF UL-AddReconfTransChInformation-r6 Since both values are 32, see below, maybe it is better to align? maxTrCH INTEGER ::= 32 -- maxTrCHpreconf should be 16 but has been set to 32 for compatibility maxTrCHpreconf INTEGER ::= 32 | 2 | To be fixed in Correction CR |
|  |  | Comments: Ericsson: Yes we can align the REL-13 ASN.1 to the Tabular by using the constant ‘maxTrch’. - We will update the REL-13 ASN.1 to use maxTrch. |  |  |
| 109 | RRCConnectionSetupComplete-va40ext-IEs ::= SEQUENCE { -- Other IEs loggedMeasAvailable ENUMERATED { true } OPTIONAL, loggedANRResultsAvailable ENUMERATED { true } OPTIONAL} # The “}” should be in another line. | RRCConnectionSetupComplete-va40ext-IEs ::= SEQUENCE { -- Other IEs loggedMeasAvailable ENUMERATED { true } OPTIONAL, loggedANRResultsAvailable ENUMERATED { true } OPTIONAL } | 3 |  |
|  |  | Comments: |  |  |
| 110 | SpecificACDC-BarringInformation ::= SEQUENCE (SIZE (1.. maxNumAcdcCategory)) OF SEQUENCE { acdc-Barring ACDC-Barring OPTIONAL, expirationTimeFactor ExpirationTimeFactor2 OPTIONAL # } ACDC-Barring ::= CHOICE { acdc-Barred ENUMERATED {true, false}, acdc-acb-barringBitmap BIT STRING (SIZE (10)) # } | Nothing wrong, but according to the tabular, the naming in ASN.1 is not consistent with the tabular, the correct way should be like: SpecificACDC-BarringInformation ::= SEQUENCE (SIZE (1.. maxNumAcdcCategory)) OF SEQUENCE { acdc-PerCategory ACDC-PerCategory OPTIONAL, expirationTimeFactor ExpirationTimeFactor2 OPTIONAL # } ACDC-PerCategory ::= CHOICE { acdc-Barred ENUMERATED {true, false}, acdc-acb-barringBitmap BIT STRING (SIZE (10)) } Or, we should update ASN.1 as: SpecificACDC-BarringInformation ::= SEQUENCE { acdc-Barring ACDC-Barring OPTIONAL } ACDC-Barring ::= SEQUENCE (SIZE (1.. maxNumAcdcCategory)) OF SEQUENCE { acdc-PerCategory ACDC-PerCategory OPTIONAL, expirationTimeFactor ExpirationTimeFactor2 OPTIONAL } ACDC-PerCategory ::= CHOICE { acdc-Barred ENUMERATED {true, false}, acdc-acb-barringBitmap BIT STRING (SIZE (10)) } | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 418. - We will align the ASN.1 to the Tabular description. |  |  |
| 111 | UE-RadioAccessCapability-vdxyext ::= SEQUENCE { ue-RadioAccessCapabBandCombList-vdxyext UE-RadioAccessCapabBandCombList-vdxyext OPTIONAL # } InterRATHandoverInfo-vdxyext-IEs ::= SEQUENCE { ue-RadioAccessCapabilityInfo-vdxyext UE-RadioAccessCapability-vdxyext OPTIONAL, ue-RadioAccessCapabilityComp2 UE-RadioAccessCapabilityComp2-vdxyext OPTIONAL # } UE-RadioAccessCapability-vdxyext-IEs ::= SEQUENCE { ue-RadioAccessCapabBandCombList-vdxyext UE-RadioAccessCapabBandCombList-vdxyext OPTIONAL, -- UE physical channel capability physicalChannelCapability PhysicalChannelCapability-vdxyext OPTIONAL, -- UE radio access capability ue-PositioningCapability UE-PositioningCapability-vdxyext OPTIONAL # } | Maybe there is no need to address this, but here “-vdxyext” should better to be removed. | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 414. - We will update to remove the ‘-vdxyext’ tag from the IE names. It is enough that the IE types are tagged with ‘-vdxyext’ |  |  |
| 112 | #### For CELL UPDATE CONFIRM message, the appearing order for two IEs in tabular is different from ASN.1 otherStateConfig OtherStateConfig OPTIONAL, retrievableConfigInfo RetrievableConfigInfo OPTIONAL, PDCP ROHC target mode; OP; PDCP ROHC target mode 10.3.4.2a; REL-5. Retrievable configuration info; OP; Retrievable configuration info 10.3.8.25; REL-13. Other state configuration info; OP; Other state configuration info 10.3.8.27; REL-13. | Could consider to make them in the same order. | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 403. - We will align the order in ASN.1 with that in the Tabular. |  |  |
| 113 | # In ASN.1 the DL-AddReconfTransChInformation is updated to DL-AddReconfTransChInformation-r13(including HARQ-info-r13), which is included in RetrievableConfigData: RetrievableConfigData ::= SEQUENCE { srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rb-InformationSetupList RB-InformationSetupList-r11 OPTIONAL, ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-TransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-TransChInfoList DL-AddReconfTransChInfoList-r11 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL } | We need to discuss, if “Blind HARQ retransmissions for HSDPA” could be part of “retrievable configuration”, if yes, then the ASN.1 should be updated as follows: Update DL-AddReconfTransChInfoList-r11 to DL-AddReconfTransChInfoList-r13: RetrievableConfigData ::= SEQUENCE { srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rb-InformationSetupList RB-InformationSetupList-r11 OPTIONAL, ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-TransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-TransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL } | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 203, Issue 319 and Issue 401. Nokia: We think it makes sense to allow it as part of the pre-configuration. Huawei: No strong opinion, just raise the question. Ericsson: Yes it makes sense. - We will update the ASN.1 to reference the DL-AddReconfTransChInfoList-r13 version. |  |  |
| 114 | # New IE “Blind HARQ retransmissions for HSDPA” is added in “HARQ Info”, which is included in SIB5 and 5bis, SIB16,Handover To UTRAN Command，RRC Connection Setup and SRNS relocation, actually this “Blind HARQ retransmissions for HSDPA”IE is not needed in these messages, and ASN.1 is consistent with this agreement, thus, a CV is needed in the corresponding tabular. | Add an CV in the table of 10.3.5.7a for the IE “Blind HARQ retransmissions for HSDPA”: CV-notmessage Condition; Explanation. notMessage; This IE is not included in HANDOVER TO UTRAN COMMAND, RRC CONNECTION SETUP, SRNS RELOCATION INFO or SYSTEM INFORMATION messages. Otherwise it is optional.. | 1 | To be fixed in Correction CR |
|  |  | Comments: Similar to issue raised in Issue 116, Issue 301, Issue 318 and Issue 420. Huawei: Think it is correct to not signal it in these messages, as per the ASN.1. We should only update the tabular as suggested to explicitly list the messages for which the IE is not needed. - We will update the tabular to add the CV statement as suggested by Huawei so that it is clear the IE is not applicable to the listed RRC messages. |  |  |
| 115 | # In ASN.1: HSDSCH-Info-r13 ::= SEQUENCE { harqInfo HARQ-Info-r13 OPTIONAL, dl-MAC-HeaderType CHOICE { mac-hs AddOrReconfMAC-dFlow, mac-ehs AddOrReconfMAC-ehs-ReordQ-r9 } OPTIONAL } # But in r11 release, the IE mac-ehs is updated to r11 release: HSDSCH-Info-r11 ::= SEQUENCE { harqInfo HARQ-Info-r11 OPTIONAL, dl-MAC-HeaderType CHOICE { mac-hs AddOrReconfMAC-dFlow, mac-ehs AddOrReconfMAC-ehs-ReordQ-r11 } OPTIONAL } | Change AddOrReconfMAC-ehs-ReordQ-r9 to AddOrReconfMAC-ehs-ReordQ-r11 in HSDSCH-Info-r13: HSDSCH-Info-r13 ::= SEQUENCE { harqInfo HARQ-Info-r13 OPTIONAL, dl-MAC-HeaderType CHOICE { mac-hs AddOrReconfMAC-dFlow, mac-ehs AddOrReconfMAC-ehs-ReordQ-r11 } OPTIONAL } | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 419. - We need to update the HSDSCH-Info-r13 so that it uses the HSDSCH-Info-r11 as the baseline, and not HSDSCH-Info-r9. |  |  |
| 116 | # New IE “Blind HARQ retransmissions for HSDPA” in “HARQ Info” is included in 10.3.6.74a Serving HS-DSCH cell information, which can be included in the ACTIVE SET UPDATE and 10.3.6.79a Target cell preconfiguration information. # But in ASN.1 the related IE does not include the new HARQ-Info-r13 | Similar as in 114, add a Or just add an CV in the tabular: Add an CV in the table of 10.3.5.7a for the IE “Blind HARQ retransmissions for HSDPA” with some texts like “This IE is not included in ACTIVE SET UPDATE message. Otherwise it is optional.” | 1 | To be fixed in Correction CR |
|  |  | Comments: Similar to issue raised in Issue 114, Issue 301, Issue 318 and Issue 420. - We will update the tabular to add the CV statement as suggested by Huawei so that it is clear the IE is not applicable to the listed RRC messages. |  |  |
| 117 | # In the table 10.3.8.26: Signalling RB information to setup list; OP. >Signalling RB information to setup; MP. >>RAB information to setup list; OP. >>>RAB information for setup; MP. # But in ASN.1: RetrievableConfigData ::= SEQUENCE { srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rb-InformationSetupList RB-InformationSetupList-r11 OPTIONAL, ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-TransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-TransChInfoList DL-AddReconfTransChInfoList-r11 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, mimoParameters MIMO-Parameters-r9 OPTIONAL, mimo4x4Parameters MIMO4x4-Parameters OPTIONAL, dch-Enhancements-Info-FDD DCH-Enhancements-Info-FDD OPTIONAL, ul-EDCH-Information UL-EDCH-Information-r11 OPTIONAL, dl-HSPDSCH-Information DL-HSPDSCH-Information-r12 OPTIONAL, dl-SecondaryCellInfoFDD DL-SecondaryCellInfoFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD AdditionalDLSecCellInfoListFDD-r11 OPTIONAL, additionalDLSecCellInfoListFDD2 AdditionalDLSecCellInfoListFDD2 OPTIONAL } # They are not consistent. | Change the table to: Signalling RB information to setup list. >Signalling RB information to setup. RAB information to setup list. >RAB information for setup. | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 201 and Issue 405. - We will fix the tabular description as suggested to align with ASN.1. |  |  |
|  |  | Qualcomm |  |  |
| 200 | RetrievableConfigInfo ::= SEQUENCE { … PreconfiguredRetrievableConfig OPTIONAL } PreconfiguredRetrievableConfig ::= SEQUENCE { retrievableConfigList RetrievableConfig-List } # RetrievableConfig-List ::= SEQUENCE (SIZE(1.. maxRetrievConfig)) OF RetrievableConfig | Do we need this? PreconfiguredRetrievableConfig ::= SEQUENCE { retrievableConfigList RetrievableConfig-List } | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 417. - We will align the ASN.1 to the tabular by removing the unnecessary PreconfiguredRetrievableConfig. |  |  |
| 201 | In tablular 10.3.8.26, Signalling RB information to setup list and RAB information to setup list should be at the same level. In ASN.1 they are at same level. | Align the tabular to ASN.1 | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 117 and Issue 405. - We will fix the tabular description as suggested to align with ASN.1. |  |  |
| 202 | # CellUpdateConfirm-r13-IEs should contain RAB-InformationSetup-r11. However, RAB-InformationSetup-r8 is included. # This seems to be the issue even from Rel-11. | Add new IE’s in Rel-11, 12, and 13 to add the missing IE. | 1 | To be fixed in Correction CR |
|  |  | Comments: QC: In REL-11 we did not update the CUC message to point to the RAB-InformationSetup-r11, even though this included the new TimerReordering IE for Multiflow. Ericsson: This seems to be in line with the agreed Multiflow CR in R2-124184. We need to check if the Use Case to include this at CUC is strong enough to update ASN.1, or we just clarify in the Tabular. [CB] After comeback, Nokia confirm that it is sufficient to fix this only from REL-13. - We will update the CellUpdateConfirm-r13-IEs definition to include the RAB-InformationSetup-r11 rather than RAB-InformationSetup-r8. |  |  |
| 203 | # RetrievableConfigData should include DL-AddReconfTransChInfoList-r13. # However, DL-AddReconfTransChInfoList-r11 is included. | Change DL-AddReconfTransChInfoList-r11 to DL-AddReconfTransChInfoList-r13. | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 113, Issue 319 and Issue 401. - We will update the ASN.1 to reference the DL-AddReconfTransChInfoList-r13 version. |  |  |
|  |  | Nokia Networks |  |  |
| 301 | # There is a new type HARQ-Info-r13, but Serving-HSDSCH-CellInformation is not updated | Serving-HSDSCH-CellInformation was not updated from –r12 to -13, even though it contains HARQ-Info, version –r13 of which exists. Once Serving-HSDSCH-CellInformation is upgraded to –r13, most of the RRC messages should be updated as well with Serving-HSDSCH-CellInformation-r13. | 1 | To be fixed in Correction CR |
|  |  | Comments: Similar to issue raised in Issue 114, Issue 116, Issue 318 and Issue 420. - We will update the tabular to add the CV statement as suggested by Huawei so that it is clear the IE is not applicable to the listed RRC messages. |  |  |
| 302 | # HSDSCH-Info-r13 is before HSDSCH-Info-r11 in the ASN.1 code | Move HSDSCH-Info-r13 after HSDSCH-Info-r11 | 2 | To be fixed in Correction CR |
|  |  | Comments: - We will place the definition of HSDSCH-Info-r13 to correct place, after the definition of HSDSCH-Info-r11. |  |  |
| 303 | # DL-AddReconfTransChInformation-r13 is before the –r11 container. | Move DL-AddReconfTransChInformation-r13 after DL-AddReconfTransChInformation-r11. | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 402. - We will place the definition of DL-AddReconfTransChInformation-r13 to correct place, after the definition of DL-AddReconfTransChInformation-r11. |  |  |
| 304 | # There is a new type DL-InformationPerRL-List-r13 type, but HandoverToUTRANCommand is not updated with R13 version. | HandoverToUTRANCommand-r12-IEs contains DL-InformationPerRL-List-r12. In Rel-13, a new type DL-InformationPerRL-List-r13 was added, but HandoverToUTRANCommand-r13-IEs was not created. The reason for creation of DL-InformationPerRL-List-r13 was TPC enhancements and Algorithm 3, so it should be decided whether Algorithm3 activation/configuration can be present during handover to UTRAN. | 1 | To be fixed in Correction CR |
|  |  | Comments: Nokia: No strong opinion, it’s just the Tabular and ASN.1 are not aligned. Huawei: No need to signal as part of Handover To UTRAN command. - We will add CV statement in the tabular to indicate that IE not needed in Handover To UTRAN command. |  |  |
| 305 | # DL-AddReconfTransChInfoList-r13 version should require new HandoverToUTRANCommand-r13-IEs | Missing HandoverToUTRANCommand-r13-IEs should encapsulate DL-AddReconfTransChInfoList-r13 | 1 | Closed No Update needed |
|  |  | Comments: Nothing needed. See Issue 304. |  |  |
| 306 | # In PriorityLevel-r13, “subpriority” IE type ENUMERATED is not aligned with qualifier types of other IEs | Add a tab stop right after “subpriority” (and before ENUMERATED) to align it with type qualifiers of other IEs. | 3 |  |
|  |  | Comments: |  |  |
| 307 | # “subpriority” type is explicitly specified in every occurrence | It might be beneficial to define ENUMERATED{oDot2,oDot4,oDot6,oDot8} only once, and then refer to a new type. | 3 |  |
|  |  | Comments: |  |  |
| 308 | # In DL-InformationPerSecondaryRL-r13, IE "dl-FDPCHInfoPerRL-SecULFreq" type is not aligned with other IEs | Add a tab stop right after “dl-FDPCHInfoPerRL-SecULFreq” | 3 |  |
|  |  | Comments: |  |  |
| 309 | # In RL-AdditionInformation-SecULFreq-r13, IE "dl-FDPCHInfoPerRL" type is not aligned with other IEs | Add a tab stop right after “dl-FDPCHInfoPerRL” | 3 |  |
|  |  | Comments: |  |  |
| 310 | # Legacy PowerControlAlgorithm IE is a CHOICE specifying a particular algorithm type, which conflicts logically with a purpose of IE PowerControlAlgorithm3 | The easiest way is to rename PowerControlAlgorithm3 to something like PowerControlAlgorithm3-Conf | 2 | To be fixed in Correction CR |
|  |  | Comments: - We will rename the IE as suggested, given that it refers to a full configuration rather than a choice as in the baseline. |  |  |
| 311 | # In PowerControlAlgorithm3, there is a discrepancy in how contained IEs are named | It is suggested to align IE names e.g. “tpcSlotPosition”, “tpcStepSize” | 1 | To be fixed in Correction CR |
|  |  | Comments: - We will align the IE names in ASN.1 as suggested. |  |  |
| 312 | # In UE-Positioning-AddPos-MeasuredResults-element, elements in CHOICE should be shifted right for better readability | Add a tab stop in front of each item inside CHOICE | 3 |  |
|  |  | Comments: |  |  |
| 313 | # IE UE-Positioning-MeasuredResults-vdxyext has two sub-IEs, each of which has UTC time stamp. | Check whether it is an intended thing that UTC time stamp is included in both UE-Positioning-AddPos-MeasuredResults-element and UE-Positioning-PositionEstimateInfo-vdxyext. In 10.3.7.109, the semantic description of “Location time stamp” says that it is used only for additional positioning methods, which gives an impression that it should not be duplicated in the additional positioning measurements. | 1 |  |
|  |  | Comments: NextNav. The Location Time stamp is needed as part of the Position Estimate Info. Also a timestamp is needed which relates to the measurements for the Indoor Positioning methods. No updates necessary. |  |  |
| 314 | # In 10.3.7 the content of the “Multi” field should be aligned on left | Apply the correct style to the “Multi” field in 10.3.7. | 3 |  |
|  |  | Comments: |  |  |
| 315 | # Tabular description for ASN.1 UTC time can be optimized. | There is the following description in tabular: ASN.1 UTCTime expressed as a String (SIZE(13)) with format YYMMDDhhmmssZ If it is a defined and commonly adopted ASN.1 type, then the “Type” field content can be reduced to e.g. ASN.1 UTC time, and further explanations if needed can be moved to semantic description. | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 101. - We will just use UTCTime in the Type field, and then add a clarification in the Semantic description field. |  |  |
| 316 | # uraPCHWithSeamlessTransitionRNTIHandlingMode is too long and logically inconsistent | Since uraPCHWithSeamlessTransitionRNTIHandlingMode is defined as ENUMERATED{true}, its name is logically inconsistent as “mode” assumes several options. If we keep it as ENUMERATED{true}, then the name should be changed (and can be shortened). As an example, keepRNTIUponReselection. | 1 | To be fixed in Correction CR |
|  |  | Comments: - We will update the IE name as suggested to be more meaningful. Name can be discussed further if needed when final CR is being reviewed. |  |  |
| 317 | # A logical place where uraPCHWithSeamlessTransitionRNTIHandlingMode should be | To be discussed, whether uraPCHWithSeamlessTransitionRNTIHandlingMode should be under “UTRAN mobility IEs” rather than “User equipment IEs” | 2 | Closed No Update needed |
|  |  | Comments: We keep it under “User equipment IEs” as its usage relates to C/H/E-RNTI. |  |  |
| 318 | # DL-AddReconfTransChInfoList-r13 is missed in SRNC-RelocationInfo-r13 | DL-AddReconfTransChInfoList was updated from Rel-11 to Rel-13, but SRNC relocation info still contains DL-AddReconfTransChInfoList-r11. The reason for updating DL-AddReconfTransChInfoList to Rel-13 was blind HARQ retransmissions. Should DL-AddReconfTransChInfoList-r11ld be checked whether this information must be present in SRNC relocation info. | 1 | To be fixed in Correction CR |
|  |  | Comments: Similar to issue raised in Issue 114, Issue 116, Issue 301 and Issue 420. - We will update the tabular to add the CV statement as suggested by Huawei so that it is clear the IE is not applicable to the listed RRC messages. |  |  |
| 319 | # RetrievableConfigData does not encapsulate DL-AddReconfTransChInfoList-r13 but rather DL-AddReconfTransChInfoList-r11 | Should this be updated to use latest version of the IE rather than Rel-11 version? | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 113, Issue 203 and Issue 401. - We will update the ASN.1 to reference the DL-AddReconfTransChInfoList-r13 version. |  |  |
|  |  | Ericsson |  |  |
| 400 | # Chapter 10.3.3.1a. Table does not include Version column | Information Element/Group name; Need; Multi; Type and reference; Semantics description; Version. Activation offset; MD; Integer (0..15); In radio frames. Default value is 0.; Rel-13. | 2 | To be fixed in Correction CR |
|  |  | Comments: - We will add the column in the tabular. |  |  |
| 401 | # In ASN.1, dl-TransChInfoList included in RetrievableConfigData refers to –r11 instead of -r13 | Current ASN.1: RetrievableConfigData ::= SEQUENCE { srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rb-InformationSetupList RB-InformationSetupList-r11 OPTIONAL, ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-TransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-TransChInfoList DL-AddReconfTransChInfoList-r11 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, Change to: RetrievableConfigData ::= SEQUENCE { srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rb-InformationSetupList RB-InformationSetupList-r11 OPTIONAL, ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-TransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-TransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, dtx-drx-Info DTX-DRX-Info-r12 OPTIONAL, hs-scch-LessInfo HS-SCCH-LessInfo-r7 OPTIONAL, | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 113, Issue 203 and Issue 319. - We will update the ASN.1 to reference the DL-AddReconfTransChInfoList-r13 version. |  |  |
| 402 | # In ASN.1 DL-AddReconfTransChInfoList-r13 | DL-AddReconfTransChInfoList-r13 should be placed after DL-AddReconfTransChInfoList-r11 instead, so that it’s more visible that it exists. Actually it would be better to move DL-AddReconfTransChInfoList-r11 to between r9 and r13 instead so that DL-AddReconfTransChInfoList-TDD128-v9c0ext doesn’t come in the middle. | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 303. - We will place the definition of DL-AddReconfTransChInformation-r13 to correct place, after the definition of DL-AddReconfTransChInformation-r11. |  |  |
| 403 | # Chapter 10.2.08: # Position mismatch for the new IEs between tabular and ASN.1 | # Cell Update Confirm message: # The new IE Positions missmatch between Tabular and ASN.1, # In Tabular: PDCP ROHC target mode; OP; PDCP ROHC target mode 10.3.4.2a; REL-5. Retrievable configuration info; OP; Retrievable configuration info 10.3.8.25; REL-13. Other state configuration info; OP; Other state configuration info 10.3.8.27; REL-13. # In ASN.1: pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, otherStateConfig OtherStateConfig OPTIONAL, retrievableConfigInfo RetrievableConfigInfo OPTIONAL, | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 112. - We will align the order in ASN.1 with that in the Tabular. |  |  |
| 404 | # Chapter 10.2.30: | # Radio Bearer Release # Different style to include “Retrievable configuration” in different message, needs to align? # One Style: Retrievable configuration. >>Retrievable configuration info; OP; Retrievable configuration info 10.3.8.25; REL-13. # Another: >>PDCP context relocation info; MP; PDCP context relocation info 10.3.4.1a; This IE is needed for each RB having PDCP and performing PDCP context relocation; REL-5. >>Retrievable configuration info; OP; Retrievable configuration info 10.3.8.25; REL-13. | 2 | To be fixed in Correction CR |
|  |  | Comments: - We should be consistent, so we will check if it can be done in a nice way. |  |  |
| 405 | # Chapter 10.2.30 | “Retrievable configuration” IE looks a bit strange. “RAB Information to setup list” is under “Signalling RB information”. Is it correct? Should RAB Information to setup list be the same level as SIB? >Signalling RB information to setup; MP; Signalling RB information to setup 10.3.4.24. >>RAB information to setup list; OP; 1 to < maxRABsetup >. >>>RAB information for setup; MP; RAB information for setup 10.3.4.10. | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 117 and Issue 201. - We will fix the tabular description as suggested to align with ASN.1. |  |  |
| 406 | # Chapter 10.2.30 and ASN.1. | Related to issue 405. Probably RB information to setup list should be defined below RAB information to setup list, like in the RB Setup message. Otherwise it is not clear which RB’s that belong to which RAB. | 1 | To be fixed in Correction CR |
|  |  | Comments: We need to check if we need the rb-InformationSetupList (given that it is already included as part of the RAB-Info). Should it be RB-InformationReconfigList-r11 instead? [CB] - After comeback we will remove the rb-InformationSetupList in ASN.1 at top level of RetrievableConfigData, as it is already in the RAB-InformationToSetupList. |  |  |
| 407 | # ASN.1 RetrievableConfigInfo | Minor comment: It would look nicer if the columns were aligned: RetrievableConfigInfo ::= SEQUENCE { retrievableConfigToBeInvoked RetrievableConfigIdentity OPTIONAL, retrievableConfigToBeStored RetrievableConfigIdentity OPTIONAL, retrievableConfigToBeRemoved RetrievableConfigListToRemove OPTIONAL, preconfiguredRetrievableConfig PreconfiguredRetrievableConfig OPTIONAL } Instead of: RetrievableConfigInfo ::= SEQUENCE { retrievableConfigToBeInvoked RetrievableConfigIdentity OPTIONAL, retrievableConfigToBeStored RetrievableConfigIdentity OPTIONAL, retrievableConfigToBeRemoved RetrievableConfigListToRemove OPTIONAL, preconfiguredRetrievableConfig PreconfiguredRetrievableConfig OPTIONAL } | 2 | To be fixed in Correction CR |
|  |  | Comments: - The alignment in the ASN.1 to be taken care of in the final CR. |  |  |
| 408 | # ASN.1 UE-AddPos-element | addPos-Mode is defined as an own information element group, but it’s not referenced from any other place and could be defined in the same way as addPos-ID instead. UE-AddPos-element ::= SEQUENCE{ addPos-ID ENUMERATED {barometricPressure, wlan, bt, mbs, spare4, spare3, spare2, spare1 }, addPos-Mode AddPos-Mode } | 1 | To be fixed in Correction CR |
|  |  | Comments: NextNav: We agree with the comment. - We will add the ENUMERATED definition directly within UE-AddPos-element. |  |  |
| 409 | # Chapter 10.3.3.25 | In most places it’s called “Support of”, but it’s called “Support for power control algorithm 3”. | 1 | To be fixed in Correction CR |
|  |  | Comments: - We will update the name to ‘Support of’. Also Huawei will bring a CR to 25.306 to align with the new name. |  |  |
| 410 | # Tabular and ASN.1 | Dedicated Priority Information is not placed at the same place in the message in Tabular and ASN.1. In Tabular it is placed under UE Information Elements, but in ASN.1 it is placed under UTRAN Mobility IEs. | 2 | To be fixed in Correction CR |
|  |  | Comments: - We will align so that ASN.1 matches the Tabular. |  |  |
| 411 | # Chapter 10.3.7.141 # UE positioning AddPos measured results # Mismatch between Tabular and ASN.1 | Measurement MEASUREMENT REPORT -> 10.3.7.141 UE positioning AddPos measured results >>>>WLAN AP Channel Frequency OP Integer(0..256) The AP channel number identification of the reported WLAN AP. REL-13 Should the value be (0..255) instead of (0..256)? | 1 | Closed No Update needed |
|  |  | Comments: NextNav: We agree with the comment that the correct range is 0.255. NextNav: This is also an issue in LPP and PCAP. QC: There is still some discussion on this. [CB] We do not fix this now as it is already consistent with PCAP & LPP. |  |  |
| 412 | # 10.2.22 | Dynamic activation time is only applicable to FDD only? | 2 | Closed No Update needed |
|  |  | Comments: No need to clarify. |  |  |
| 413 | # 10.2.22 | Retrievable configuration info is only applicable to FDD only? | 2 | Closed No Update needed |
|  |  | Comments: No need to clarify. |  |  |
| 414 | # 10.2.16d | The highlighted text is not needed. UE-RadioAccessCapability-vdxyext ::= SEQUENCE { ue-RadioAccessCapabBandCombList-vdxyext UE-RadioAccessCapabBandCombList-vdxyext OPTIONAL } | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 111. - We will update to remove the ‘-vdxyext’ tag from the IE names. It is enough that the IE types are tagged with ‘-vdxyext’ |  |  |
| 415 | # 10.2.16d | The nesting level with the ASN.1 for InterRATHandoverInfo-NonCriticalExtension-v6b0-IEs has reached 16 which will likely cause some compilers to fail. At RAN2#89 companies decided that work around can be done by companies themselves, but is it better to be pro-active and ensure it is done in consistent way? Also from a readability point of view it might be good to break the indentation. | 1 | Closed No Update needed |
|  |  | Comments: [CB] It was agreed that a more general check is done on all messages which may be affected and then to discuss at later meeting if the nesting levels should be reset for all at the same time. |  |  |
| 416 | # 10.2.33 | The tabular and ASN.1 are not aligned since REL-6/7, when the CHOICE ‘Default configuration’ was removed: RB Information Elements. CHOICE specification mode; MP; REL-6. >Complete specification; In this version of the specification, only this value is specified; REL-6. RadioBearerSetup-r13-IEs ::= SEQUENCE { -- User equipment IEs integrityProtectionModeInfo IntegrityProtectionModeInfo-r7 OPTIONAL, cipheringModeInfo CipheringModeInfo-r7 OPTIONAL, sr-vcc-Info SR-VCC-Info OPTIONAL, activationTime ActivationTime OPTIONAL, dynamicActivationTime DynamicActivationTime OPTIONAL, new-U-RNTI U-RNTI OPTIONAL, new-C-RNTI C-RNTI OPTIONAL, -- The IE "new-DSCH-RNTI" should not be included in FDD mode, -- and if received the UE behaviour is unspecified new-DSCH-RNTI DSCH-RNTI OPTIONAL, new-H-RNTI H-RNTI OPTIONAL, newPrimary-E-RNTI E-RNTI OPTIONAL, newSecondary-E-RNTI E-RNTI OPTIONAL, rrc-StateIndicator RRC-StateIndicator, utran-DRX-CycleLengthCoeff UTRAN-DRX-CycleLengthCoefficient-r7 OPTIONAL, accessGroupIdentity AccessGroupIdentity OPTIONAL, -- UTRAN mobility IEs ura-Identity URA-Identity OPTIONAL, supportForChangeOfUE-Capability BOOLEAN OPTIONAL, -- Core network IEs cn-InformationInfo CN-InformationInfo-r6 OPTIONAL, -- Radio bearer IEs srb-InformationSetupList SRB-InformationSetupList-r11 OPTIONAL, rab-InformationSetupList RAB-InformationSetupList-r11 OPTIONAL, rab-InformationReconfigList RAB-InformationReconfigList-r8 OPTIONAL, rb-InformationReconfigList RB-InformationReconfigList-r11 OPTIONAL, rb-InformationAffectedList RB-InformationAffectedList-r8 OPTIONAL, dl-CounterSynchronisationInfo DL-CounterSynchronisationInfo-r5 OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-deletedTransChInfoList UL-DeletedTransChInfoList-r6 OPTIONAL, ul-AddReconfTransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-DeletedTransChInfoList DL-DeletedTransChInfoList-r7 OPTIONAL, dl-AddReconfTransChInfoList DL-AddReconfTransChInfoList-r13 OPTIONAL, -- Retrievable configuration retrievableConfigInfo RetrievableConfigInfo OPTIONAL, -- Physical channel IEs frequencyInfo FrequencyInfo OPTIONAL, | 2 | To be fixed in Correction CR |
|  |  | Comments: - We will align the Tabular to the ASN.1. |  |  |
| 417 | # 10.2.33 | The ASN.1 can be updated as follows to align better with Tabular description RetrievableConfigInfo ::= SEQUENCE { retrievableConfigToBeInvoked RetrievableConfigIdentity OPTIONAL, retrievableConfigToBeStored RetrievableConfigIdentity OPTIONAL, retrievableConfigToBeRemoved RetrievableConfigListToRemove OPTIONAL, preconfiguredRetrievableConfig PreconfiguredRetrievableConfig OPTIONAL } PreconfiguredRetrievableConfig ::= SEQUENCE { retrievableConfigList RetrievableConfig-List } RetrievableConfig-List ::= SEQUENCE (SIZE(1.. maxRetrievConfig)) OF RetrievableConfig RetrievableConfig ::= SEQUENCE { retrievableConfigIdentity RetrievableConfigIdentity, retrievableConfigData RetrievableConfigData } | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 200. - We will align the ASN.1 to the tabular by removing the unnecessary PreconfiguredRetrievableConfig. |  |  |
| 418 | # 10.2.48.8.28 | The ASN.1 and tabular do not match Information Element/Group name; Need; Multi; Type and reference; Semantics description; Version. ACDC Parameters; MP; If the cell is operating in MBSFN mode as indicated in subclause 8.1.1.6.3, the UE behaviour upon reception of this IE is unspecified.; REL-13. >CHOICE barring representation; MP; REL-13. SysInfoType25 ::= SEQUENCE { acdc-Parameters ACDC-Parameters, nonCriticalExtensions SEQUENCE {} OPTIONAL } ACDC-Parameters ::= CHOICE { acdc-ParametersPerPLMN-List ACDC-ParametersPerPLMN-List, acdc-ParametersForAll ACDC-ParametersForAll } | 2 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 110. - We will align the ASN.1 to the Tabular description. |  |  |
| 419 | # 14.12.4.2 | In the ASN.1, the definition of HSDSCH-Info-r13 is based on HSDSCH-Info-r9 and not HSDSCH-Info-r11. HSDSCH-Info-r9 ::= SEQUENCE { harqInfo HARQ-Info-r7 OPTIONAL, dl-MAC-HeaderType CHOICE { mac-hs AddOrReconfMAC-dFlow, mac-ehs AddOrReconfMAC-ehs-ReordQ-r9 } OPTIONAL } HSDSCH-Info-r11 ::= SEQUENCE { harqInfo HARQ-Info-r11 OPTIONAL, dl-MAC-HeaderType CHOICE { mac-hs AddOrReconfMAC-dFlow, mac-ehs AddOrReconfMAC-ehs-ReordQ-r11 } OPTIONAL } HSDSCH-Info-r13 ::= SEQUENCE { harqInfo HARQ-Info-r13 OPTIONAL, dl-MAC-HeaderType CHOICE { mac-hs AddOrReconfMAC-dFlow, mac-ehs AddOrReconfMAC-ehs-ReordQ-r9 } OPTIONAL } | 1 | To be fixed in Correction CR |
|  |  | Comments: Same issue as reported in Issue 115. - We need to update the HSDSCH-Info-r13 so that it uses the HSDSCH-Info-r11 as the baseline, and not HSDSCH-Info-r9. |  |  |
| 420 | # 14.12.4.2 | In ASN.1, dl-TransChInfoList included in SRNC-RelocationInfo-r13-IEs refers to –r11 instead of -r13 -- Radio bearer IEs predefinedConfigStatusList PredefinedConfigStatusList, srb-InformationList SRB-InformationSetupList-r11, rab-InformationList RAB-InformationSetupList-r11 OPTIONAL, pdcp-ROHC-TargetMode PDCP-ROHC-TargetMode OPTIONAL, -- Transport channel IEs ul-CommonTransChInfo UL-CommonTransChInfo-r12 OPTIONAL, ul-TransChInfoList UL-AddReconfTransChInfoList-r8 OPTIONAL, dl-CommonTransChInfo DL-CommonTransChInfo-r4 OPTIONAL, dl-TransChInfoList DL-AddReconfTransChInfoList-r11 OPTIONAL, -- PhyCH IEs tpc-CombinationInfoList TPC-CombinationInfoList OPTIONAL, e-RGCH-CombinationInfoList E-RGCH-CombinationInfoList OPTIONAL, storedCompressedModeInfo StoredCompressedModeInfo-r8 OPTIONAL, secondary-tpc-CombinationInfoList TPC-CombinationInfoList-r9 OPTIONAL, secondary-e-RGCH-CombinationInfoList E-RGCH-CombinationInfoList-r9 OPTIONAL, | 1 | To be fixed in Correction CR |
|  |  | Comments: Similar to issue raised in Issue 114, Issue 116, Issue 301 and Issue 318 - We will update the tabular to add the CV statement as suggested by Huawei so that it is clear the IE is not applicable to the listed RRC messages. |  |  |
| 421 | # 10.2.19 There is no need for the additionalMeasuredResults IE, as the measured results IE can contain results for up to 8 AddPos methods (some of which are a list themselves) | MeasurementReport-vdxyext-IEs ::= SEQUENCE { -- Measurement Ies measuredResults MeasuredResults-vdxyext OPTIONAL, additionalMeasuredResults MeasuredResultsList-vdxyext OPTIONAL } | 1 | To be fixed in Correction CR |
|  |  | Comments: NextNav: We will check, could be that we just followed what was there for existing positioning methods. [CB] - We will remove the additionalMeasuredResults from the ASN.1. |  |  |
| 422 | # 10.2.62 # The Priority Level List IE has been extended to a size of 16, but yet there are only 11 priorities. # Should the value range of the “priority” IE also be extended upto 16? | >>Priority Level List; OP; 1 to <maxPrio>. 1 to <maxPrio-ext>. >>>priority; OP; Integer (0.. <maxPrio–1>); If this IE is absent then the UE behaviour is unspecified. 0 is the lowest priority and maxPrio-1 is the highest.. | 1 | Closed No Update needed |
|  |  | Comments: Nokia: We intentionally left this during our CR drafting. So no update needed. |  |  |
| 423 | # 10.2.62 # If the UE behaviour is unspecified if the “priority” IE is not present, does this mean that in reality the IE should be MP rather than OP? # If so, should we change this in the ASN.1 for PriorityLevel-r13? | >>Priority Level List; OP; 1 to <maxPrio>; REL-8. 1 to <maxPrio-ext>; REL-13. >>>priority; OP; Integer (0.. <maxPrio–1>); If this IE is absent then the UE behaviour is unspecified. 0 is the lowest priority and maxPrio-1 is the highest.; REL-8. PriorityLevel-r13 ::= SEQUENCE { priority INTEGER (0..maxPrio-1) OPTIONAL, subpriority ENUMERATED {oDot2,oDot4,oDot6,oDot8} OPTIONAL, radioAccessTechnology CHOICE { utraFDD SEQUENCE (SIZE (1..maxNumFDDFreqs)) OF SEQUENCE{ uarfcn UARFCN }, utraTDD SEQUENCE (SIZE (1..maxNumTDDFreqs)) OF SEQUENCE{ uarfcn UARFCN }, eutra SEQUENCE (SIZE (1..maxNumEUTRAFreqs)) OF EARFCNRange, gsm SEQUENCE { gsm-CellGroup GSM-CellGroup } } } | 1 | Closed No Update needed |
|  |  | Comments: QC: It may have been the case that it was OP and then later agreed that it was MP, but as we could not update ASN.1 it was just clarified in Semantic Description. We will not change now. |  |  |
| 424 | # 13.4.15c # Similar comment as 423. | Priority Info List; OP; 1 to <maxNumPrio>; REL-8. >priority; OP; Integer (0..<maxPrio–1>); Absence of this IE indicates that no priority is assigned to the indicated frequencies; REL-8. | 2 | Closed No Update needed |
|  |  | Comments: Nothing needed. See Issue 423. |  |  |
| 425 | # 10.3.7.141 # Missalignment between tabular and ASN.1 | In tabular, IE “Code Phase RMS Error” is defined as Enumerated. However in ASN.1 it’s INTEGER UE-Positioning-MBSMeasurementList-element ::= SEQUENCE { transmitterId INTEGER (0..32767), codePhase INTEGER (0..2097151), codePhaseRmsError INTEGER (0..63) It’s suggested to change to INTEGER in tabular and in the comment refer to the Range index column of note 2 | 1 | To be fixed in Correction CR |
|  |  | Comments: NextNav: We agree that there is a misalignment. - We will update the Tabular to align with the ASN.1 |  |  |