3GPP TSG-SA3 Meeting #82	S3-160277

# Dubrovnik, Croatia, February 1 – 5, 2016				revision of S3-152251

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.401 | CR | 0555 | rev | 1 | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security support for LWIP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent, Nokia Networks, AT&T, Sprint, T-Mobile, Intel, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_WLAN_radio_legacy-Core |  |  |  |  |  |  | Date: |  | 2016-2-2 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | LTE-WLAN integration using legacy WLAN (LWIP) solution proposed by RAN requires security support to prevent potential vulnerabilities related to exposure of the eNB to the public internet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To alleviate the possible attacks on the eNB and the operator network from the public Internet, the Security Gateway LWIP-SeGW function is placed between the 3GPP operator network and the WLAN. The public IP address of the LWIP-SeGW is sent to the UE over the secure RRC signalling. The LWIP-SeGW is informed by the eNB to expect the IKEv2 initiation from the UE using the IDi assigned by the eNB. The IKEv2 handshake between the UE and the LWIP-SeGW is using the PSK mode with the key bootstrapped from the current AS security association. This key is provided by the eNB to the LWIP-SeGW, in association with the IDi, during the off-load initiation. Use of these measures mitigates potential sited vulnerabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of security support for the LWIP feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, New Clause Y, New Annex A.Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

************************************* start of change **************************

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AES	Advanced Encryption Standard

AK	Anonymity Key

AKA	Authentication and Key Agreement

AMF	Authentication Management Field

AN	Access Network

AS	Access Stratum

AUTN	Authentication token

AV	Authentication Vector

ASME	Access Security Management Entity

Cell-ID	Cell Identity as used in TS 36.331 [21]

CK	Cipher Key

CKSN	Cipher Key Sequence Number

C-RNTI	Cell RNTI as used in TS 36.331 [21]

CRL	Certificate Revocation List

DeNB	Donor eNB

DoS	Denial of Service

DSCP	Differentiated Services Code Point

EARFCN-DL	E-UTRA Absolute Radio Frequency Channel Number-Down Link

ECM	EPS Connection Management

EEA	EPS Encryption Algorithm

EIA	EPS Integrity Algorithm

eKSI	Key Set Identifier in E-UTRAN

EMM	EPS Mobility Management

eNB	Evolved Node-B

EPC	Evolved Packet Core

EPS	Evolved Packet System

EPS-AV	EPS authentication vector

E-UTRAN	Evolved UTRAN

GERAN	GSM EDGE Radio Access Network

GUTI	Globally Unique Temporary Identity

HE	Home Environment

HFN	Hyper Frame Number

HO	Hand Over

HSS	Home Subscriber Server

IK	Integrity Key

IKE	Internet Key Exchange

IMEI	International Mobile Station Equipment Identity

IMEISV	International Mobile Station Equipment Identity and Software Version number

IMSI	International Mobile Subscriber Identity

IRAT	Inter-Radio Access Technology

ISR	Idle Mode Signaling Reduction

KDF	Key Derivation Function

KSI	Key Set Identifier

LWIP				LTE WLAN RAN Level Integration using IPSec

LSB				Least Significant Bit

LSM	Limited Service Mode

MAC-I	Message Authentication Code for Integrity (terminology of TS36.323 [12])

MACT	Message Authentication Code T used in AES CMAC calculation

ME	Mobile Equipment

MME	Mobility Management Entity

MME-RN	MME serving the RN

MS	Mobile Station

MSC	Mobile Switching Center

MSIN	Mobile Station Identification Number

NAS	Non Access Stratum

NAS-MAC	Message Authentication Code for NAS for Integrity (called MAC in TS24.301 [9])

NCC	Next hop Chaining Counter

NH	Next Hop

OCSP	Online Certificate Status Protocol

OTA	Over-The-Air (update of UICCs)

PCI	Physical Cell Identity as used in TS 36.331 [21]

PDCP	Packet Data Convergence Protocol

PLMN	Public Land Mobile Network

PRNG	Pseudo Random Number Generator

PSK	Pre-shared Key

P-TMSI	Packet- Temporary Mobile Subscriber Identity

RAND	RANDom number

RAU	Routing Area Update

RN	Relay Node

RRC	Radio Resource Control

SEG	Security Gateway

SGSN	Serving GPRS Support Node

SIM	Subscriber Identity Module

SMC	Security Mode Command

SN	Serving Network

SN id	Serving Network identity

SQN	Sequence Number

SRB	Source Route Bridge

SRVCC	Single Radio Voice Call Continuity

S-TMSI	S-Temporary Mobile Subscriber Identity

TAI	Tracking Area Identity

TAU	Tracking Area Update

UE	User Equipment

UEA	UMTS Encryption Algorithm

UIA	UMTS Integrity Algorithm

UICC	Universal Integrated Circuit Card

UMTS	Universal Mobile Telecommunication System

UP	User Plane

USIM	Universal Subscriber Identity Module

UTRAN	Universal Terrestrial Radio Access Network

XRES	Expected Response

*************************************** end changes *******************************

*************************************** begin changes ***********************

# Y. LTE-WLAN RAN level integration using IPSec tunnelling

## Y.1	General

This clause describes the security functions necessary to support LTE-WLAN integration using IPSec tunnelling as described in TS 36.300 [30].

The LTE-WLAN integration architecture is shown in Figure Y.1-1.

Figure Y.1-1 LTE-WLAN integration architecture using IPSec tunnelling

For LTE-WLAN integration using IPSec tunnelling the integration happens using PDCP SDUs above the PDCP layer. The eNB controls activation of the integration based on the UE connectivity with a specific WLAN. Once the integration is activated, the eNB segregates incoming DL packets towards the UE for offloading via the WLAN at a layer above PDCP. The UL packets from the UE are aggregated by the eNB at the same logical point.

Since PDCP security is bypassed for the data routed through the WLAN and security of the legacy WLAN is not assumed, security for the PDCP SDUs and protection of the operator network shall be achieved in the following way:

-	A LWIP-SeGW shall be placed between the eNB and the WLAN network for security of packets that traverse WLAN and to protect the Operator’s network.

-	The interface between the eNB and the LWIP-SeGW shall be confidentiality and integrity protected by NDS/IP TS 33.210 [xx].

-	An UE-specific IPsec tunnel shall be established between the UE and the public IP port of the LWIP-SeGW.

-	In addition to terminating IPsec from the UE, the LWIP-SeGW shall perform rate limitation for DoS protection on the eNB and its backhaul links.

-	UEs, including authenticated and authorized UEs using LWIP, shall not have IP connectivity to the eNB.

-	The UE and the LWIP-SeGW function shall perform mutual authentication in the phase 2 of the IKEv2 handshake during the IPsec tunnel establishment, using the authentication key derived from the current AS security association.

In addition, before the IPsec tunnel is established between the UE and the LWIP-SeGW, and before the offload can be performed, the UE needs to obtain IP connectivity across the WLAN network, which may require an access authentication independent of the EPC authentication, and is outside the scope of this specification.

## Y.2	Security of LTE-WLAN integration using IPSec Tunnelling

### Y.2.1	eNB to UE interaction for setting up the LWIP offload

When the eNB initially establishes LWIP with the UE, the eNB and the UE shall generate the LWIP security key, LWIP-PSK, as specified in clause Y.4, to be used as the PSK for the IPSec tunnel set up between the UE and the LWIP-SeGW, as described in clause Y.2.2.

The eNB shall provide to the UE, over the secure RRC signalling, the following parameters:

- IP address of the LWIP-SeGW for the IKEv2 handshake,

- The Initiator Identity value, IDi, that the UE shall use in the IKEv2 handshake.

### Y.2.2	UE to LWIP-SeGW interaction for setting up the LWIP offload.

LTE-WLAN integration (LWIP) over legacy WLAN is secured using an IPSec in a tunnel mode established between the UE, via the WLAN, and the LWIP-SeGW function. The IPSec in tunnel mode is established using the IKEv2 handshake based on the pre-shared key, PSK [Ref. IETF RFC7296]. The UE and LWIP-SeGW shall use the LWIP-PSK as the PSK for authentication in the second phase of IKEv2.

When conducting the IKEv2 handshake, the UE shall use the value of IDi and the IP address of the LWIP-SeGW received from the eNB.

The LWIP-SeGW shall use the received value of IDi to locate the corresponding LWIP-PSK.

Note: To improve the DoS protection of the public IP port of the LWIP-SeGW, the LWIP-SeGW function may expect initiation of the IKEv2 handshake from the UE for a limited time window, based on a configuration. After expiration of this window, the LWIP-SeGW function may delete the LWIP-PSK and associated IDi, and reject any IKEv2 handshake initiations.

After successful completion of the IKEv2 handshake, the LWIP-SeGW and the UE shall delete the LWIP-PSK.

For LWIP offloaded traffic, the eNB shall only be reachable through the LWIP-SeGW.

The LWIP-SEGW shall allow communication of the UE only to the eNB that initiated the LWIP offload, and only to the interface on this eNB allowed for the LWIP offload.

The profiles for IKEv2 and IPsec ESP as defined in TS 33.310 [ref] shall be used.

## Y.2.3	eNB to LWIP-SeGW interaction for setting the LWIP offload

The PDCP SDUs between the eNB and LWIP SeGW shall be encapsulated in a tunnelling protocol in order to avoid that the eNB needs to interpret IP packets coming from the UE.

The eNB shall inform the LWIP-SeGW function of the expected initiation of IKEv2 handshake by a UE, for subsequent establishment of the IPsec, and provide the following parameters:

- the Initiator ID value, (IDi) that the UE will use in the IKEv2 handshake,

- the LWIP-PSK.

The standardized interface between the eNB and the LWIP-SeGW shall be confidentiality and integrity protected by NDS/IP TS 33.210 [xx].

## Y.3	Addition and modification of DRB in LTE-WLAN integration

All DRBs associated with the same UE and routed through WLAN shall use the same IPSec tunnel established between the UE and the LWIP-SeGW function. The eNB manages the DRB addition and deletion as specified in TS 36.300 [30]. When the last DRB between the eNB and UE is deleted, the eNB shall instruct the LWIP-SeGW and the UE to release the IPSec tunnel.

## Y.4	Security Key for IKEv2 handshake

### Y.4.1	Security Key (LWIP-PSK) Derivation

The UE and eNB shall derive the security key LWIP-PSK for the IPSec tunnel set up as shown on the Fig.Y.4.1-1 and defined in Annex A.Y of the present specification.

Fig.Y.4.1-1	LWIP-PSK Derivation.

### Y.4.2	Security key (LWIP-PSK) update

The eNB may update the LWIP-PSK for any reason. If the eNB refreshes its AS security context, the eNB shall instruct the LWIP-SeGW function to release the current IPSec tunnel, and provide  a new LWIP-PSK derived from the new KeNB to support establishment of the new IPSec. The eNB shall instruct the UE over the RRC signaling to re-initiate the IKEv2 using the new LWIP-PSK to establish a new IPSec tunnel.

## Y. 5	Handover procedures

During S1 and X2 handover, the IPSec tunnel between the UE and the LWIP-SeGW shall be released. The eNB shall instruct the LWIP-SeGW and the UE to release the IPSec. Both the LWIP-SeGW and the UE shall delete the LWIP-PSK.

## Y. 6	LWIP radio link failure

When a LTE radio link failure is detected, the IPSec tunnel between the UE and the LWIP SeGW shall be released, either by the eNB informing the LWIP-SeGW of this event, or at the UE. If the IPSec tunnel between the UE and the LWIP-SeGW is released due to WLAN connectivity issues, a fresh LWIP IPSec tunnel set up may be performed when WLAN wireless connectivity is restored.

********************************************* End of change

************************************************* start of change ************

## A.Y	Derivation of LWIP-PSK

This input string is used when the eNB and UE derive LWIP-PSK from KeNB during LTE WLAN integration using IPSec. The following input parameters shall be used:

-	FC = 0x1E

-	P0 = “LWIP” (i.e. 0x4c 0x57 0x49 0x50)

-	L0 = 4 octets, (i.e 0x00 0x04).

The input key shall be KeNB of the eNB.

*************************************** end changes *******************************