3GPP TSG RAN WG2 Meeting #93bis	R2-162186

# Dubrovnik, Croatia, 11th  – 15th  April 2016

Source:	CATT

Title:	UE Location Collection for WLAN Mobility Set Configuration

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In Rel-13 LWA, eNB can configure WLAN mobility set to UE, e.g. based on UEs measurement reports. The size and content of WLAN mobility set are based on eNB’s implementation. Since the UE’s measurement report trigger is based on BeaconRSSI, eNB may collect measurement reports for many times to determine the WLAN mobility set for a UE. This will cause large signalling overhead. To improve the WLAN mobility set configuration in Rel-14[1], this document suggests some more information to be considered by eNB, such as UE’s location and mobility state.

Discussion

The WLAN mobility set for a UE mainly includes some WLAN APs nearby, and UE can move in this WLAN mobility set transparently to the eNB. Therefore, if eNB knows the deployment of WLAN APs and the UE’s location, it can determine the UE’s WLAN mobility set configuration easily. An example is shown in figure 1.

Figure 1 WLAN mobility set configuration

In figure1, there are two groups of APs in the coverage of eNB, and UE sends the measurement report (including BeaconRSSI of AP1 and UE’s location) to eNB when it moves into the coverage of AP1in AP Group1.  Then the eNB can configure all the APs (AP1, AP2 and AP3) in AP Group1 to UE as the WLAN mobility set. This will reduce the configuration latency and the measurement report signalling.

Proposal 1: Include UE’s location information optionally in the WLAN related measurement reports.

Besides the UE’s location information, the UE’s mobility state is also useful for the WLAN mobility set configuration. For example, for a low mobility UE, the coverage of APs in WLAN mobility set can be small. This can also improve the performance of WLAN mobility set configuration.

Proposal 2: Include UE’s mobility state optionally in the WLAN related measurement reports.

Conclusion

It is proposed to discuss and capture the following proposals at RAN2:

Proposal 1: Include UE’s location information optionally in the WLAN related measurement reports.

Proposal 2: Include UE’s mobility state optionally in the WLAN related measurement reports.

References

[1] RP-160600, New Work Item on enhanced LTE-WLAN Aggregation (LWA), by Intel, Qualcomm, China Telecom