3GPP TSG-RAN WG2 Meeting #93-BIS	R2-162787

Dubrovnik, Croatia, 11 - 15 April, 2016

Source: 	Huawei, HiSilicon

Title: 	Emerging Vehicular Communication Applications

Agenda Item:	9.4.2

Document for:	Discussion and Approval

# Introduction

3GPP is studying scenarios and RAN level requirements for 5G networks [1]. This study is based on three families of use cases: (a) Enhanced Mobile Broadband, (b) Massive Machine-type Communications, and (c) Ultra-reliable and Low Latency Communications. The classification into these three families tries to capture the characteristics that are relevant to the RAN; it is based on a 3GPP SA study on support of new services and markets [2], and on the output of other industry bodies such as ITU [3], NGMN [4], 5GPPP [5], etc.

The automotive industry is expected to be one of the major beneficiaries of 5G infrastructure as vehicular technologies increasingly move towards automation. Driving automation (self-driving cars) has emerged as one of the important use cases [4][5] and there is a clear desire to ensure appropriate support for it in 5G infrastructure. Features related to automation will be included in vehicles in a phased manner and the various forms of automation will require network connectivity. We refer to such vehicles as “Connected Vehicles”.

Connected Vehicles are UEs from the point of view of a network; however, they have characteristics that are very different from traditional UEs. In this contribution we analyze how connected vehicles can impact the network and air-interface procedures.

# Discussion

## Categories of Vehicular Communications Applications

We refer to vehicles with (some) automated driving capabilities and network connectivity as Connected Vehicles (CVs). The degree of automation and the extent to which network connectivity is used can vary. However, the types of data usage in CVs can be classified into the following main categories:

- Navigation, Warning and Safety Applications:

  - Navigation capabilities to assist drivers are already present in many vehicles; however it is expected that they will be improved with assistance of network communication to have better traffic flow and minimize congestion. As vehicles move towards more automation, the need for high accuracy in navigation increases.

  - Warnings and Safety Applications include warnings to a driver about unexpected events, such as a vehicle around a blind corner, speeding vehicles that may cause a collision, etc. Such warnings provided to human drivers are clearly beneficial and can help minimize traffic accidents. With increasing driving automation, such applications are even more important to the driving function, because the sensing capabilities of vehicles are limited by distance.

- Network Assisted Driving: A CV with some automated driving capabilities can utilize fine-grained information about the environment around the car. Such information can include information that the cars sensors cannot provide (such as vehicles or obstacles that are outside the range of the sensors or blocked). For example, vehicle A could be provided an indication that vehicle B in front of it is likely to collide with a vehicle C in front of vehicle B – which could then be used to apply the brakes and avoid a multi-vehicle collision. Clearly such applications require low latency communication between the CV and the infrastructure.

- Remote Processing of Driving functions:

  - Some functions that assist in driving can be performed remotely. For example, a node in the infrastructure can construct a visualization or 3D map of activity at an upcoming intersection. Constructing such a visualization for the user requires collecting data from multiple sensors/cameras and vehicles and may be computationally intensive in a vehicle. Such applications would require high bandwidth and low latency.

  - Furthermore, vehicles that can be remotely driven are being considered – i.e., replacement of the human driver in the vehicle with either a remote human driver [7] or sophisticated centralized driving software in the network. If the centralized driving software is used, it would be possible to use data from other vehicles.

- “Infotainment”: As driving becomes less and less of a human task, occupants of the vehicle – including the human driver – can perform other tasks. This may include activities such as catching up on news, working or even watching a movie. Unlike smartphones, vehicles can be equipped with relatively large video monitors and augmented reality capabilities. Therefore the data rate demand from CVs is expected to be much higher than that for smartphones.

Based on the above classes of applications, communications involving CVs are expected to have the following characteristics: (a) High data rates with high mobility, (b) Low Latency, and (c) High Reliability. Combined with the fact the vehicle density in urban areas can be very high, CVs simultaneously need all of the three major drivers that 5G is targeting: namely Enhanced mobile broadband, Massive machine-type-communications, and Ultra reliable and low latency Communications.

3GPP RAN has recently agreed on deployment scenarios related to vehicular communication in [9] for purposes of evaluation. A highway scenario and an urban grid for connected car scenario have been agreed for inclusion in [1].

## Characteristics of CV communication and Network Impacts

Below we look further into the aspects of CV communication that can influence air interface and network design.

### High Data Rate and Low latency with High Mobility

As discussed above, practically all of the CV communication scenarios require high data rates. Other than infotainment, all the scenarios will require low latency as well.

To better understand the need for low latency, consider the network assisted driving use case. Suppose it is necessary to communicate to a CV that a small obstacle has suddenly entered the road (e.g. a rock has fallen on the road or an animal has strayed onto the road) at a distance of 20 ft (~6 m) in front of the vehicle. If the vehicle is moving at 80 m.p.h., the vehicle has only about 200 ms before it has to complete the maneuver to avoid the obstacle. Given that the mechanical elements of the vehicle can take a large portion of the 200 ms, it is clearly necessary to ensure that the indication is delivered as quickly as possible.

Reference [1] has two related requirements. The target for user plane latency is as low as 0.5 ms. The target for mobility interruption time is 0 ms! These aggressive requirements, particularly the one for interruption time, are very suitable for CVs. However, it is important to ensure that such aggressive requirements are not met by adding substantial complexity to the RAN. For example, if data for each UE has to be available at large numbers of TRPs in order to meet the 0 ms interruption requirement, the backhaul capacities needed can be overwhelming.

Considering that vehicle density in urban areas can be very high during busy periods, the frequency of mobility events can be very high. Smartphones, even if they are in vehicles, typically do not experience such high frequency of mobility events (most would be in idle or at least long DRX).

For some vehicular communication applications, such as safety applications and network assisted driving, any interruption of communication may have disastrous results. Thus guaranteeing a 0 ms mobility interruption target is absolutely critical in such scenarios.

Proposal 1: NR needs to support enhanced procedures for handling mobility, such that 0 interruption time can be achieved for critical data streams.

### Need for highly reliable connectivity

Highly reliable connectivity is crucial for CVs. However, a wireless link to a fast moving device faces very challenging radio conditions. It is not unlikely that consecutive packet failures can effectively interrupt data transmission for an unacceptable time. When such a situation occurs, it is essential to detect it and recover the link quickly. In LTE, the radio link recovery procedure takes at least 200-300 ms. For the CV communication applications, this is clearly not sufficient.

Observation 1: For CVs, mechanisms to detect and recover from consecutive packet failures that are much quicker than radio link recovery in LTE need to be designed.

Proposal 2: NR to target recovery mechanisms for CVs that limit interruption to ~10 ms in case of consecutive packet failures.

Another aspect to consider is what constitutes reliable connectivity. For example, one could define highly reliable network connectivity in the following form: the probability of a UE within a radius r from the access point experiences a connection outage for a period of at least t ms is no more than p. Such reliability considerations are under discussion and [9] provides an initial framework.

### Infrastructure power consumption constraints

Road-side infrastructure for CVs may be able to use components of existing transportation infrastructure (e.g., traffic lights) where available, enabling a continuous power supply. However, in rural areas, road-side infrastructure will need to be placed even if no transportation infrastructure is available. Alternative means of powering the infrastructure using batteries along with other means such as solar and wind power will be needed in such cases. Given such limitations, it is important to minimize power consumption at the infrastructure. This is in contrast to the current model, where we normally try to minimize the power consumption of the UE and not the RAN.

Even if power supply is obtained from existing transportation infrastructure, interruption in power supply should not cause a failure of communication – this can have disastrous consequences. So, even for such cases, minimization of power consumption at the infrastructure is important.

Observation 2: The air interface has to be designed to ensure that TRPs can provide essential communication capabilities even under challenging power supply conditions.

Proposal 3: RAN2 should study design solutions for the NR air interface that allow for cases in which the TRP is in a low power state but the UE is not in a low power state.

### Vehicle to Vehicle Communication

Wherever possible, V2V communication bypassing road-side infrastructure can be used to support safety applications. The V2V communication framework in LTE is based on having a group of two or more UEs that have discovered each other, and performing broadcast transmissions. For safety applications, it is necessary to have very fast discovery and synchronization. Moreover, not every message or communication between vehicles should be broadcast. In particular some control messages (e.g. to request the forwarding or broadcast of certain sensor information) may need to be sent to a specific vehicle. Hence, unicast V2V communications are necessary.

Proposal 4: NR based should support fast UE discovery and at least unicast Vehicle to Vehicle communication.

### Broadcast & Multicast Communication

Broadcast and Multicast communications also can play an important role in the case of warnings and safety applications. For example, it may be very critical to provide information about an event (e.g. highway accident) to vehicles that are in close proximity, so that automated braking can be applied. In this case, the information may be provided to a subset of vehicles that are in immediate danger, but it may not be necessary for every vehicle in the cell to get this information. Multicast can be a very effective way to disseminate such information to a subset of vehicles that need it. On the other hand, it may be useful to broadcast information about the resulting delays to all vehicles, so that navigation systems can adjust planned routes to avoid the area of the incident.

Proposal 5: NR should support both broadcast and multicast mechanisms for efficient communication to a group of UEs.

# Conclusion

In this contribution we considered the emerging use cases related to connected vehicles. Many of these use cases are not implementable with the currently available technology (including LTE technology). CV communication is expected to depend on all three of the major drivers that 5G is targeting: Enhanced mobile broadband, Massive machine-type-communications, and Ultra reliable and low latency Communications.

Based on our discussion and analysis we have the following observations and proposals:

Proposal 1: NR needs to support enhanced procedures for handling mobility, such that 0 interruption time can be achieved for critical data streams.

Observation 1: For CVs, mechanisms to detect and recover from consecutive packet failures that are much quicker than radio link recovery in LTE need to be designed.

Proposal 2: NR to target recovery mechanisms for CVs that limit interruption to ~10 ms in case of consecutive packet failures.

Observation 2: The air interface has to be designed to ensure that TRPs can provide essential communication capabilities even under challenging power supply conditions.

Proposal 3: RAN2 should study design solutions for the NR air interface that allow for cases in which the TRP is in a low power state but the UE is not in a low power state.

Proposal 4: NR based should support fast UE discovery and at least unicast Vehicle to Vehicle communication.

Proposal 5: NR should support both broadcast and multicast mechanisms for efficient communication to a group of UEs.

# References

- 3GPP TR 38.913 V0.2.0, “Study on Scenarios and Requirements for Next Generation Access Technologies”, February 2016.

- 3GPP TR 22.891 V1.3.2, “Feasibility Study on New Services and Markets Technology Enablers,” February 2016.

- Recommendation ITU-R M.2083: IMT Vision - "Framework and overall objectives of the future development of IMT for 2020 and beyond"​ (September 2015).

- ”Automotive Perspective on 5G,” Toyota, NGMN P1 WS2 Vertical Requirements Meeting, San Jose, CA, U.S.A., February 2016.

- ”5G Automotive Vision,” 5GPPP, Oct 2015.

- http://www.forbes.com/sites/federicoguerrini/2014/10/14/traffic-congestion-costs-americans-124-billion-a-year-report-says/#565d8d096252

- http://www.wired.com/2015/01/fords-working-remote-control-car/

- RP-151109, “New SI proposal: Feasibility Study on LTE-based V2X Services”.

- RP-160637, “CR on eV2X scenarios and KPIs”, 3GPP TSG-RAN#71.