3GPP TSG-RAN WG2 #93bis	R2-162869

# Dubrovnik, Croatia, Apr. 11 – 15, 2016

Agenda item:	7.14.2.3 (NB_IOT-Core)

Source:	LG Electronics Inc.

Title: 	Measurement Rules for Cell Reselection for NB-IoT

Document for:	Discussion and Decision

Introduction

In the last RAN2 meetings [1]-[3], RAN2 agreed followings regarding cell reselection for NB-IoT:

Agreements from RAN2 #92 meeting [1]

Following SI fields are supported with different values of field than those in Rel-13 LTE:

- interFreqCarrierFreqList (*) excluding t-ReselectionEUTRA-SF, allowedMeasBandwidth, 	presenceAntennaPort1, cellReselectionPriority, neighCellConfig (which is not supported)

Agreements from RAN2 NB-IoT Ad hoc meeting [2]

1) Intra-frequency cell reselection is based on ranking.

Agreements from RAN2 #93 meeting [3]

1) NB-IoT supports a separate measurement threshold for intra-frequency and inter-frequency measurements (i.e. SIntraSearch and SnonIntraSearch).

2) The UE is not required to perform intra-frequency measurements when the serving cell is above the intra-frequency measurement threshold.

3) The UE is not required to perform inter-frequency measurements when the serving cell is above the inter-frequency measurement threshold.

4) Ranking based mechanism is supported for inter-frequency cell reselection for NB-IOT.

5) Inter-frequency mobility based on priority is not supported

In this contribution, we observed that it is not clear from current specification (running CR [4]) and give a Text Proposal relevant to measurement rules for cell reselection for NB-IoT to be captured in the TS 36.304.

Discussion

According to current 36.304 running CR [4], measurement rules for cell reselection for NB-IoT are specified in different subsection 5.2.4.2a instead of 5.2.4.2 (legacy LTE measurement rules for cell reselection) since NB-IoT only supports ranking based cell reselection without consideration on frequency priority. Triggering conditions to perform measurements are clearly specified, but inter-frequency carrier list to inform available/supported inter-frequencies needs to be specified in the part of measurement rules for inter-frequencies. As already agreed in RAN2 #92 meeting [1], it is possible to specify list of inter-frequencies supported in NB-IoT. We propose that UE should apply inter-frequency measurement rules only for the listed inter-frequencies indicated in system information.

Proposal: Confirm that UE should apply inter-frequency measurement rules only for the listed inter-frequencies indicated in system information.

Measurement rules for cell reselection for NB-IoT specified in current 36.304 running CR [4] are as follows:

#### 5.2.4.2 Measurement rules for cell re-selection For NB-IoT measurement rules for cell re-selection is defined in sub-clause 5.2.4.2.a. When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell. … #### 5.2.4.2a Measurement rules for cell re-selection for NB-IoT Editor’s note: It is FFS whether Squal is supported for NB-IoT Editor’s note: It is FFS whether redistributionInterFreqInfo is supported for NB-IoT When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell. Following rules are used by the UE to limit needed measurements: - If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements. - Otherwise, the UE shall perform intra-frequency measurements. - If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform inter-frequency measurements. - Otherwise, the UE shall perform inter-frequency measurements.

Text Proposal

------------------------------------------------------------Text Start ------------------------------------------------------------------------

#### 5.2.4.2	Measurement rules for cell re-selection

For NB-IoT measurement rules for cell re-selection is defined in sub-clause 5.2.4.2.a.

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to [10].

#### 5.2.4.2a	Measurement rules for cell re-selection for NB-IoT

Editor’s note: It is FFS whether Squal is supported for NB-IoT

Editor’s note: It is FFS whether redistributionInterFreqInfo is supported for NB-IoT

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for inter-frequencies which are indicated in system information:

-	For an inter-frequency, if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE may choose not to perform measurements of inter-frequencies.

-	Otherwise, the UE shall perform measurements of inter-frequencies.

------------------------------------------------------------Text End-------------------------------------------------------------------------

Conclusion

RAN2 is kindly asked to:

Proposal: Confirm that UE should apply inter-frequency measurement rules only for the listed inter-frequencies indicated in system information.

References

[1] R2-157014, “Report of the LTE breakout session (NB-IoT)”

[2] R2-161069, “Report of 3GPP TSG-RAN WG2 NB-IOT Ad-hoc Meeting”

[3] R2-161774, “Report of the LTE breakout session (NB-IoT)”

[4] R2-162071, “36.304 running CR to capture agreements on NB-IoT”