3GPP TSG-RAN WG2 Meeting #93bis	R2-162914

Dubrovnik, Croatia, 11th – 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | xxxx | rev | - | Current version: | Draft 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Preferred DRX Cycle Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  |  | Date: |  | 2016-04-1 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | In RAN2#92 meeting, a new C-DRX Cycle 60ms was introduced to Release 13 by R2-161967 in order to reduce UE VoLTE power consumption. Based on our analysis and simulations, the 60ms CDRX cycles can increase the end-to-end VoLTE delay by tens of milliseconds. So we believe that the C-DRX Cycle 60ms should be used mainly when the end-to-end VoLTE quality is good. However, the eNB may not be aware of the end-to-end VoLTE quality. Therefore, we propose that the UE can send C-DRX cycle preference indication to the eNB. Even without the 60ms CDRX, some UEs may experience long end-to-end Volte delay due to bad channel condition in the far-end UE, not due to the concerned UE’s CDRX setting or channel conditions. In this case, with our proposal, the UE can send C-DRX cycle preference indication with shorter CDRX cycles or “No CDRX” to the eNB. This can improve the VoLTE quality and reduce call drop rate KPI. More detailed Reasons are in the companion discussion paper R2-162912. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In UL-SCH data transfer, we add one procedure to enable the UE to report the preferred DRX cycle. - In MAC Control Element, we add one MAC control element, which is used to report the preferred DRX cycle. - In MAC Control Element subheader, we add one LCID to indicate the preferred DRX cycle reporting. - Impact analysis - Impacted functionality: - LTE UE Preferred DRX Cycle Reporting - Inter-operability: If eNB implements this CR and the UE does not, the UE will not report the capability of supporting the Preferred DRX Cycle reporting, hence the eNB will not configure this function. So there is no inter-operability issue. If UE implements this CR and the eNB does not, the UE will report the capability of supporting the Preferred DRX Cycle reporting, but the eNB will not configure this function. So there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot report preferred DRX cycle to the eNB. Some UEs with good VoLTE quality may suffer unnecessarily high power consumption in the VoLTE communication; whereas some UEs with bad VoLTE quality (due to core network delay or other reasons) may suffer unnecessarily bad VoLTE quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, 6.1.3, 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 36.306, TS 36.331 draft CRs |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.4	UL-SCH data transfer

[First Change]

### 5.4.x	Preferred DRX Cycle Reporting

The Preferred DRX Cycle reporting procedure is used to provide the serving eNB with information about the preferred DRX cycle. RRC controls Preferred DRX Cycle reporting by configuring the timer prohibitDrxPref-Timer [8].

A Preferred DRX Cycle (PDC) reporting shall be triggered if any of the following events occur:

-	upon configuration or reconfiguration of the preferred DRX cycle reporting functionality by upper layers [8], which is not used to disable the  function;

-	prohibitDrxPref-Timer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true in this TTI:

-	there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the preferred DRX cycle has changed since the last transmission of PDC when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

If the MAC entity has UL resources allocated for new transmission for this TTI the MAC entity shall:

-	if the Preferred DRX Cycle reporting procedure determines that at least one PDC has been triggered and not cancelled, and;

-	if the allocated UL resources can accommodate the MAC control element for PDC which the MAC entity is configured to transmit, plus its subheader, as a result of logical channel prioritization:

-	obtain the value of the preferred DRX cycle;

-	instruct the Multiplexing and Assembly procedure to generate and transmit a PDC MAC control element as defined in subclause 6.1.3.x based on the value of the preferred DRX cycle;

-	start of restart prohibitDrxPref-Timer;

-	cancel all triggered PDC(s).

[End of First Change]

### 6.1.3	MAC Control Elements

[Second Change]

#### 6.1.3.x	Preferred DRX Cycle Report MAC Control Element

The Preferred DRX Cycle Report MAC control element is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-2. It has a fixed size and consists of a single octet defined as follows (figure 6.1.3.x-1):

-	R: reserved bit, set to "0";

-	Preferred DRX Cycle (PDC): this field indicates the preferred DRX cycle. The length of the field is 2 bits. The reported PDC and the corresponding preferred DRX cycle are shown in Table 6.1.3.x-1 below. (Value sf20 corresponds to 20 subframes, sf40 corresponds to 40 subframes, and so on.)

Figure 6.1.3.x-1: PDC MAC control element

Table 6.1.3.x-1: Preferred DRX Cycle for PDC

| PDC | Preferred DRX Cycle |
| --- | --- |
| 0 | No C-DRX |
| 1 | sf20 |
| 2 | sf40 |
| 3 | sf60 |

[End of Second Change]

### 6.2.1	MAC header for DL-SCH, UL-SCH and MCH

[Third Change]

The MAC header is of variable size and consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element or padding as described in tables 6.2.1-1, 6.2.1-2 and 6.2.1-4 for the DL-SCH, UL-SCH and MCH respectively. There is one LCID field for each MAC SDU, MAC control element or padding included in the MAC PDU. In addition to that, one or two additional LCID fields are included in the MAC PDU, when single-byte or two-byte padding is required but cannot be achieved by padding at the end of the MAC PDU. A UE of Category 0 [12] shall indicate CCCH using LCID "01011", otherwise the UE shall indicate CCCH using LCID "00000". The LCID field size is 5 bits;

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field and F2 field;

-	F: The Format field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F field per MAC PDU subheader except for the last subheader and subheaders corresponding to fixed-sized MAC control elements and except for when F2 is set to 1. The size of the F field is 1 bit. If the F field is included; if the size of the MAC SDU or variable-sized MAC control element is less than 128 bytes, the value of the F field is set to 0, otherwise it is set to 1;

-	F2: The Format2 field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F2 field per MAC PDU subheader. The size of the F2 field is 1 bit. If the size of the MAC SDU or variable-sized MAC control element is larger than 32767 bytes, and if the corresponding subheader is not the last subheader, the value of the F2 field is set to 1, otherwise it is set to 0.

-	E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate another set of at least R/F2/E/LCID fields. The E field is set to "0" to indicate that either a MAC SDU, a MAC control element or padding starts at the next byte;

-	R: Reserved bit, set to "0".

The MAC header and subheaders are octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011-10111 | Reserved |
| 11000 | Activation/Deactivation (4 octets) |
| 11001 | SC-MCCH, SC-MTCH (see note) |
| 11010 | Long DRX Command |
| 11011 | Activation/Deactivation (1 octet) |
| 11100 | UE Contention Resolution Identity |
| 11101 | Timing Advance Command |
| 11110 | DRX Command |
| 11111 | Padding |
| NOTE: Both SC-MCCH and SC-MTCH cannot be multiplexed with other logical channels in the same MAC PDU except for Padding |  |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 00000 | CCCH |
| 00001-01010 | Identity of the logical channel |
| 01011 | CCCH |
| 01100-10100 | Reserved |
| 10101 | Preferred DRX Cycle Report |
| 10110 | Truncated Sidelink BSR |
| 10111 | Sidelink BSR |
| 11000 | Dual Connectivity Power Headroom Report |
| 11001 | Extended Power Headroom Report |
| 11010 | Power Headroom Report |
| 11011 | C-RNTI |
| 11100 | Truncated BSR |
| 11101 | Short BSR |
| 11110 | Long BSR |
| 11111 | Padding |

Table 6.2.1-3 Values of F and F2 fields:

| Index of F2 | Index of F | Size of Length field (in bits) |
| --- | --- | --- |
| 0 | 0 | 7 |
|  | 1 | 15 |
| 1 | - | 16 |

Table 6.2.1-4 Values of LCID for MCH

| Index | LCID values |
| --- | --- |
| 00000 | MCCH (see note) |
| 00001-11100 | MTCH |
| 11101 | Reserved |
| 11110 | MCH Scheduling Information or Extended MCH Scheduling Information |
| 11111 | Padding |
| NOTE: If there is no MCCH on MCH, an MTCH could use this value. |  |

[End of Third Change]