3GPP TSG-RAN2 Meeting 93BIS 	R2-163100

# Dubrovnik, Croatia, 11th – 15th April 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.323 | CR | CRNum | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of LWA status report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_WLAN_radio-Core |  |  |  |  |  |  | Date: |  | 2016-04-11 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Clarify how the HRW field in LWA status report is set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The current text does not describe which PDCP PDUs received over WLAN are considered for setting the HRW field in LWA status report. The HRW field should be determined by those PDCP PDUs received over WLAN that are stored for reordering and not discarded. Impact analysis Impacted functionality: LTE-WLAN aggregation Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous description in specification text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### *********First change**********

### 5.10.2	LWA status report

When LWA status report is triggered, the UE shall:

-	compile a status report as indicated below, and submit it to lower layers as the first PDCP PDU for the transmission, by:

-	setting the FMS field to the PDCP SN of the first missing PDCP SDU;

-	setting the HRW field to the PDCP SN of the PDCP SDU received on WLAN with highest PDCP COUNT value that has not been discarded or to FMS if no such PDCP SDUs have been received on WLAN;

-	setting the NMP field to the number of missing PDCP SNs.