3GPP TSG-RAN WG2 Meeting #94						R2-163454

Nanjing, China, 23th – 27th May 2016

Agenda Item:		7.4.1

Souce:		Samsung

Title:			Dedicated Signalling of SI in Rel-13 eMTC

Document for:	Discussion

# 1. Introduction

For SI update, RAN2 has the following agreement:

The UE is not required to detect SIB change while being in RRC_CONNECTED. The NW may release the UE to IDLE if it wants the UE to acquire changed SIB or provide the updated SIB by dedicated signalling

However, RAN2 has not discussed the details on the dedicated signalling. For now, it is found that the current dedicated RRC signalling is insufficient to provide the System Information required for Rel-13 eMTC UE.

This paper will discuss how to provide the deidicated signalling for SIB update, and investigate whether the additional dedicated signalling is required upon Handover.

# 2. Discussion

Firstly,

Proposal 1: For SI update, it is up to NW implementation if NW releases the UE to IDLE or provide the updated SIB by dedicated signalling if it wants the UE to acquire the changed SIB.

We review the options for transferring SI by dedicated signalling that are currently available in the specification:

- Upon handover, the source cell can transfer the most essential SI parameters of the target cell that are required in connected by means of MobilityControlInfo.

- For the PCell, it is furthermore possible to transfer SIB1 by means of systemInfomationBlockType1Dedicated

- For SCells, it is possible to transfer the SI parameters required in connected by means of RadioResourceConfigCommonSCell.

For eMTC, it should be possible to re-use options a) and b). In the following we will assess whether this covers all required eMTC information, distinguishing the case of SI update and handover. Some remarks:

- For eMTC parameters that hardly ever change, there is no need to support update by dedicated signalling (i.e. moving UEs to idle should be sufficient)

- It would be preferable to only introduce dedicated signalling for the parameters that require it (i.e. same principle as used for MCI and SCells). However, as ASN.1 is already frozen, ASN.1 impacts should be minimized. Hence it is proposed that we should only introduce new dedicated signalling for important parameters not covered by the current signalling.

The table below provides an overview of the different System Information (SI) fields, including remarks whether dedicated signalling may be required. Basically, we can exclude the fields being specific for idle mode, accessibility or SI scheduling from the dedicated signalling.

|  | Parameters | For SI update |  | For handover |
| --- | --- | --- | --- | --- |
| MIB | dl-Bandwidth | Not needed because hardly changed in the cell |  | Needed. But eMTC UE acquires MIB broadcast in the target cell (see Section 5.3.5.4 in TS36.331) |
|  | Phich-Config | Not needed because PHICH is not supported in eMTC |  |  |
|  | systemFrameNumber | Not needed because the offset of SFN is hardly changed in the cell |  |  |
|  | schedulingInfoSIB1-BR-r13 | Not needed because it’s a SIB1 scheduling information required to obtain SIB1-BR in IDLE. In CONN, SIB1-BR can be transferred via the field, systemInfomationBlockType1Dedicated |  |  |
| SIB1 | cellAccessRelatedInfo | Not needed because hardly changed in the cell | Needed in the target PCell |  |
|  | p-Max | Needed |  |  |
|  | freqBandIndicator freqBandIndicator-v9e0 | Not needed because hardly changed in the cell |  |  |
|  | multiBandInfoList-v9e0 multiBandInfoList-v10j0 | Not needed because hardly changed in the cell |  |  |
|  | tdd-Config tdd-Config-v1130 | Needed |  |  |
|  | systemInfoValueTag | Needed |  |  |
|  | freqBandInfo-r10 | Not needed because hardly changed in the cell |  |  |
|  | category0Allowed-r12 | Not needed because hardly changed in the cell |  |  |
|  | freqBandIndicatorPriority-r12 | Not needed because hardly changed in the cell |  |  |
|  | hyperSFN-r13 | Not needed because the offset of hyperSFN is hardly changed in the cell |  |  |
|  | eDRX-Allowed-r13 | Not needed because hardly changed in the cell |  |  |
|  | fdd-DownlinkOrTddSubframeBitmapLC-r13 | Needed |  |  |
|  | fdd-UplinkSubframeBitmapLC-r13 | Needed |  |  |
|  | startSymbolLC-r13 | Needed |  |  |
|  | si-ValidityTime-r13 | Needed |  |  |
|  | systemInfoValueTagList-r13 | Needed |  |  |
|  | cellSelectionInfo cellSelectionInfo-v920 cellSelectionInfo-v1130 cellSelectionInfo-v1250 | Not needed because it’s for IDLE operation |  |  |
|  | schedulingInfoList | Not needed because it’s the SI scheduling information |  |  |
|  | si-WindowLength | Not needed because it’s the SI scheduling information |  |  |
|  | si-HoppingConfigCommon-r13 | Not needed because it’s the SI hopping information |  |  |
| SIB2 | radioResourceConfigCommon | Needed. For now, no dedicated signalling |  | Needed. But he parameter is already provided with MobilityControlInfo |
|  | mbsfn-SubframeConfigList | Needed. For now, no dedicated signalling |  | Needed. For now, no dedicated signalling |
|  | freqInfo | Not needed because hardly changed in the cell |  | Not needed because UL carrier frequency of the target PCell is provided with carrierFreq of MobilityControlInfo. It seems sufficient |
|  | ue-TimersAndConstants | Not needed. The IE includes the configuration on T300, T301, T310, n310, T311 and n311. The IE is not needed upon HO. And, it can be provided with RLF-TimersAndConstants of radioResourceConfigDedicated |  |  |
|  | voiceServiceCauseIndication | Not needed because it is for new cause value upon connection establishment. |  |  |
|  | ac-BarringInfo ssac-BarringForMMTEL-Voice-r9 ssac-BarringForMMTEL-Video-r9 ac-BarringForCSsFB-r10 ac-BarringSkipForMMTELVoice-r12 ac-BarringSkipForMMTELVideo-r12 ac-BarringSkipForSMS-r12 ac-BarringPerPLMN-List-r12 voiceServiceCauseIndication-r12 acdc-BarringForCommon-r13 acdc-BarringPerPLMN-List-r13 | Not needed because it’s SI related to accessibility in IDLE |  |  |

We need not define new dedicated signalling for MIB and SIB1. Since the MIB contents are hardly changed or not useful for SI update, we can exclude it from the dedicating signalling. Furthermore, upon HO, MTC UE has to acquire MIB in the target PCell. We can also provide SIB1 contents on the dedicated signalling, i.e. systemInfomationBlockType1Dedicated. The current signalling is sufficient. We could optimize SIB1 parameters to be transferred via the dedicated signalling for eMTC (some parameters seems hardly changed even though it could be somewhat subjective), but it seems negative because ASN.1 impacts should be minimized. One issue is whether SIB1-BR can be also provided with the current systemInfomationBlockType1Dedicated. But RAN2 agreed that the anything applicable for a particular SIB or SI message equally applies to the corresponding BR version unless explicitly stated otherwise (see Section 5.2.1.1 of TS36.331). Hence it is assumed that SIB1-BR can be also transferred with systemInfomationBlockType1Dedicated, i.e. systemInfomationBlockType1Dedicated transferred over the narrowband always includes SIB1-BR instead of SIB1. If needed, we can clearly indicate it in the field description.

Observation 1: We need not define new dedicated signalling for MIB and SIB1 contents.

The remaining issue is how to provide SIB2 contents on the dedicated signalling. It is still a valid approach to minimize ASN.1 impact.

Upon handover, the IE mobilityControlInfo can transfer the most essential information of SIB2. Since the mbsfn-SubframeConfigList is useful, the mobilityConrtolInfo can include it into the mobilityControlInfo in addition. As explained in detail, MBMS is not supported in eMTC. But, still mbsfn-SubframeConfigList needs to be provided to MTC UE to exclude the MBSFN subframes in the decoding. In current spec, eNB can provide fdd-DownlinkOrTddSubframeBitmapLC via SIB1. The field indicates the DL available subframes to MTC UE while considering also MBSFN subframes. Therefore, if the field is provided, MTC UE need not know mbsfn-SubframeConfigList. The problem is that the field is optional. If the field is absent, MTC UE has to still know mbsfn-SubframeConfigList.

For SI update, there is no dedicated signalling to deliver SIB2 contents. One option is to reuse the mobilityControlInfo even for SI update. However, since the mobilityControlInfo results in a random access, it seems unsuitable. Accordingly, we can define a new IE radioResourceConfigCommonMTC, to carry some essential SIB2 contents upon SI change. The IE would include radioResourceConfigCommon possibly including mbsfn-SubframeConfigList. Alternatively, the new IE can be defined by typecasting it as the IE mobilityControlInfo to avoid the random access, i.e. radioResourceConfigCommonMTC-r13	OCTET STRING (CONTAINING MobilityControlInfo).

Consequently, MTC UE can acquire the updated SI or the SI required in target PCell via RRCConnectionReconfiguration including:

| SI update | Handover |
| --- | --- |
| systemInfomationBlockType1Dedicated + new IE radioResourceConfigCommonMTC | systemInfomationBlockType1Dedicated + mobilityControlInfo |

Where, the existing IE mobilityControlInfo includes in addition:

- mbsfn-SubframeConfigList

also, the new IE radioResourceConfigCommonMTC includes:

- radioResourceConfigCommon

- mbsfn-SubframeConfigList

or

- radioResourceConfigCommonMTC-r13	OCTET STRING (CONTAINING MobilityControlInfo)

A corresponding draft CR is provided in R2-163455.

Proposal 2: The IE mobilityControlInfo includes mbsfn-SubframeConfigList.

Proposal 3: A new IE radioResourceConfigCommonMTC is definded while including at least radioResourceConfigCommon and mbsfn-SubframeConfigList.

# 3. Conclusion

It is proposed that

Proposal 1: For SI update, it is up to NW implementation if NW releases the UE to IDLE or provide the updated SIB by dedicated signalling if it wants the UE to acquire the changed SIB.

Proposal 2: The IE mobilityControlInfo includes mbsfn-SubframeConfigList.

Proposal 3: A new IE radioResourceConfigCommonMTC is definded while including at least radioResourceConfigCommon and mbsfn-SubframeConfigList.

# 4. References

- [1] TS36.331	Radio Resource Control (RRC); Protocol specification (Release 13)