3GPP TSG-RAN WG2 #94	Tdoc R2-163781

Nanjing, China, 23–27 May 2016

Agenda Item:	8.8.1

Source:	Ericsson

Title:	Acknowledgements for SPS commands

Document for:	Discussion, Decision

# Introduction

A new work item for L2 latency reduction techniques has been approved by RAN plenary [2]. With the intended technique of short SPS periods and skipping of padding transmissions, faster UL access is achieved, while UE power consumption and UL interference is kept low.

In RAN2#93bis, feedback for deactivation commands was agreed whereas an acknowledgement for SPS activation was left open. In this contribution we discuss in particular solutions for deactivation acknowledgements as well as the need for acknowledgements for SPS activation commands.

# Discussion

In current SPS, as response to the PDCCH grant activating SPS, the UE transmits on the first SPS resource either with a regular data transmission, or in case of empty UE buffer, with a new MAC PDU containing zero MAC SDUs. This way, the activation of SPS is acknowledged to the eNB. As result to the PDCCH grant releasing SPS, the UE stops all transmissions on the SPS resources, and by the absence of transmissions, the eNB is implicitly informed of successful SPS release.

For use of SPS with skipped padding transmissions, the need and different means for acknowledgement of activation and release have been discussed in [1].

## Acknowledgement for SPS deactivation

In legacy SPS, the PDCCH grant indicating SPS release is not explicitly acknowledged. In legacy, invalid values in the DCI are used to indicate SPS release. After receiving this command, the UE immediately releases the SPS resources. Subsequent transmission on the resources is not possible anymore.

Different alternative solutions for acknowledgment of SPS release have been proposed:

- Solution based on the transmission of a MAC CE in the first SPS location after release.

- Solution based on DL grant given as a result of a SR on which the transmission of a triggered MAC CE is sent.

- Solution based on single PUSCH padding transmission delaying SPS release with one SPS occasion.

Here, we believe that solutions not based on padding transmissions are overly complex. For example, triggering a SR for transmitting a MAC CE or transmitting a MAC CE in a next SPS occasion would only seem necessary if additional actions based on this information would be expected from the eNB that released the SPS resource. As no further actions based on this information is expected, we think there is no need for defining new MAC CEs as in solution 1 and 2. Furthermore, triggering an SR requires waiting for the next SR opportunity, which can be several TTIs into the future whereas using the next SPS occasion can be just a few subframes ahead.

With solution 3, in case of SPS with skipped padding transmissions, the SPS resources can be released just one SPS interval later, and a padding transmission on the last SPS resources is issued. This means that padding is not skipped on the last SPS resource as a result of a deactivation command, but a padding transmission is mandated. A padding transmission should be mandated as deactivation ack – instead of just skipping padding – so that the eNB can uniquely identify the padding as deactivation ack. Allowing data transmission as deactivation ack is however ambiguous, since data transmissions may happen also independent of reception of the deactivation grant. For this last transmission, the DCI received in the SPS activation grant is used (as in all other transmissions on SPS resources) and the DCI with the release-format (and it’s invalid values) from the SPS release is ignored. Note that this solution is closest to legacy behavior. Furthermore, this is also aligned with a solution of acknowledging SPS activation with a padding transmission in the first SPS occasion after activation.

- Upon SPS release for SPS with skipped padding, UE delays the release of SPS resources by one period, and sends padding on the last SPS resource.

The eNB, upon receiving the last PUSCH transmission containing padding only, considers the SPS resource as successfully released and can schedule the resources to other UEs. If errors are detected, the eNB resends the explicit release command to the UE. Assuming here also a minimum delay of 4TTI (or higher, if SPS period is higher) to the PUSCH transmission after receiving the SPS release, the eNB is informed of the SPS release after minimum 5TTI.

Remaining or new data in UE buffer would trigger resource request procedures as legacy.

## Acknowledgement for SPS activation

### Cost of missed activation

If a UE misses the SPS activation command it will not have a readily available UL resource. Therefore, when data arrives to the UE buffer it will initiate a SR process. Assuming that the eNB sends a grant 4 TTI after the SR is transmitted transmission will be delayed on average 8.5ms if the SR period is 1ms. If the SR period is 5 or 10ms the delay will on average be 10.5ms or 13ms, respectively.

Observation 1	A missed SPS activation will delay the UL transmission with 8.5ms or more on average.

Especially for delay sensitive use cases, such as Critical Machine Type Communications (C-MTC) such a delay will be disastrous. At least for such critical scenarios it should be possible to configure the system so that a missed activation command is detected by the eNB and resent, in order to always have an active UL grant. If not, a polling mechanism would have to be introduced where the critical UE asks for SPS activation as long as it is needed. This mechanism would have bigger specification impact than an acknowledgement for the SPS activation.

Observation 2	Critical applications need a readily available grant for reliable operation. If SPS activation acknowledgement is not introduced a polling mechanism may be required instead.

Furthermore, by granting SPS resources, the eNB reserves the resources exclusively for this UE and cannot schedule other UEs on these resources. In case the SPS resource grant is not acknowledged to the eNB, but the granting of resources was unsuccessful, the resources are wasted for all users.

### Solution for acknowledgement

In legacy SPS, the acknowledgment of the SPS activation is given by the first PUSCH transmission on the activated SPS resources. It is straightforward to reuse this method also in the solution of SPS with skipped padding transmissions; to specify an exception of skipping padding for the first transmission on SPS resources after receiving the SPS activation grant, i.e. to apply legacy behavior in this case.

UE does not skip padding transmission on the first SPS resource after receiving the SPS activation grant.

We believe that this is the simplest solution, as can also be seen from a specification impact perspective. A draft CR can be found in [3]. Alternative solutions based on e.g. ACK on PUCCH seem overly complex here, as they would have also considerable RAN1 impact.

Also, since the main motivation of SPS in general is to decrease PDCCH load, we believe that the SPS activation is not triggered very frequently and thus the additional overhead and UL interference from this acknowledgment is negligible. With this in mind, both repetition of the PDCCH grant activating the resource and using a more robust PDCCH resource allocation would increase the PDCCH load and resource utilization. Furthermore, one can assume that as SPS functionality become more efficient, the need for very frequent SR opportunity occasions is smaller, thus making the recovery of missed activation detection rather slow.

When it comes to the timing of SPS activation, it is assumed that the first SPS resource can be used within the same time as a dynamic uplink grant becomes usable (4TTI). This way, the first PUSCH (data or padding) acknowledging the SPS grant can be received in the eNB within 5TTI. From this time on the eNB can be sure that SPS has been successfully activated. Otherwise the eNB may retransmit the PDCCH activating the SPS grant.

The UE receiving the PDCCH activation command in SF n considers the first SPS grant available in SF n+4.

## SPS timed release

For the case of SPS with a skipped padding configuration RAN2 decided that implicit release is not supported. However, there may be situations where the eNB would like to be able to explicitly configure a finite duration of the validity of an active SPS configuration. I.e., by defining that a release is done after a number of configurable subframes/time. The use of explicit deactivation commands and subsequent UL signalling as a result of those can then be avoided.

This timed release could be used for example when a pre-scheduling function in the eNB is not exactly sure when data arrives in the UL buffer, in this case the eNB could provide the SPS-grant to the UE that lasts for some subframes. The UE can then use the grant immediately when the data is available.

The UE may be RRC configured with a time duration for which an activated SPS resource is valid.

# Conclusion

Based on the discussion in the previous section we make the following proposals:

- Upon SPS release for SPS with skipped padding, UE delays the release of SPS resources by one period, and does not skip padding on the last SPS resource.

UE does not skip padding transmission on the first SPS resource after receiving the SPS activation grant.

The UE receiving the PDCCH activation command in SF n considers the first SPS grant available in SF n+4

The UE may be RRC configured with a time duration for which an activated SPS resource is valid.

# References

TR 36.881

RP-160667, New WI proposal: L2 latency reduction techniques for LTE

R2-163943: 36321_draftCR_(Rel-14) L2 Latency reduction techniques, RAN2#94, May 2016

R2-163793, SPS Interval Alignment, RAN2#94, May 2016