3GPP TSG-RAN WG2 #94	Tdoc R2-163785

Nanjing, China, 23–27 May 2016

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Non-adaptive retransmissions for SPS with skip padding and short periods

Document for:	Discussion, Decision

# Introduction

The work on the L2 latency reduction techniques [2] started in RAN2#93bis. With the intended technique of short SPS periods and skipping of padding transmissions, faster UL access is achieved, while UE power consumption and UL interference is kept low. The following agreements have been reached in RAN2#93bis:

Agreements

- FFS what SPS periodicity values other than 1ms should be introduced

- For retransmission colliding with SPS resources we will not only rely on adaptive retransmissions.   FFS if the UE can retransmit on the SPS resources if no new data is available.

- For TDD, if SPS configuration is below 10ms the existing text in the spec “the UE shall round this parameter down to the nearest integer (of 10 sub-frames)” doesn’t apply.

- We will not consider RAN1 specific solutions.  If companies agrees that there is an error case with the DTX detection probability we can considers RAN2 based solutions

- Feedback for SPS deactivation will be introduced.  FFS whether we have feedback for SPS activation.

- RAN2 assumption is that the UE will send the BSR/PHR according to legacy trigger (e.g. we will not specify skipping of BSR/PHR when no data is available in the buffer).

- Padding BSR should not be transmitted.  FFS if anything needs to be specified.

- If the UE is configured with SPS and UL skipping, then implicit release is not supported.  How this is captured in the specs is FFS

In this contribution we discuss retransmission handling on SPS with skip padding and short periods, non-adaptive and adaptive retransmissions, as well as potential error cases.

# Discussion

In RAN2#93bis, retransmission handling for SPS resources has been discussed. It was agreed that for retransmissions colliding with SPS resources, we will not only rely on adaptive retransmissions, and that it is FFS if the UE can retransmit on the SPS resources if no new data is available.

Currently, according to the current HARQ entity procedure in the MAC specification, if there is an uplink SPS grant configured, the UE cannot perform a non-adaptive retransmission on this resource. In case of configuring the UE with a 1sf (subframe) interval for the SPS configured grant interval, retransmissions thus always collide with a granted resource.

Example:

- A legacy SPS interval is configured to 10sf. As a result of NACK on PHICH at n, the UE checks in n+8 if an UL grant has been indicated for this TTI. As there is no grant in this subframe (assuming no PDCCH received, and since this is no SPS granted occasion), the UE can perform a non-adaptive retransmission, as the HARQ buffer is not empty.

- SPS interval of 8sf is configured. As a result of NACK on PHICH, the HARQ buffer is non-empty, but as the subframe n+8 has a grant (SPS granted occasion) a new transmission has to be triggered overriding the current HARQ buffer, i.e. discarding the previous, actually NACKed data.

With a 1sf SPS interval configuration (or less or equal 8sf), consequently, even if the UE finds a NACK on PHICH the UE is mandated to use the process for new data unless the eNB sends also an adaptive retransmission grant.

When using short SPS intervals, the eNB can schedule retransmissions only by triggering adaptive retransmissions with PDCCH due to that non-adaptive retransmissions may be colliding with granted resources.

With the support of skipped padding, the eNB may however not be able to distinguish between missed UL transmissions and skipped transmissions (i.e. DTX) due to empty UE buffer as the absence of UL transmissions needs to be differentiated from decoding errors.

When skipped padding is configured with short SPS intervals, the eNB cannot distinguish between skipped transmissions (DTX) and decoding error, consequently the eNB may not able to reliably avoid scheduling adaptive retransmissions.

Consequently, depending on the eNBs DTX capability and the DTX design target, redundant adaptive retransmissions would be scheduled by the eNB to stay on the save side and avoid data loss on HARQ level. This increases PDCCH load.

In the following we discuss further details of allowing non-adaptive retransmissions on configured SPS resources, and relying only on adaptive retransmissions.

## Allowing non-adaptive retransmissions on configured SPS grants

In this section we evaluate what is required to introduce non-adaptive retransmissions also in case of configured SPS grants.

Whether a non-adaptive retransmission is required is indicated on PHICH. If the eNB did not detect any transmission, or fails to decode a transmission, NACK is indicated; otherwise ACK is indicated. Since UE capability to detect PHICH DTX cannot be always assumed, the eNB should always send NACK in case it did not detect or decode, otherwise the UE may falsely interpret received noise on PHICH as ACK. It is noteworthy that PHICH resources are allocated anymore for the UE, thus no additional overhead is introduced when the eNB always transmits NACK.

When the UE receives the feedback on PHICH for an SPS resource, two cases have to be distinguished when skip padding is enabled:

- If the UE has no new uplink data, i.e. the SPS resource is unused and can thus be used for non-adaptive retransmissions.

  - If ACK is received, the UE does not transmit, but keeps previous data in the HARQ buffer

  - If NACK is received, the UE does a non-adaptive retransmission of previous data.

- If UE has new uplink data, i.e. both new transmission and non-adaptive retransmission collide since both could be send over the SPS resource

  - If ACK is received, the UE triggers the new transmission, overriding the current HARQ buffer

  - If NACK is received, the UE does a non-adaptive retransmission of previous data.

One detail needs further attention in such a scheme. When skip padding is enabled on SPS resources, the eNB does not know when the UE attempted the initial transmission. And thus the eNB is not able count and identify the UE’s redundancy version used in the non-adaptive retransmission. To solve this issue, and to not mandate the eNB to need to test and decode all possible redundancy versions, we propose that when skip padding is enabled, always redundancy version RV0 is used for non-adaptive retransmissions. If a higher redundancy version is needed, the eNB can still fall back to scheduling an adaptive retransmission.

Looking at potential error cases where data can get lost, with the scheme outlined above, HARQ data loss, requiring a later RLC retransmission, can only happen in case of NACK-to-ACK PHICH errors (<10^-3), and in case new data is available for transmission. That means the robustness of this solution depends on PHICH robustness, as the legacy operation. Further looking into potential issues: new transmissions could be unnecessarily delayed, due to non-adaptive retransmissions being prioritized. This could happen in case of ACK-to-NACK PHICH errors. The chances for these errors are however the same as in legacy operation, and require no further optimizations.

We implemented the non-adaptive retransmission scheme in our draft MAC CR [3]. Thereby, following UE states need to be handled differently. To provide an overview, they are listed in Table 1 below:

Table 1: UE states and UE actions for SPS with skipped padding and non-adaptive retransmissions.

| HARQ buffer of current process | PHICH reception | New data available? | UE ACTION |
| --- | --- | --- | --- |
| Empty | ACK | Yes | Transmit new data |
| Empty | ACK | No | Do nothing |
| Empty | NACK | Yes | Transmit new data |
| Empty | NACK | No | Do nothing |
| Full | ACK | Yes | Transmit new data (HARQ buffer override) |
| Full | ACK | No | Do nothing (keep HARQ buffer) |
| Full | NACK | Yes | Non-adaptive retransmission* (prioritized over new data) |
| Full | NACK | No | Non-adaptive retransmission** |

*   as per agreement “for retransmission colliding with SPS resources we will not rely on adaptive retransmission”
** solving “FFS if the UE can retransmit on the SPS resources if no new data is available”

We believe a hybrid solution, where we on the one hand prioritize non-adaptive retransmission over new data, but on the other hand when no new data is available do not allow non-adaptive retransmission is not useful. In this case an adaptive retransmission would have to be scheduled, but since the eNB is not aware of whether new data is available in the UE, the eNB could/would not schedule it. Alternatively, if the eNB always schedules adaptive retransmissions, no new data transmission could take place. Therefore we propose to fully enable non-adaptive retransmissions as in Table 1.

## Relying on PDCCH scheduled adaptive retransmissions for SPS

Without introducing the non-adaptive retransmission scheme, the UE would always send new transmissions on SPS granted resources, unless the eNB schedules an adaptive retransmissions via PDCCH (to SPS CRNTI + NDI =1). In this case the adaptive retransmission based on the current HARQ buffer is done, which postpones transmission of new data.

This means that HARQ data loss (triggering a later RLC retransmission) can occur when no PDCCH with adaptive retransmission grant was received by UE or not even send by eNB. Therefore, the eNB needs to send the PDCCH always when it assumes that an uplink transmission has been lost. The efficiency to distinguish between skipped uplink transmissions and not decode able uplink transmission, and thus the efficiency of not wasting too many PDCCH resources, depends on the eNBs DTX detection capability.

Evaluating potential error cases for this scheme, data can get lost on HARQ in the following cases:

- eNB detects DTX wrongly (false positive), even though the UE make a transmission attempt, so eNB does not schedule an adaptive retransmission.

- eNB cannot decode (PUSCH error ~10%), schedules an adaptive retransmission, but UE cannot decode PDCCH (1%) and has new data available at the retransmission time.

Looking at these error probabilities, it becomes obvious that they are higher than in the non-adaptive retransmission case, i.e. PDCCH error probability higher than PHICH error probability. Making the PDCCH more reliable would come additionally with higher overhead, which in case of continuous PDCCH transmission for retransmission scheduling is significant.

Attention has to be paid to that the HARQ buffer of the UE can actually be empty, if no transmission was done before, but the eNB falsely scheduled an adaptive retransmission for that HARQ process. We believe that in this case it makes sense that the UE sends new data or padding. Consequently, when receiving this new data, the eNB would stop scheduling further retransmissions for that HARQ process.

## Way forward

We analysed both introducing non-adaptive retransmissions on SPS granted resources, as well as relying only on adaptive retransmissions for SPS above. It became obvious that both are valid options to handle retransmissions in SPS, but also that allowing non-adaptive retransmissions provides a higher reliability against losing data on HARQ and requiring an RLC retransmission. While non-adaptive retransmissions come with a somewhat higher implementation complexity, it appears a natural solution, as the scheme is very similar to non-adaptive retransmission handling of consecutive dynamic grants. The adaptive retransmission solution on the other hand is somewhat easier to implement in the standard, but requires additional PDCCH resources. In case DTX detection by the eNB cannot always be assumed, the introduced PDCCH load can be significant.

In conclusion, we propose to use non-adaptive retransmissions:

If the UE has no new UL data available, allow non-adaptive retransmissions on SPS resources based on ACK/NACK on PHICH.

If the UE has new UL data available, allow and prioritize non-adaptive retransmissions on SPS resources in case NACK is received. In case of ACK, transmit new data.

Non-adaptive retransmissions are done based on RV0, since the eNB does not know when the UE did the initial transmission when skip-padding is configured.

Allowing non-adaptive retransmissions should however not prevent scheduling adaptive retransmissions. Adaptive retransmissions are for example useful to increase the redundancy version. Therefore, adaptive retransmissions should be prioritized over non-adaptive retransmissions. In case initial transmissions had been skipped before and the HARQ buffer is still empty, a new data transmission can be carried out when the adaptive retransmission is scheduled.

Adaptive retransmissions are prioritized over non-adaptive retransmissions, and are not skipped.

A corresponding MAC CR is provided in [3].

# Conclusion

Based on the discussion in the previous section we make the following proposals:

Proposal 1	If the UE has no new UL data available, allow non-adaptive retransmissions on SPS resources based on ACK/NACK on PHICH.

Proposal 2	If the UE has new UL data available, allow and prioritize non-adaptive retransmissions on SPS resources in case NACK is received. In case of ACK, transmit new data.

Proposal 3	Non-adaptive retransmissions are done based on RV0, since the eNB does not know when the UE did the initial transmission when skip-padding is configured.

Proposal 4	Adaptive retransmissions are prioritized over non-adaptive retransmissions, and are not skipped.

# References

3GPP TR 36.881, Study on latency reduction techniques for LTE (Release 13)

RP-160667, New WI proposal: L2 latency reduction techniques for LTE

R2-163943, L2 Latency reduction techniques (TS 36.321 draft CR), RAN2#94, May 2016