3GPP TSG-RAN WG2 #94	Tdoc R2-163792

Nanjing, China, 23–27 May 2016

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Periodic BSR in SPS with skip padding

Document for:	Discussion, Decision

# Introduction

The work on the L2 latency reduction techniques [2] started in RAN2#93bis. With the intended technique of short SPS periods and skipping of padding transmissions, faster UL access is achieved, while UE power consumption and UL interference is kept low. The following agreements have been reached in RAN2#93bis:

Agreements

- FFS what SPS periodicity values other than 1ms should be introduced

- For retransmission colliding with SPS resources we will not only rely on adaptive retransmissions.   FFS if the UE can retransmit on the SPS resources if no new data is available.

- For TDD, if SPS configuration is below 10ms the existing text in the spec “the UE shall round this parameter down to the nearest integer (of 10 sub-frames)” doesn’t apply.

- We will not consider RAN1 specific solutions.  If companies agrees that there is an error case with the DTX detection probability we can considers RAN2 based solutions

- Feedback for SPS deactivation will be introduced.  FFS whether we have feedback for SPS activation.

- RAN2 assumption is that the UE will send the BSR/PHR according to legacy trigger (e.g. we will not specify skipping of BSR/PHR when no data is available in the buffer).

- Padding BSR should not be transmitted.  FFS if anything needs to be specified.

- If the UE is configured with SPS and UL skipping, then implicit release is not supported.  How this is captured in the specs is FFS

In this contribution we discuss handling of PeriodicBSR in SPS with skip padding and short periods.

# Discussion

In RAN2#93bis, retransmission handling for SPS resources has been discussed. It was assumed by RAN2 that for periodic BSR and PHR the legacy behaviour applies, i.e. a UE configured with skip padding will continue to transmit a periodic BSR and PHR even if the UE buffer is empty and the configured grant would be “unused” due to skipping.

Currently, according to the current MAC specification; if there is an uplink SPS grant configured, the UE may periodically report a periodic BSR by a RRC configured timer, periodicBSR-Timer. This consequently entails that a Buffer Status Report (BSR) shall be triggered if the periodicBSR-Timer expires.

In defining skipped padding, this results in that when a periodic BSR is triggered, the BSR transmitted in a TTI reflect a zero buffer status for this TTI. As a consequence, it can be expected that the periodicBSR-Timer would be set to a higher value, or infinity, in order to minimize possible frequent transmission of padding in the form of a Periodic BSR indicating zero buffer content. This can be expected as a result of frequent use of a more efficient SPS grant utilization due to the skipped padding and the decreased latency by prescheduling.

In legacy systems, the periodic BSR is commonly used to have up-to-date knowledge about the UE's buffer for the scheduler. Thus, if an eNB sees that an UE transmitting has a lot of data in its buffers, the eNB may provide larger grants to the UE. Note that this is the only way for the eNB to have an up-to-date view of the UE's buffers as the regular BSR is not triggered when an already existing buffers keeps building up and no higher priority logical channel is queued for transmission. In order for this mechanism to be effective, the periodicity of the periodic BSR may be short.

A periodic BSR triggered when the UE transmit buffer is empty is, similarly to a padding BSR, not useful for the scheduler and uses a granted resource unnecessarily.

When the UE is having data in its buffers, frequent periodic BSR is useful to have up-to-date knowledge about the UE's buffer for the scheduler

To continuously send the periodic BSRs when the UE has no data in its buffers would not only cause UE power waste but would also generate interference in the system which should be avoided.

Based on the above it would thus be beneficial that the UE periodically sends a periodic BSR while transmitting on the granted resource and that a periodic BSR containing indication of empty buffers only are discarded. By this, frequent UE buffer fill information is conveyed to the NW for efficient scheduling; while unnecessary utilization of the UL resources are avoided.

The UE configured with periodic BSR only transmit when the UE buffer is non-empty

It is our belief that the implementation of this behaviour is quite straightforward; a corresponding MAC CR containing this change is provided in [3].

To specify the condition of skipping an SPS grant/transmission, it had been proposed to check the obtained MAC PDU from the MAC multiplexing and assembly entity, whether it “containing only padding BSR”. It is straightforward to extend this to “containing only padding BSR or period BSR with zero MAC SDUs”. It can even be simplified to “MAC PDU containing BSR with zero MAC SDUs”. If one would not like to refer to the MAC PDU, once could formulate the skipping of SPS grant/transmission condition as “only BSR with zero MAC SDUs” available for transmission.

# Conclusion

Based on the discussion in the previous section we make the following observation and proposals:

- A periodic BSR triggered when the UE transmit buffer is empty is similarly to a padding BSR not useful for the scheduler and uses a granted resource unnecessarily.

When the UE is transmitting, frequent periodic BSR is useful to have up-to-date knowledge about the UE's buffer for the scheduler

- The UE configured with periodic BSR only transmit when the UE buffer is non-empty

It is proposed to agree on the proposals above and update the corresponding stage 2 and include in stage 3 running CRs once created.

# References

TR 36.881

RP-160667, New WI proposal: L2 latency reduction techniques for LTE

R2-163943, 36321_draftCR_(Rel-14)_ L2 Latency reduction techniques, RAN2#94, May 2016