3GPP TSG-RAN WG2 Meeting #94	R2-164090

# Nanjing, P.R. China, 23rd – 27th May 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | CRNum | rev | - | Current version: | 13.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TDD configurations with skip padding and short intervals |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LATRED_L2 |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | TDD SPS configuration for short SPS intervals. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Short UL SPS intervals of 1 ms, 2 ms 4 ms and 8 ms are introduced for TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will be ambigous how the short SPS intervals are to be interpreted for TDD. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Next change

### 5.10.2	Uplink

After a Semi-Persistent Scheduling uplink grant is configured, the MAC entity shall:

-	if twoIntervalsConfig is enabled by upper layer:

-	set the Subframe_Offset according to Table 7.4-1.

-	else:

-	set Subframe_Offset to 0.

-	if frame structure 2 with semiPersistSchedIntervalUL equal to sf1, sf2, sf4 or sf8

-	consider that the grant occurs in the UL subframe for which:

-	if indicated in Table 5.10.2-1, the corresponding UL HARQ process (HP) belong to a group of HARQ processes, where the group is defined from SFNstart time and subframestart time or

-	if indicated in Table 5.10.2-1, the corresponding UL subframe (SF) belonging to a group of subframes or

-	if indicated in Table 5.10.2-1, consider sequentially that the Nth grant occurs in the subframe for which:

-	(10 * SFN + subframe) = [(10 * SFNstart time + subframestart time) + N * schedIntervalUL + Subframe_Offset * (N modulo 2)] modulo 10240.

Table 5.10.2-1: UL HARQ process numbers or subframe numbers considered for SPS grant

| Uplink-downlink configuration | semiPersistSchedIntervalUL |  |  |  |
| --- | --- | --- | --- | --- |
|  | sf1 | sf2 | sf4 | sf8 |
| 0 | All subframes | HP 0, 2, 4, 6 or HP 1, 3, 5 see Table 5.10.2-2 | HP 0, 4 or HP 1, 3, 5 or HP 2, 6 see Table 5.10.2-2 | Each HARQ process is one group |
| 1 | All subframes | SF 2, 7 or SF 3, 8 | schedIntervalUL = 10 | schedIntervalUL = 20 |
| 2 | All subframes | schedIntervalUL = 10 | schedIntervalUL = 20 | schedIntervalUL = 40 |
| 3 | All subframes | SF 2, 4 or SF 3 | schedIntervalUL = 10 | schedIntervalUL = 20 |
| 4 | All subframes | schedIntervalUL = 10 | schedIntervalUL = 20 | schedIntervalUL = 40 |
| 5 | All subframes | schedIntervalUL = 20 | schedIntervalUL = 40 | schedIntervalUL = 80 |
| 6 | All subframes | HP 0, 2, 4 or HP 1, 3, 5 see Table 5.10.2-3 | HP 0, 3 or HP 1, 4 or HP 2, 5 see Table 5.10.2-3 | Each HARQ process is one group |

-	else

-	consider sequentially that the Nth grant occurs in the subframe for which:

-	(10 * SFN + subframe) = [(10 * SFNstart time + subframestart time) + N * semiPersistSchedIntervalUL + Subframe_Offset * (N modulo 2)] modulo 10240.

Where SFNstart time and subframestart time are the SFN and subframe, respectively, at the time the configured uplink grant were (re-)initialised.

If the MAC entity is not configured with skipUplinkTxSPS, the MAC entity shall clear the configured uplink grant immediately after implicitReleaseAfter [8] number of consecutive new MAC PDUs each containing zero MAC SDUs have been provided by the Multiplexing and Assembly entity, on the Semi-Persistent Scheduling resource.

If releaseAfterTTI is configured, the MAC entity shall clear the configured uplink grant immediately after the time of releaseAfterTTI [8] after initializing or re-initializing a configured uplink grant.

NOTE:	Retransmissions for Semi-Persistent Scheduling can continue after clearing the configured uplink grant.

Table 5.10.2-2: TDD UL/DL configuration 0 HARQ process numbers

| SFNstart time modulo 7 | subframestart time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 |  |  | 0 | 1 | 2 |  |  | 3 | 4 | 5 |
| 1 |  |  | 6 | 0 | 1 |  |  | 2 | 3 | 4 |
| 2 |  |  | 5 | 6 | 0 |  |  | 1 | 2 | 3 |
| 3 |  |  | 4 | 5 | 6 |  |  | 0 | 1 | 2 |
| 4 |  |  | 3 | 4 | 5 |  |  | 6 | 0 | 1 |
| 5 |  |  | 2 | 3 | 4 |  |  | 5 | 6 | 0 |
| 6 |  |  | 1 | 2 | 3 |  |  | 4 | 5 | 6 |

Table 5.10.2-3: TDD UL/DL configuration 6 HARQ process numbers

| SFNstart time modulo 7 | subframestart time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 |  |  | 0 | 1 | 2 |  |  | 3 | 4 |  |
| 1 |  |  | 5 | 0 | 1 |  |  | 2 | 3 |  |
| 2 |  |  | 4 | 5 | 0 |  |  | 1 | 2 |  |
| 3 |  |  | 3 | 4 | 5 |  |  | 0 | 1 |  |
| 4 |  |  | 2 | 3 | 4 |  |  | 5 | 0 |  |
| 5 |  |  | 1 | 2 | 3 |  |  | 4 | 5 |  |