3GPP TSG-RAN WG2 Meeting #95	R2-164794

Gothenburg, Sweden, 22 - 26 August 2016

Agenda item:	9.4.3.4

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Efficient Small Data Transmissions

WID/SID:	FS_ NR_newRAT - Release 14

Document for:	Discussion and Decision

# 1	Introduction

A new study item was agreed at RAN#71 where the main objective aims to develop a New Radio Access Technology (N-RAT) where “Target a single technical framework addressing all usage scenarios, requirements and deployment scenarios defined in TR38.913 including, Enhanced mobile broadband, Massive machine-type-communications, Ultra reliable and low latency communications” [1]. The new radio access technology enables a diverse and wide range of services which needs to deliver high throughput, connect numerous of devices or terminals and provide immediate feedback on demand.

The NG system should give the end user the perception of being always connected, to handle diverse traffic profile and provide the always-on connectivity. In this paper, we provide our initial views about some key characteristics and principles to support small data transmission with mobility.

# 2	Discussions

The data traffic increases significantly with the growth of the various devices and applications, e.g. the always-on applications which need to be constantly reachable with the network. Keep-alive messages generated from always-on applications refer to the autonomous exchange of user plane data packets between the UE and the network in order to maintain IP connection in the absence of a specific user interaction with device. This kind of traffic is generally low in volume and comprises packets that may be widely dispersed in time [2].

On the other hand, most of the applications for MTC use cases may not require large amounts of data transmission with high data rates. The MTC devices, like the massive amount of low-cost battery-powered sensors or the remote-controlled utility meters, are expected to communicate with infrequent small burst transmission which may occur possibly of the order of one or two messages per day per device.

Observation 1: significant number of UEs create bursty data traffic.

To reduce the UE power consumption, the UE can be sent to RRC_IDLE by eNB according to an inactivity timer. It has been observed that in LTE networks inactivity timers are typically configured to be quite short (between 10-60 seconds) in order to preserve battery life and network resources. Therefore, the power constrained MTC devices which only send small size payloads at very low frequency usually stay in RRC_IDLE state to minimize the power consumption. However, to communicate with network a small amount of data, the signalling overhead for setting up the connection could become undesirably high comparing with the actual user application data.

Observation 2: To save power and resources, UEs with no active data transmissions are typically pushed back to IDLE, which is at the expense of signalling overhead for small data transfer.

In [3] and [4], the target for control plane latency is 10ms, which refers to the time to move from a battery efficient state (e.g., IDLE) to start of continuous data transfer (e.g., ACTIVE). This requirement is relevant for infrequent application layer small packet/message transfer, considering the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU at the mobile device to the radio protocol layer 2/3 SDU in the NG RAN, when the mobile device starts from its most “battery efficient” state. However the legacy procedure to establish the connection [2] requires the RAN side signalling steps as well as the interaction with Core Network, and state transition latency from RRC_IDLE to RRC_CONNECTED is around 50ms [5]. Such delay for setting up the connection would not satisfy the latency requirements.

Observation 3: the ordinary connection setup procedure cannot achieve the required control plane latency.

In order to minimize UE state transitions, those UEs which need to maintain the IP connection for the always-on applications may be kept in RRC_CONNECTED state to reduce the potential signalling. However if UEs are kept in RRC_CONNECTED for long times, as per the legacy LTE packet transmission procedure (e.g. scheduling requests, measurements, etc.), the radio resource e.g. PUCCH resources utilization efficiency may be negatively impacted, small data transmissions would consume the (control) channel capacity and lower down the cell throughput.

Observation 4: keeping all UEs with bursty data traffic in RRC_CONNECTED is challenging.

Based on the observations above, improvements in the area of efficient small data transmission for low activity UEs are of interest. The always-on applications or the MTC type of applications would require minimal signalling overhead to enable efficient short message transactions to avoid signalling exchange for network attachment, link establishment and scheduling. As a result, mechanisms for improving RRC connection management, optimizing control plane signalling to reduce connection setup delay should be studied. To handle the UE which keeps inactive most of the time, RAN2 #94 [6] agreed to study the introduction of a RAN controlled “state”, during which UEs should incur minimum signalling, minimise power consumption, minimise resource costs in the RAN/CN, making it possible to maximise the number of UEs utilising (and benefiting from) this state. Within this RAN controlled “state”, the CN and RAN connection may be maintained, the AS context is stored in RAN and there might be no dedicated resources for UE. The UE’s location is known to network within an area and UE performs mobility within that area without notifying the network.

This new RAN controlled “state” will enable more flexible radio resource management, however in order to satisfy different use cases, it is also necessary to take into account various factors when managing the radio resource of the connection. Moreover, the specific enhancement for either higher layer or the physical layer could be selected in response to variations in the service requirements.

Proposal 1: To provide efficient connection and resource management for the UEs in low activity, the following key requirements should be considered for the new “state” in addition to the assumed characterises:

-	UEs with low activity may keep connected from core network perspective and intermittent data transfers should be kept transparent to the core.

-	Specific enhancements may be configurable based on the service requirements or device type. 	,

Except for the signalling connection management, efficient management and utilisation of physical resources for the UEs in low data activity period are also desired. RAN1#84bis [7] has already discussed some potential physical layer techniques, and agreed to investigate the non-orthogonal multiple access for diversified NR usage scenarios and use cases. As discussed in RAN2# 94 [6], within the new RAN state, the resource costs in RAN/CN need to be minimized and there might be no dedicated resource allocated for UE specifically. The autonomous/grant-free/contention based non-orthogonal multiple access might be one potential transmission scheme at least for UL mMTC case.

Besides, SA2 [8] is studying the architecture for Next Generation System, the high level requirements of the architecture includes the minimize of the signalling (and delay) required to start the traffic exchange between the UE and the Data Network, i.e. signalling overhead and latency at transition from a period where UE has no data traffic to a period with data traffic. How to manage the session is one key issue including setting up the IP or non-IP traffic connectivity of the UE as well managing the user plane for that connectivity. Scenarios and mechanism, which may not require the session to be established, e.g., to provide efficient support for infrequent small user data transmission, will be investigated. One of the solution of the session management is to transmit data with ConnectionLess RAN-Core interface. UE uses RRC and NAS signalling to establish connectionless services and use previously assigned configuration to send and receive data.  The contention-based access is one of the candidate solutions for uplink data transmission. With this solution, (Massive) IoT UE that may move and need to send sporadic (potentially small amount) data, may send data to the NextGen Core via the user plane path without requiring more signalling in the network (RAN and Core) than the amount of user data to be transferred.

The details of contention-based transmission and the ConnectionLess RAN-Core interface are being investigated in RAN1 and SA2 respectively. Both will bring impact to RAN2 discussion obviously, thus the aspects from RAN2 side need to be elaborated as well to obtain a full picture and accurate assessment of the overall gains for those two schemes. The potential implications to the higher layer procedures need to be analysed including the RRC state under which those two schemes are applicable and how the mobility and security are supported, etc.

Proposal 2: the RAN2 aspects to apply the autonomous/grant-free/contention based transmissions and ConnectionLess interface need to be studied.

# 3	Conclusions

This contribution has analysed key issues for improving the performance of the UEs in low activity mode and the following observations were made:

Observation 1: significant number of UEs create bursty data traffic.

Observation 2: to save power and resources, UEs with no active data transmissions are typically pushed back to IDLE, which is at the expense of signalling overhead for small data transfer.

Observation 3: the ordinary connection setup procedure cannot achieve the required control plane latency.

Observation 4: keeping all UEs with bursty data traffic in RRC_CONNECTED is challenging.

As a conclusion, we propose:

Proposal 1: to provide efficient connection and resource management for the UEs in low activity, the following key requirements should be considered for the new “state” in addition to the assumed characterises:

- UEs with low activity may keep connected from core network perspective and intermittent data transfers should be kept transparent to the core.

- Specific enhancements may be configurable based on the service requirements or device type. 	,

Proposal 2: the RAN2 aspects to apply the autonomous/grant-free/contention based transmissions and ConnectionLess interface need to be studied.

# References

- RP-160671, Study on NR New Radio Access Technology, NTT DOCOMO

- 3GPP TR 36.822, LTE RAN enhancements for diverse data applications, V0.2.0, Nov. 2011.

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies, RAN

- NGMN Alliance, “KPIs and Deployment Scenarios for Consideration for IMT2020”, Version: 1.0, 4th March 2016

- 3GPP TR 36.912, “Feasibility study for Further Advancements for E-UTRA”, Rel.11

- R2-16xxxx_draft_report_RAN2_94_Nanjing_v0.1.zip, Draft Report of 3GPP TSG RAN WG2 #95 v0.1.0, Nanjing, China, 2016

- Draft_Minutes_report_RAN1#84bis_v010.zip, Draft Report of 3GPP TSG RAN WG1 #84bis v0.1.0, Busan, Korea, 2016

- 3GPP TR 23.799, Study on Architecture for Next Generation System