3GPP TSG RAN WG2 Meeting #95		R2-164899

Gothenburg, Sweden, 22–26 August 2016

Agenda Item:	9.5.1.3

Title:	RLM and RLF in HF NR

Document for:	Discussion and decision

# 1	Introduction

One of the objectives in NR is to support frequency ranges up to 100 GHz. In High Frequency (HF)-NR systems, beamforming is a key enabling technology to compensate the propagation loss through high antenna gain. In HF system with beamforming, UE can detect multiple beams and beam management is used to select one of them for communication. In this contribution, we discussion how RLM is performed with beam management and how RLF is declared.

# 2	Discussion

## 2.1 RLM and RLF in LTE

In LTE, the Pcell/PScell, which is covered by omni-directional beam or a sector beam, is considered as one radio link. RLF is performed by UE to monitor the downlink link quality based on the cell-specific reference signal in order to detect the downlink radio quality of the serving cell. The quality is actually mapped to a hypothetical PDCCH block error rate. It is compared to the thresholds Qout and Qin, which are corresponding to 10% BLER and 2% BLER of a hypothetical PDCCH transmission respectively. So that Qout and Qin are indicated to RRC, which is used for RLF procedure. Upon receiving consecutive numbers of Qout, a timer T310 is started. The timer is used to supervise whether the radio link can be recovered with consecutive numbers of Qin. RLF is declared when the timer expires. So the downlink radio link quality problem of the serving cell can be detected through RLM procedure.

The main characteristics of RLM/RLF in LTE are:

- UE performs RLM on the serving cell covered by an omni-directional beam or a sector beam to detect the downlink radio link quality.

- Radio link is considered as recovered if consecutive numbers of Qin is indicated before T310 expiry and T310 is stopped if radio link is recovered.

- RLF is declared upon T310 expiry.

## 2.2 RLM and RLF in HF-NR

For HF-NR with beam operation, the cell is covered by multiple beams. UE side beamforming is required in most of the DL scenarios and uplink scenarios in order to extend coverage with reasonable spectral efficiency. So the communication link is actually a Tx-Rx beam pair. UE side beamforming for Rx beam is not only used for both data transmission but also used for measurement, which means that the measurement result for each Tx beam contains certain receiver array gain at the UE side. Although UE side beamforming is used for DL data reception, UE only need to consider the multiple Tx beams from the network side for downlink quality detection.

Observation 1: The cell is covered by multiple Tx beams and UE performs measurement on different beams.

Proposal 1: UE shall consider RLM and RLF with multiple Tx beams from the network side.

If RLM/RLF considers multiple beams, one question is how to utilize the collective measurement results of different beams to represent the radio link quality of the serving cell.

Beam management is required to select the best beam to maintain the connection between the network and the UE.  In order to support fast beam switching, UE performs CSI measurement and reporting on multiple beams.  The selected beam used for communication is considered as serving beam; other beams with CSI measurement and reporting are considered as candidate beams. So the serving beam changes among a set of candidate beams.

Observation 2: The purpose of beam management is to select a beam among a set of beams. The selected beam for communication is considered as serving beam and the set of beams are considered as candidate beam.

Beam management is performed by lower layer for fast beam switching, which should be able to adapt to the channel variation. Just as illustrated in Figure 1, beam management procedure is performed in parallel with RLM. According to the beam switching command received from network, UE picks the corresponding beam as serving beam and collects the measurement results of the serving beam. Then Qout or Qin are indicated to RRC based on the estimation over their own evaluation periods respectively. Considering the serving beam is the radio link used for data transmission, it shall be used to represent the downlink radio link quality in normal case.

Figure 1 Beam Management and RLM

Observation 3: Beam management is performed in parallel with RLM.

Proposal 2: RLM is performed on the serving beam.

Beam management determines which beam is used as serving beam, where RLM is performed. The interaction between beam management and RLM should be considered.

Since the HF signal suffer from severe penetration loss when passing through solid materials, blockage may occurs frequently depending on the density of the blockers around as well as the distance to UE. In some cases, it is possible that both the serving beam and candidate beams are not available, especially when UE moves to an area with deep shadowing.

Due to the fast channel variation, it is possible that UE and the network loss the beam between each other if any one of the signalling required for beam switching is not received timely and correctly. When beam misalignment occurs, UE can’t detect it timely and continues the RLM on the serving beam, the channel quality of which has already degraded. It is very likely that RLF will be declared.

Beam management may not be able to handle the above abnormal cases. The desired radio link recovery mechanism for those two events may be different. For the first case, the straightforward way is to perform re-establishment, because other beams may also not available. For the second case, although re-establishment is still workable, considering the cost of re-establishment procedure, it is better that UE can initiate UE side beam alignment and try to acquire other beams. Although the reason for RLF may be different, UE can’t differentiate them exactly. A common criterion for RLF detection as well as radio link recovery mechanism is required to handle all of the cases.

Observation 4: when radio link problem is detected on the serving beam, there may be other good beams from the cell.

When RLF is declared, UE performs re-establishment and cell selection is initiated. The connection re-establishment succeeds only if the selected cell has a valid UE context. If the concerned cell is not prepared, re-establishment fails and UE goes to IDLE. In HF with beamforming, RLF declaration should be conservative even if radio link problem is detected on the serving beam. It is possible that there are other candidate beams available, which can be used to resume data transmission. So if radio link problem is detected on the serving beam, UE can initiate UE side beam alignment and try to acquire other candidate beams in the serving cell for data transmission.

Proposal 3: RAN2 study UE triggered beam recovery when radio link problem is detected on the serving beam.

Taking Figure 2 for example, if consecutive number of Qout are indicated based on the measurement on serving beam, link recovery is initiated.  So UE tries other candidate beams with beam alignment. UE performs RA procedure with the candidate beams according to its CSI measurement results on those beams. If no beam can be used to resume the connection with the serving cell before expiry of the timer, RLF can be declared. Then UE can initiate RRC connection re-establishment procedure for the last chance to recover the RRC connection.

Figure 2 RLM/RLF in HF-NR

Proposal 4: RLF is declared if there is no recovery with other beams after a pre-defined timer.

Since the purpose of recovery is to resume the connection with the serving cell, the candidate beams in the serving cell is the target to perform the recovery procedure. If on beam can be acquired in the serving cell (in the first phase), RLF is declared. UE performs RRC connection reestablishment procedure (in the second phase). As a result of cell selection procedure, UE may try another set of beams in different cells, or even on different frequencies in NR. If re-establishment fails, UE goes back to IDLE. In this case, UE may initiate inter-RAT search.

# 3	Conclusion

In this contribution, RLM and RLF procedure for NR-HF is discussed.

Based on the observations that:

Observation 1: The cell is covered by multiple Tx beams and UE performs measurement on different beams.

Observation 2: The purpose of beam management is to select a beam among a set of beams. The selected beam for communication is considered as serving beam and the set of beams are considered as candidate beam.

Observation 3: Beam management is performed in parallel with RLM.

Observation 4: when radio link problem is detected on the serving beam, there may be other good beams from the cell.

We have following proposals:

Proposal 1: UE shall consider RLM and RLF with multiple Tx beams from the network side.

Proposal 2: RLM is performed on the serving beam.

Proposal 3: RAN2 study UE triggered beam recovery when radio link problem is detected on the serving beam.

Proposal 4: RLF is declared if there is no recovery with other beams after a pre-defined timer.