3GPP TSG-RAN WG2 Meeting #95	R2-165169

Gothenburg, Sweden, August 22 – 26, 2016

Source:	Samsung

Title:	Considerations for new multiple access schemes

Agenda Item:	9.4.3.4

Document for:	Discussion

# Introduction

- Autonomous/grant-free/contention based UL non-orthogonal multiple access has the following characteristics

  - A transmission from UE does not need the dynamic and explicit scheduling grant from eNB

  - Multiple UEs can share the same time and frequency resources

- For autonomous/grant-free/contention based UL non-orthogonal multiple access, the following should be studied

  - Collision of  time/frequency resources from different UEs, solutions potentially including

    - E.g., code, sequence, interleaver pattern

  - ……

- NR supports at least synchronous/scheduling-based orthogonal multiple access for DL/UL transmission schemes, at least targeting for eMBB

RAN1#85 agreed to consider the following aspects for new multiple access schemes for NR [1]

OFDMA, as used in LTE, requires strict orthogonal requirements between different users and the same set of resources cannot be re-used across different users. However, with the advances in multi user detection algorithms, it is now possible to support several users (with distinguishing features such as different codes from a fixed codebook, power levels, different interleaver’s etc) on same set of resource blocks and yet successfully decode their data successfully. Such techniques are broadly referred to as non-orthogonal multiple access techniques. Several new multiple access schemes have been proposed in the literature to support these non-orthogonal multiple access techniques [1]. These techniques help to re-use the resources and thereby improve the spectral efficiency of the overall NR network and also enable to support massive number of users which is a requirement for mMTC, one of the services supported by NR.

Owing to the various requirements of the various services that will be supported by NR (URLLC, mMTC and eMBB), different multiple access schemes may be used for different services. Significant study and analysis is necessary before these schemes may actually be used for the various scenarios. In this contribution, we lay out some directions that needs to be considered by RAN2 to support these multiple access schemes.

# Multiple Access Schemes for NR

2.1 Multiple Access Scheme Definitions

Here, we briefly explain the various multiple access (MA) schemes that are under consideration for NR --

- Grant-based (GB)  This is the regular LTE Mechanism where UE transmits only when a grant is received; other faster mechanisms may also be considered for NR

- Contention-based (CB)  Pre-configure the resource pool (set of RB’s) a priori which is signaled to the UEs under consideration. The UL transmission is autonomous and is not triggered by NB’s grant i.e., users can send data in these pre-configured resources without the explicit grant procedure and contentions may be allowed between users.

2.2 MA Scheme Classification

An example MA scheme classification based on the NR services is shown in Table 1. URLLC service may need to support grant-based schemes due to its reliability considerations. Due to the massive number of devices that may be supported by mMTC service, it may not be feasible to enable grant-based schemes. Furthermore, due to the dynamic and varied nature of operations of the mMTC devices, it would be advised to only support contention-based access schemes. Since eMBB services have deep coverage, high mobility and low latency requirements, supporting contention-based access schemes may result in collisions and thereby not enable to achieve these goals. However, note that it is up to the NB’s discretion when, where and to which devices it supports the various MA schemes.

Table 1: An example MA scheme classification based on NR Services (used for data transmission).

| MA Scheme | URLLC | eMBB | mMTC |
| --- | --- | --- | --- |
| Grant-based | YES | YES | NO |
| Contention-based | NO | NO | YES |

Observation 1: Different NR services may support different types of MA schemes.

The MA schemes can also be classified based on the resource allocation as shown below in Table 2 -

Table 2: MA scheme classification based on resource allocation

| MA Scheme | Resource Allocation |
| --- | --- |
| Grant-based | Dynamic |
| Contention-based | Semi-Static |

The grant-based mechanism dynamically allots the freely available resources to the UE that requested for the resources. For contention-based schemes, the resource pool is a semi-statically allotted one, where a small portion can remain static and the rest can be dynamically indicated on a need basis (based on the network load, for example). The static allocation can help to address the allocations to a massive number of devices (in the case of mMTC) which can be signaled via SIB information, as an example.

Observation 2: It is helpful to investigate what type of resource pool allocation is applicable for each supported NR service and the specific MA schemes they support.

In Table 3, the applicability of the new MA schemes have been classified based on the UE RRC state. A new RRC state (which is termed as INACTIVE) has been under discussion since RAN2#94. This state may support small amounts of data transfer [2-5] and will be used as an efficient mode of transfer between IDLE and CONNECTED states. CB schemes may enable to support the devices in this RRC state. However, since data transfer support in this state is still under study, the applicability of CB schemes may be considered for future study. For control signaling, the legacy schemes based on GB schemes may still be used. For example, in the IDLE mode, when the UE performs a RACH procedure to perform connection establishment, it is similar to a GB procedure. A similar procedure should at least be supported in the INACTIVE state and hence GB may also be allowed in this state [5].

Table 3: MA scheme classification based on the UE RRC state.

| MA Scheme | IDLE | INACTIVE*[NOTE] | CONNECTED |
| --- | --- | --- | --- |
| Grant-based | YES | YES | Depends on the NR Service |
| Contention-based | N/A | YES | Depends on the NR Service |

*[NOTE] As in [5], once a UE initiates UL activity in the INACTIVE state, it moves logically to the CONNECTED state.

Observation 3: It is beneficial to consider the applicability of the various MA schemes based on the various NR services and the UE operating (RRC) states.

Proposal 1: The applicability of the various MA schemes based on the requirements of the NR services and the UE operating states (RRC states) needs to be investigated by RAN2.

# Resource pool allocation and configuration

For contention-based MA schemes, a pre-defined resource pool should be allotted and this resource pool is signaled a priori to the users that support such a MA scheme. An example resource pool allocation is shown below.

= contention-based pool

Freq.

eMBB

URLLC

SI

UL Resource Pool for CB Scheme

Distributed Allocation

URLLC

Time

DCI

UL Resource Pool for CB Scheme

Time

Freq.

UL Resource Pool for CB Scheme

eMBB

URLLC

SI

Contiguous Allocation

URLLC

DCI

Figure 1: An example resource pool allocation for CB schemes; shown is an example signaling via SI and DCI to indicate the resource pool locations

Since service-specific resource allocations have been under discussion in 3GPP, we show eMBB and URLLC specific RB allocations (as an example) in the figure above. The orange boxes indicate the set of RBs that are allotted for supporting the CB scheme for whatever NR service may support it. Some implicit UE specific resource access mechanisms may also be considered inside this CB resource pool that can minimize the collisions.

Once these resource pools have been allotted, the NR NB needs to indicate/signal these allocations to all UE’s that support these MA schemes and need such a resource pool allocation. The associated signalling mechanisms will change based on whether contiguous or distributed allocations are used. An example signalling mechanism via SI and DCI is shown in Figure 1. It is also to be studied how, when and where to signal these resource pool’s to the UE’s. Since the exact type of MA schemes are being investigated in RAN1, and the resource pool allocation will be decided in RAN1, the associated signalling procedures may follow the decisions taken in RAN1 and RAN2 may consider working on these only after RAN1 decisions have been made.

Observation 4: The signaling procedures for indicating the different resource pool allocations may follow the decisions taken in RAN1 about the new MA schemes.

Proposal 2: The impact of new MA schemes and resource pool allocations on the layer-2 functionalities, including signaling, scheduling etc. based on the decisions taken in RAN1 needs to be considered.

Proposal 3: RAN2 should leverage upon RAN1 analysis of UL contention based performance to assess applicability of those schemes to different scenarios and to conclude upon the overall system level gains.

# Summary

Observation 1: Different NR services may support different types of MA schemes.

Observation 2: It is helpful to investigate what type of resource pool allocation is applicable for each supported NR service and the specific MA schemes they support.

Observation 3: It is beneficial to consider the applicability of the various MA schemes based on the various NR services and the UE operating (RRC) states.

Observation 4: The signaling procedures for indicating the different resource pool allocations may follow the decisions taken in RAN1 about the new MA schemes.

Proposal 1: The applicability of the various MA schemes based on the requirements of the NR services and the UE operating states (RRC states) needs to be investigated by RAN2.

Proposal 2: The impact of new MA schemes and resource pool allocations on the layer-2 functionalities, including signaling, scheduling etc. based on the decisions taken in RAN1 needs to be considered.

Proposal 3: RAN2 should leverage upon RAN1 analysis of UL contention based performance to assess applicability of those schemes to different scenarios and to conclude upon the overall system level gains.

# References

[1] 3GPP RAN1#84 Meeting Report, May 2016

[2] R2-163442, "Efficient Small Data Transmissions ", Nokia, Alcatel-Lucent

[3] R2-164095, "NR RRC states definition", Qualcomm Incorporated, Convida Wireless

[4] R2-164028, "Infrequent small data transmissions for inactive UEs", Ericsson

[5] R2-164679, “Design principles for the new RAN controlled state”, Samsung.